| RAPPORTEUR MEETING OF QUESTION 21/16 |  |  |  | TD-06 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | H.VSMprot Editor |  |  |  |
| Title: | H.VSMprot “Signalling and protocols for mobile visual surveillance” (New): Output draft (Changzhou, 26-29 September 2016) |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This document contains the editors’ output draft for H.VSMprot “Signalling and protocols for mobile visual surveillance” (New) from the SG16, Changzhou, 26-29 September 2016 meeting.

Updates were based on the following contributions:

| AVD-4783 | China Telecom | H.VSMprot: Proposal to add VAU-VS Reference point in clause 5.8 and revise clause 5 | Q21/16 |
| --- | --- | --- | --- |
| AVD-4784 | China Telecom | H.VSMprot: Proposal to add real-time video/audio acquisition to clause 6.1 and recording and playback to clause 6.2 | Q21/16 |
| AVD-4785 | China Telecom | H.VSMprot: Proposal to add PTZ control to clause 6.3 and resource query to clause 6.4 | Q21/16 |
| AVD-4786 | China Telecom | H.VSMprot: Proposal to update text | Q21/16 |

NOTE– Editorial adjustments introduced by TSB.

# CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 7 4 Abbreviations 7 5 Protocol reference points 8 5.1 Reference point Cms: CMU-MSP 9 5.1.1 Interface function 9 5.1.2 Interface protocol 10 5.1.3 Message format 10 5.2 Reference point Msm: MSP-M_CU 17 5.2.1 Interface function 17 5.2.2 Interface Protocol 17 5.2.3 Message Format 17 5.3 Reference point Wm: WAP GW- MSP 20 5.3.1 Interface function 20 5.3.2 Interface protocol 20 5.3.3 Message format 20 5.4 Reference point Psm: M_CU- PSS 21 5.4.1 Interface function 21 5.4.2 Interface protocol 21 5.4.3 Message format 21 5.5 Reference point Vm: M_CU- VAU 24 5.5.1 Interface function 24 5.5.2 Interface protocol 24 5.5.3 Message format 25 5.6 Reference point Mv: M_CU-VAU 26 5.6.1 Interface function 26 5.6.2 Interface protocol 26 5.6.3 Message format 26 5.7 Reference point Vp: VAU-PSS 28 5.7.1 Interface function 28 5.7.2 Interface protocol 28 5.7.3 Message format 28 5.8 Reference point Mdv/Pv/Vs:VAU-VS system 30 6 Signalling and protocols 30 6.1 Real-time video/audio acquisition 30 6.2 Recording and playback 32 6.2.1 Playback in platform 32 6.3 PTZ control 36 6.4 Resource query 38 Bibliography 38

# List of Tables

Page

Table 5-1-1: Interface: MSP->CMU 10 Table 5-1-2: Interface: CMU->MSP 10 Table 5-1-3: Interface: MSP->CMU 11 Table 5-1-4: Interface: CMU->MSP 11 Table 5-1-5: Interface: MSP->CMU 12 Table 5-1-6: Interface: CMU->MSP 12 Table 5-1-7: Interface: MSP->CMU 13 Table 5-1-8: Interface: CMU->MSP 13 Table 5-1-9: Interface: MSP->CMU 13 Table 5-1-10: Interface: CMU->MSP 14 Table 5-1-11: reqVauUrlReques Interface 14 Table 5-1-12: reqVauAddResponse Interface 15 Table 5-1-13: contain fields 16 Table 5-2-1: M_CU Authentication certification and Packet access 17 Table 5-2-2: MSP-MCU 17 Table 5-2-3: Surveillance points list query 18 Table 5-2-4: returns list of surveillance point 18 Table 5-2-5: Video surveillance point play entrance query 19 Table 5-2-6: Returns the link address 19 Table 5-2-7: The statistics of mobile phone surveillance video data flows, duration 20 Table 5-3-1: Interface: WAP GW->MSP 20 Table 5-4-1: HTTP Parameters from M_CU to PSS 22 Table 5-4-2: RTSP Parameters from M_CU to PSS 23 Table 5-5-1: Common Parameters from M_CU to VAU 24 Table 5-5-2: Realtime Mode Parameters from M_CU to VAU 24 Table 5-7-1: VAU interface 28 Table 5-7-2: VAU multi –platform data model 29 Table 5-7-3: VAU state management information in system model 29 Table 5-7-4: PSS configuration management information in system model 29 Table 5-7-5: PSS state management information in system model 30

# List of Figures

Page

Figure 5-1 Functional Architecture Framework of mobile Visual Surveillance 9 Figure 5-2 Interface protocol 21 Figure 5-3 Signalling 21 Figure 6-1 High level procedural flows for real-time media acquisition 31 Figure 6-2 Video Playback from the platform procedural flows 33 Figure 6-3 Video Playback from the PU procedural flows 35 Figure 6-4 PTZ control procedural flows 37 Figure 6-5 USER resource query procedural flows 38

# Draft new Recommendation ITU-T H.VSMprot

# Signalling and protocols for mobile visual surveillance

# AAP Summary

[To be provided before Consent]

# Summary

This Recommendation defines the detailed specification of reference points, signalling and

control methods, and overall protocols of a mobile visual surveillance system based on the requirements described in ITU-T Recommendation F.743 and functional architecture described in ITU-T Recommendation H.626.1.

This Recommendation focus on the signalling and protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

This Recommendation gives detailed description of the reference points defined in ITU-T Recommendation H.626.1 as well as some newly defined reference points.

The relevant protocols are identified, the message format and message text (such as SIP, RTP, RTSP, HTTP) are also provided.

# Keywords

Mobile Visual Surveillance, Reference Point, Signalling and Protocols

# 1	Scope

The Recommendation H.VSMprot focuses on the Signalling and Protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

This Recommendation defines the reference points, signalling and protocols of a mobile visual surveillance system, based on the architecture defined in the Recommendation ITU-T H. 626.1.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance.

[ITU-T H. 627]	Recommendation ITU-T H.627 (2012), Signalling and Protocols for visual surveillance.

[ITU-T H. 626.1]	Recommendation ITU-T H.VSMarch (2011), Architectural requirements for mobile visual surveillance.

[3GPP TS 26.234] 3rd Generation Partnership Project V10.3.0 (2011-11); Technical Specification Group Services and System Aspects; Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs (Release 10).

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP).

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1.

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1 <Application> [ITU-T Y.101]: A structured set of capabilities, which provide value-added functionality supported by one or more services.

3.1.2 <Customer> [ITU-T M.60]: An entity which receives services offered by a service provider based on a contractual relationship. It may include the role of a network user.

NOTE 1 – In this Recommendation, Customer has the same meaning as End User.

3.1.3 <Customer unit> [ITU-T H.626]: A device located at the customer part of a Visual Surveillance system and used to present multimedia information (such as audio, video, image, alarm signal, etc.) to the end user.

3.1.4 <Functional architecture> [ITU-T Y.2012]: A set of functional entities and the reference points between them used to describe the structure of an NGN. These functional entities are separated by reference points, and thus they define the distribution of functions.

NOTE 2 – The functional entities can be used to describe a set of reference configurations. These reference configurations identify which reference points are visible at the boundaries of equipment implementations and between administrative domains.

NOTE 3 – The definition is not only applicable to NGN, but also to other IP packet switch based networks.

3.1.5 <Interface> [ITU-T Y.101]: A shared boundary between two functional units.

NOTE 4 – An interface is defined by various characteristics pertaining to the functions, physical interconnections, signal exchanges and other characteristics as appropriate.

3.1.6 <Media> [ITU-T Y.101]: Plural of medium.

3.1.7 <Medium> [ITU-T Y.101]: a) Specific physical support for transmission or storage of information. b) Type of presentation of information (i.e., video, audio, text, etc.).

3.1.8 <Mobile customer unit> [ITU-T H.626.1]: Mobile client software installed in customers’ mobile devices. It is used to initiate the service and provide video viewing to customers.

3.1.9 <Mobile service portal> [ITU-T H.626.1]: A series of devices and subsystems located at the central part of a mobile visual surveillance. It is the business application platform of the mobile visual surveillance. The main functions include unified access entrance to the business entrance of user authentication, user monitoring permissions list display, and it can realize user authentication, querying of monitoring list, synchronization of user account’s information, assisting service

3.1.10 <Reference point> [ITU-T Y.2012]: A conceptual point at the conjunction of two non-overlapping functional entities that can be used to identify the type of information passing between these functional entities.

NOTE 5 – A reference point may correspond to one or more physical interfaces between pieces of equipment.

3.1.11 <Service> [ITU-T Y.101]: A structure set of capabilities intended to support applications.

3.1.12 <Packet-switched streaming service> [ITU-T H.626.1]: A device located at the central part of a mobile visual surveillance, it is the stream media server and processes media stream distribution between M_CU and VAU. The main functions of PSS include responding to the service request message sent by M_CU, getting the real-time media stream from VAU and distributing the stream to multiple M_CUs.

3.1.13 <Premises unit> [ITU-T H.626]: A device located at the remote part of a Visual Surveillance system and used to capture multimedia information (such as audio, video, image, alarm signal, etc.) from a surveilled object.

3.1.14 <Video access unit> [ITU-T H.626.1]: A device located at the central part of a mobile visual surveillance, it is used to implement the communication between mobile customer unit and the units defined in ITU-T Rec. H.626. The main functions of it includes: request scheduling information from CMU used to establish the media session between MDU and packet-switched streaming service, agent mobile customer unit’s request to control PTZ, distribute and transcode multimedia data.

3.1.15 <Visual surveillance> [ITU-T H.626]: A telecommunication service focusing on video (but including audio) application technology, which is used to remotely capture multimedia (such as audio, video, image, alarm signal, etc.) and present them to the end user in a user friendly manner, based on managed broadband network with ensured quality, security and reliability.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

None.

# 4	Abbreviations

| CDMA CMU CU GW HTTP ID IP IPC M_CU MIB MDN MDU MSP NVR PSS PTZ PU RTCP RTP RTSP R/W SCU SOAP TCP UDP URL VAU VS WAP XML | Code Division Multi Address Centre Management Unit Customer Unit Gateway HyperText Transfer Protocol Identification Internet Protocol IP Camera Mobile Customer Unit Management Information Base Mobile Directory Number Media Distribution Unit Mobile Service Portal Network Video Recorder Packet-switched streaming service Pan/Tilt/Zoom Premises Unit Real-time Transport Control Protocol Real-time Transport Protocol Real-time Streaming Protocol Read/Write Service Control Unit Simple Object Access Protocol Transmission Control Protocol User Datagram Protocol Uniform Resource Locator Video Access Unit Visual Surveillance Wireless Application Protocol Extensible Markup Language |
| --- | --- |

# 5	Protocol reference points

Recommendation [ITU-T H. 626.1] describes overall architecture framework of mobile Visual Surveillance as the following figure, and it also introduces reference points of architecture. This document gives more detailed description of those reference points as well as some newly defined reference points.

# Figure 5-1  Functional Architecture Framework of mobile Visual Surveillance

The interfaces and protocols which involved in mobile video surveillance are as follow:

- Reference point Cms: CMU-MSP, uses the Web Service interface protocol (SOAP/HTTP);

- Reference point Msm: MSP-M_CU, uses the HTTP protocol;

- Reference point Mv: M_CU-VAU, uses the UDP communication protocol;

- Reference point Vm: VAU-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Psm: PSS-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Vs: VAU-SCU, uses the HTTP protocol;

- Reference point Mdv: MDU-VAU, uses the RTP/RTCP protocol;

- Reference point Pv: PU-VAU, uses the RTP/RTCP protocol;

- Reference point Vp: VAU-PSS, uses the RTSP protocol;

- Reference point Wm: WAP GW-MSP, WAP GW puts the MDN number in the head domain of HTTP.

## 5.1	Reference point Cms: CMU-MSP

### 5.1.1	Interface function

The interface is used for querying the attributions of the surveillance point when the users log inand the information of all surveillance points available for mobile surveillance service.

- User authentication and surveillance point query: When user logs in MSP by M_CU, MSP could get the UserID and password of the current user (If user choose the universal sign-in way. MSP needs to translate the MDN number into the corresponding UserID and password locally), then MSP authenticates the user to the CMU of video surveillance platform via the User ID and password. If authentication fails, the failed message would be returned. If succeed, CMU would inquiry the surveillance point information according to current UserID, CMU returns the corresponding surveillance point and parameter list under the UserID in the MSP.

- Inquiry the enterprise surveillance points attribution: When enterprise users log in Enterprise Self-Sevice System through the MSP, MSP could get the inquired  information of the current surveillance point list that enterprise has activated on CMU video surveillance platform through the enterprise account (such as CustID).

- RTSP media stream entrance inquiry: When user inquiry some kind of media streaming of one surveillance point, CMU would returns RTSP access address that user get through internal inquiry scheduling on the platform, which need the access token.

Universal sign-in user requests for resource  accord to the original interface specification, the method called "reqUserChannelInfo".

### 5.1.2	Interface protocol

The interface is hosted on the IP protocol, using the Web Service interface protocol (SOAP/HTTP).

### 5.1.3	Message format

1)	The enterprise attribute information query

MSP query the enterprise attribute information (used when handling business opening worksheet by Telecom administrator) according to the corporate identify (customer ID) .

# Table 5-1-1: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerId | Enterprise ID | string(64) | Telecom administrator manually input when the MSP service opened (18 bit) |

Explanation: the first six bits of customerID refer to the domain ID (Platform domain)

# Table 5-1-2: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerName | Customer names | String(60) |  |
| CustomerType | Customer type | int(4) | Only : Enterprise:1 |
| Address | Customer address | String(100) |  |
| PhoneNumber | Customer phone number | String(30) |  |
| Fax | Customer fax number | String(30) |  |
| ZipCode | Customer zip code | String(6) |  |
| Email | Customer E-mail | String(60) |  |
| TransactPerson | The operator | String(30) |  |
| AdminAccount | video surveillance admin account | String(64) |  |
| ResultCode | Return code | Int(1) | 0 for enterprise ID not exist 1 for accession success |

Explanation: User uses "adminAccount prefix" to login MSP Enterprise Self-Service page, the user ID and user account are the same in format.

2)	Search enterprise information list

MSP query the enterprise attribute information (when handling business opening worksheet by Telecom administrator) according to the corporate identify (customer ID) or customer name.

# Table 5-1-3: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| searchCID | Enterprise ID | string(64) | Telecom administrator manually input when the MSP service opened (18) |
| searchCN | Enterprise name | string(255) | Company name |

Explanation: the first six bits of customerID refer to the domain ID (Platform domain).

# Table 5-1-4: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerName | Customer names | String(60) |  |
| CustomerType | Customer type | int(4) | Only : Enterprise:1 |
| Address | Customer address | String(100) |  |
| PhoneNumber | Customer phone number | String(30) |  |
| Fax | Customer fax number | String(30) |  |
| ZipCode | Customer zip code | String(6) |  |
| Email | Customer E-mail | String(60) |  |
| TransactPerson | The operator | String(30) |  |
| AdminAccount | video surveillance admin account | String(64) |  |

Explanation: User use "adminAccount prefix" to login MSP Enterprise Self Service page, the user ID and user account are the same in format.

3)	Accept MSP user authentication

Accept MSP user authentication, including user’s login through the mobile phone and the enterprise administrator.

# Table 5-1-5: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform does not have the customer domain, only need to fill in the user name. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| AccountType | User account type | Int(1) | 0 for user 1 for enterprise administrator |

Explanation: The account is MSP user account or the enterprise administrator account which has prefix processed.

# Table 5-1-6: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Authorization | Certification results | Int(1) | 0 for failed, 1 for success |
| The following list of fields |  |  |  |
| CategoryID | Catalogue number | String(128) |  |
| CategoryName | Catalogue name | string(128) |  |
| PID | The parent ID | string(128) | The parent directory CategoryID When the root directory PID element value defined for null values |

4)	User account query under the customer

The enterprise administrator could query for the list of all user accounts opened by enterprise.

# Table 5-1-7: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerId | Enterprise ID | string(64) |  |

Explanation: the first six bits of  customerID refer to the domain ID.

# Table 5-1-8: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| ResultCode | Return code | Int(1) | 0 for enterprise ID not exist 1 for accession success |
| Account | User account | String(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |

5)	Surveillance point query where user attribution

Query the authorized surveillance list under the specific user account.

# Table 5-1-9: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| CategoryID | Catalogue number | string(64) | Catalogue numbers is a null value representing the query root directory |

Explanation: The account is MSP user account which has prefix processed.

# Table 5-1-10: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| PuId-ChannelNo | PU Channel Number | string(128) |  |
| PuName | PUname | string(128) |  |
| PuProperty | PUProperty | structure type | Include ptzFlag, playbackFlag, online parameters at present, |
| PtzFlag | Control authority flag | int(1) | 0 for no authorization; 1 for authorization |
| PlaybackFlag | Playback authority flag | int(1) | 0 for no authorization; 1 for authorization |
| Online | Online flag | Int(1) | 0 for offline; 1 for online |
| ResultCode | Return code | Int(1) | 0 for no user account; 1 for success |

6)	Query the VAU entrance

Query the RTSP address of VAU entrance、PTZ entrance.

# Table 5-1-11: reqVauUrlReques Interface

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| M_cuIp (Optional) | M_CU’s IP address | String(32) | Because VAU can span multiple network deployment. This parameter is provided to CMUand used to determine the return VAU which segment address. By default M_CU fill in 0.0.0.0, CMU returns VAU foreign service public address. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | Surveillance point unique identifier | String(128) |  |
| StreamingType | Streaming media type | Int(4) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |
| PlayMethod | Play type | Int(1) | 0: real-time video |

Response message (video surveillance system ->MSP)

# Table 5-1-12: reqVauAddResponse Interface

| CMU-MSP interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CMUPtzUrl | CMU PTZ URL address | String(128) | Return CMU PTZ URL address |
| VideoPlayUrl | Direct access entrance | String(128) | Return Direct access entrance URL address (PSS or VAU address) |

Among these, in addition to the VideoPlayUrl VAU/PSS address, it should contain the following fields:

# Table 5-1-13: contain fields

| Parameter | Parameter name | Data type | Notes |
| --- | --- | --- | --- |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | Surveillance point unique identifier | String(128) |  |
| StreamingType | Streaming media type | Int(4) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |
| PlayMethod | Play type | Int(1) | 0: real-time video (fixed for 0 at present) |
| ExpireTime | Requesting the link failure time | String(16) | 20120406155530 Year-month-day-hour-min-sec |
| HashToken | MD5 abstract | String(64) | Sixteen hexadecimal calculated by the MD5 (Account=xx& Password =MD5 (password) &PuId-ChanelId=xx&StreamingType=xx& ExpireTime=xx) |

HashToken requirements:

- If the HashToken parameter values in VideoPlayUrl was artificially modified, VAU should denied accessment;

- If VideoPlayUrl video link requests time exceeds the ExpireTime failure time, the link is invalid;

Video streaming could play sustainably in the established RTSP media channels, not subject to ExpireTime time limit;

## 5.2	Reference point Msm: MSP-M_CU

### 5.2.1	Interface function

M_CU and MSP interface use Web Service interface protocol (SOAP/HTTP), mainly to complete the following functions:

- Send the service request; realize video surveillance business docking and authentication through MSP;

- Get control point play entrance;

- Check version.

### 5.2.2	Interface Protocol

The interface is hosted on the IP protocol, using the Web Service interface protocol (SOAP/HTTP).

### 5.2.3	Message Format

# Table 5-2-1: M_CU Authentication certification and Packet access

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| LoginType | Login type | Int(1) | 0: Account-password login 1: Local login |
| Version | M_CU version | String(16) | For example: V1.05.004, larger version needs to be consistent with MSP. If the version of MSP>M_CU that M_CU L / class; if M_CU>MSP version, then prompts the current version of M_CU is too high, please use the low version. |

Table 5-2-2: MSP-MCU

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter nam | Data type(unit: bit) | Notes |
| Result | Certification results | Int(2) | 0:Success 1: user name not exist 2:wrong password 3: M_CU version too new 4:M_CU version too old |
| The following list |  |  |  |
| CategoryID | CategoryID | String(128) |  |
| CategoryName | CategoryName | string(128) |  |
| PID | The parent ID | string(128) | The parent directory CategoryID When the root directory PID element value defined for null values |

Table 5-2-3: Surveillance points list query

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| M_cuIp | M_CU’s IP address | String(32) | Because VAU can span multiple network deployment. This parameter is provided to CMU and used to determine the return VAU which segment address. By default M_CU fill in 0.0.0.0, CMU returns VAU foreign service public address. |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| CategoryID | Catalogue number | string(64) | Catalogue numbers is a null value representing the query root directory |

Table 5-2-4: returns list of surveillance point

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| PuId-ChannelNo | PU number and Channel Number | String(64) |  |
| PuProperty | PU ability of PTZ control | Int(4) | If no control ability, default is 0 |
| PuName | Surveillance point name | String(64) | For prompting the user to PU the specific significance in M_CU |
| Online | Online flag | Int(1) | 0 for offline; 1 for online |

PuId-ChannelNo: Corresponding to the PU number and channel number；

PuProperty: PU ability of PTZ control

# Table 5-2-5: Video surveillance point play entrance query

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | PU number and Channel Number | String(64) |  |
| StreamingType | Streaming media type | Int(2) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |

Table 5-2-6: Returns the link address

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CMUPtzUrl | CMU PTZ URL address | String(128) | Return CMU PTZ URL address |
| VideoPlayUrl | Direct access entrance | String(128) | Return Direct access entrance URL address (PSS or VAU address) |

Table 5-2-7: The statistics of mobile phone surveillance video data flows, duration

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| BRun | Start / end flag | Int(2) | 0: Start 1: Process (heartbeat) 2:Quit |
| NStreamBit | Video data flow | Int(2) | The unit is KB |
| NStreamTime | Video data time | Int(2) | The unit is Second |

Note: the statistical MCU access to the MSP, and the total length of the video data flow from MCU to MSP report once in theory every normal exit. But the abnormal exit MCU, general users in more than 10 seconds, need to be reported to MSP every 10 seconds . MSP judges statistics according to the account to "exit" identification report shall prevail, or report the last to process shall prevail.

## 5.3	Reference point Wm: WAP GW- MSP

### 5.3.1	Interface function

Reference point Mw is between MSP and WAP GW.

It is used to return the MDN number of user’s cell phone to MSP.

### 5.3.2	Interface protocol

The WAP GW put the MDN number in the head domain of HTTP.

### 5.3.3	Message format

MSP queries the enterprise attribute information (used when handling business opening worksheet by Telecom administrator) according to the corporate identify (customer ID)

# Table 5-3-1: Interface: WAP GW->MSP

| WAP GW->MSP interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| user-agent | Terminal Type | string(256) | Identify the type of browser used in user’s terminal (255 bit), Apply for WAP1.X and WAP2.0. For terminals that do not support UAProfile, this parameter represents the type of terminal. |
| x-up-calling-line-id | MDN number(cell phone number) | String(32) | User ID, the MDN number (cell phone number) of user. |

## 5.4	Reference point Psm: M_CU- PSS

### 5.4.1	Interface function

Reference point Psm is between M_CU and PSS.

It is used to transmit data through HTTP and RTSP to:

Send service request, carrying service and user information;

Establish connection and  transmitting the real-time stream of media and real-time video;

Play video according to operations, such as pause, continue, forward and other orders.

### 5.4.2	Interface protocol

The Interface is actually between the terminal and the streaming server, it follows the TCP / IP, RTP / RTSP / RTCP protocol, as shown in the Figure 5-2.

| RTP | HTTP | RTSP |  |
| --- | --- | --- | --- |
| UDP | TCP |  | UDP |
| IP |  |  |  |

Figure 5-2  Interface protocol

And the signalling is shown in Figure 5-3.

M_CU

PSS

RTSP request

Return media stream

Return RTSP

dynamic address

HTTP connection

M_CU

VAU/ PSS

RTSP Request

Media stream

# Figure 5-3  Signalling

### 5.4.3	Message format

- HTTP interface

The interface function is realized through the parameters carried by http URL. When user clicks on the URL generated by MSP, the following parameters are acquired and transferred to PSS without processing.

See IETF RFC 2616 for HTTP message.

# Table 5-4-1: HTTP Parameters from M_CU to PSS

| M_CU→PSS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter Name | Data Type | Value |
| UserId | User ID | String(BCD20) |  |
| PUID-ChannelNo | Unique identification of surveillance point | String(BCD20) |  |
| PuProperty | Whether PU has PTZ function | Int(4) | Note: Default value is 0 if PU doesn’t have PTZ function. |
| VAUADD | VAU address and PTZ control port | String(BCD20) | Format: xxx.xxx.xxx.xxx:xxx(ip: port) Note: Default value is 0 if PU doesn’t have PTZ function. This parameter is validate only in realtime mode. |

When user requests for PSS service, the http request format is like this:

http://pssIp:port/service?userid=xx&puid-chanelid=xx&puproperty=xx&vauadd=xx。

PSS responds to the M_CU request, and sends the dynamically generated RTSP URL to end user automatically through html page jumping, the address is like this:

<head>

<meta content=”refresh” url=”0,rtsp://xxxxxxxxxxxxxxxxx”/>

</head>

- RTSP interface

RTSP message is for establishment and control of streaming transferring dialog, see RFC2326 for details.

The RTSP request sent by M_CU to PSS should carry following parameters:

# Table 5-4-2: RTSP Parameters from M_CU to PSS

| M_CU→PSS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type | Value |
| UserId | User ID | String(BCD20) |  |
| PUID-ChannelNo | Unique identification of surveillance point | String(BCD20) |  |
| PlayMetod | Media type | Int(1) | 0: Realtime 1: Video record |
| StartTime | Start time of recording | String(BCD20) | Format: YYYY-MM-DD-HH:MM:SS |
| PuProperty | Whether PU has PTZ function | Int(4) | Note: Default value is 0 if PU doesn’t have PTZ function. |
| VAUADD | VAU address and PTZ control port | String(BCD20) | Format: xxx.xxx.xxx.xxx:xxx(ip: port) Note: Default value is 0 if PU doesn’t have PTZ function. This parameter is validate only in realtime mode. |
| hashtoken | For billing authentication request | String(BCD20) | Used by manufactories for anti-stealing-link, not be processed by M_CU. |

Parameters are transmitted through:

RTSP://localhost:port/service?UserID=×××& PID= XXX……

The server of the RTSP extracts parameters while responding to the request and processes them according to the service logic.

M_CU send RTSP requests through VideoPlayUrl CMS querying by MSP.

URL is like this:

rtsp://VideoPlayUrl/service?Account=xx&PuId-ChanelId=xx&StreamingType=xx&‌ExpireTime=xx&‌HashToken=xx

## 5.5	Reference point Vm: M_CU- VAU

### 5.5.1	Interface function

Reference point Vm is between M_CU and VAU.

It is used to transmit the real-time video through RTSP.

### 5.5.2	Interface protocol

The interface uses RTSP protocol, M_CU works as the RTSP client, VAU works as the RTSP server which can be configured freely, and the default port is 554. The RTSP message format and parameters are as follow:

# Table 5-5-1: Common Parameters from M_CU to VAU

| M_CU→VAU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter Name | Data Type | Value |
| PlayMethod | Media type | String(BCD1) | 0：Realtime |

Table 5-5-2: Realtime Mode Parameters from M_CU to VAU

| M_CU→VAU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter Name | Data Type | Value |
| PUID- ChannelNo | Channel identification, the PUID and channel number in VS system | String(BCD30) | Device ID + Channel Type + Channel Number |

The format of realtime video viewing RTSP URL is like this:

rtsp://vauaddress:port/service?PUID-ChannelNo=XXXXXXXXXXXX-X PlayMethod=0, for instance: rtsp://172.24.202.190/service?PUID-ChannelNo=320101000200000001-1&&PlayMethod=0,  which refers to accessing the real time video stream of first channel of PU NO.320101000200000001.

### 5.5.3	Message format

M_CU works as the client, and VAU works as the server, the

interface meets following needs:

- Meet the minimum requirements of client and server basic playback in the Appendix D  of RFC2326 "Real Time Streaming Protocol (RTSP)".

- In accordance with the requirements of 3GPP TS 26.234, the client and server should support the RTSP DESCRIBE method.

- In accordance with the requirements of 3GPP TS 26.234, the client and server should support the RTSP Range header field.

- In accordance with the requirements of 3GPP TS 26.234, the server need to include the Range field in the responding message to PLAY.

DESCRIBE message

- M_CU sends DESCRIBE message.

- VAU returns 200 OK and messages with correct message body to M_CU, establishing the session. The message body includes default video CODEC of transferred video stream, and related profile-level-id, frame rate, frame size and so on. VAU configures the default value.

SETUP message

- M_CU sends SETUP message, describing the interface which receives the media streaming.

- VAU returns 200 OK to M_CU, describing the interface which sends the media streaming.

PLAY message

- M_CU sends PLAY message.

- VAU starts to play real time video or video record.

- VAU returns  200 OK to M_CU.

- VAU starts to send media stream to M_CU.

PAUSE message

- M_CU sends the correct PAUSE message.

- VAU pauses the real time video or video record playing.

- VAU returns 200 OK to M_CU.

- VAU stops sending media stream.

TEARDOWN message

- M_CU sends TEARDOWN message.

- VAU stops the real time video or video record playing.

- VAU returns 200 OK to M_CU.

- VAU stops sending media stream.

- VAU RTSP SOCK stops.

OPTIONS message

- VAU sends OPTIONS message to M_CU (with Session).

- M_CU searches for the target Session.

- If the target Session exists, M_CU returns 200 OK. If the target Session doesn’t exist, M_CU returns 404 error, and VAU disconnect the Session with CMS and release the source.

## 5.6	 Reference point Mv: M_CU-VAU

### 5.6.1	Interface function

Reference point Mv is between M_CU and VAU.

It is used to control PTZ for end-users when browsing real-time video.

### 5.6.2	Interface protocol

The communication  between M_CU and  VAU is according to UDP protocol. The default communication port is 5060. The pack for request and response reserves 200 bytes for via fields in head of SIP.

M_CU sends the PTZ control messages to VAU without 200OK response or resend. But if within the definition time M_CU doesn’t send the stop message to VAU, VAU will send PTZ stop message to the platform automatically.

Considering several M_CUs visit one PUID-ChannelNo at the same time, M_CU will send the PUID-ChannelNo carried by RTSP request to VAU.VAU distinguishes PTZ message target device according to the PTZ message body.

<IE_HEADER MessageType="MSG_PTZ_SET_REQ"

UserID=”User name”

DestID = "PUID-ChannelNo"

/>

- M_CU

M_CU sends request and receives response .When it receives response, M_CU must check XML

Message_Type and Sequence_Number.  If the parameter is wrong, the response must be discarded.

- VAU

VAU should be able to receive request from any IP address and port, and reply the port from  M_CU request.

### 5.6.3	Message format

generic-message = start-line

*message-header

CRLF

[ message-body ]

- start-line   start-line = Request-Line / Status-Line

SIP  request:

Request-Line = Method SP Request-URI SP SIP-Version CRLF

SIP response:

Status-Line = SIP-Version SP Status-Code SP Reason-Phrase CRLF

- message-header

Request  for message-header:

INFO sip:xxx

Content-Type: application/global_eye_v10+xml

Content-Length: xxx

To: xxx

From: xxx

CSeq: xxx INFO

Call-ID: xxx

Max-Forwards: xxx

Via: xxx

Contact: xxx

[CrLF]

Response  to message-header:

200 OK sip:xxx

Content-Type: application/global_eye_v10+xml

Content-Length: xxx

To: xxx

From: xxx

CSeq: xxx INFO

Call-ID: xxx

Max-Forwards: xxx

Via: xxx

Contact: xxx

[CrLF]

- Message-body

PTZ control

a)	Function: The user platform sends PTZ control message to another platform.

b)	Message:

<!—PTZ control  -->

<?xml version="1.0" encoding="UTF-8" ?>

<Message Verison="1.0">

<IE_HEADER MessageType="MSG_PTZ_SET_REQ"

UserID=”User name”

DestID=”PUID-ChannelNo”

/>

<IE_PTZ OpId="code"

Param1=" parameter 1"

Param2=" parameter 2"

/>

</Message>

This message needs no response.

- Code and parameter

1: turn up (1--10)

2: turn down (1--10)

3: turn left (1--10)

4: turn right (1--1)

5: open iris (1--10)

6: close iris   (1--10)

7: zoom in (1--10)

8: zoom out (1--10)

9: focus near (1--10)

10: focus far (1--10)

11: RESERVED

12: RESERVED

13: RESERVED

14: RESERVED

15: stop

20: scan off

21: scan on (1--10)

24: RESERVED

25: RESERVED

26: RESERVED

## 5.7	 Reference point Vp: VAU-PSS

### 5.7.1	Interface function

Reference point Vp is between VAU and PSS.

It is used to transfer performance data, alarm data and topological data.

### 5.7.2	Interface protocol

Interface protocols areHTPP＋XML, SOAP or CORBA for  different platforms and systems.

### 5.7.3	Message format

#### 5.7.3.1	 VAU interface

It is used to control and manage VAU according to the definition of MIB.

- Configuration management

VAU information model as in Table 5-7-1, Table 5-7-2, Table 5-7-3.

# Table 5-7-1: VAU interface

| Name | Data type | Read/Write |
| --- | --- | --- |
| VAU device ID | String | R/W |
| Version number | String | R/W |
| Equipment manufacturer code | String | R/W |
| Equipment manufacturer name | String | R/W |
| IP network parameter | String | R/W |
| Maximum number of video input | Integer | R/W |
| Maximum number of video output | Integer | R/W |
| Transcoding type | String |  |
| Platform number | Integer |  |

Table 5-7-2: VAU multi –platform data model

| Name | Data type | Read/Write |
| --- | --- | --- |
| CMS device ID | String | R/W |
| IP address | String | R/W |
| Port | String | R/W |

- state management

# Table 5-7-3: VAU state management information in system model

| Name | Date type | Read/Write |
| --- | --- | --- |
| VAU device ID | String | R |
| CPU usage | String | R |
| Memory usage | String | R |
| Current processes number | Integer | R |
| Current thread number | Integer | R |
| Registration status | String | R |
| Current video inputs | Integer | R |
| Current video outputs | Integer | R |
| Current transcoding number | Integer | R |
| VAU heartbeat statue | String(1) | R |

#### 5.7.3.2	PSS interface

It is the MIB definition of PSS management, which can be used to control and manage PSS

- Configuration management

# Table 5-7-4: PSS configuration management information in system model

| Name | Data type | Read/Write |
| --- | --- | --- |
| PSS device ID | String | R/W |
| Version Number | String | R/W |
| Equipment manufacturer code | String | R/W |
| Equipment manufacturer name | String | R/W |
| Maximum number of video input | Integer | R/W |
| Maximum number of video output | Integer | R/W |

- Statue management

# Table 5-7-5: PSS state management information in system model

| Name | Data type | Read/Write |
| --- | --- | --- |
| PSS device ID | String | R |
| CPU usage | String | R |
| Memory usage | String | R |
| Current processes number | Integer | R |
| Current thread number | Integer | R |
| Registration status | String | R |
| Current video inputs | Integer | R |
| Current transcoding number | Integer | R |
| PSS heartbeat statue | String (1) | R |

## 5.8	 Reference point Mdv/Pv/Vs:VAU-VS system

Reference point Mdv, Pv, Vs are all between VAU and the VS system which are all internal interfaces, there is no specific definition in this recommendation.

# 6	Signalling and protocols

The following is some illustrative items:

- The SIP message format should comply with the definition in RFC3261;

- The RTSP message format should comply with the definition in RFC2326;

- The HTTP message format should comply with HTTP1.1 defined in RFC2616;

- RTP/RTCP signaling and message refers to RFC3550;

## 6.1	Real-time video/audio acquisition

The Real-time Video/Audio Acquisition is used to view the real-time surveillance video/audio data by mobile terminals. After MSP performs authentication and returns the VAU information, the M_CU chooses and requests to the VAU for real-time video/audio data.

# Figure 6-1  High level procedural flows for real-time media acquisition

- The M_CU sends  RTSP DESCRIBE (PUID-ChannelNo)  to the VAU.

- The VAU returns response message to the M_CU according to parameter configuration.

- The M_CU sends RTSP SETUP request to the VAU.

- The VAU distributes a channel and returns RTSP 200OK response message to the M_CU.

- The M_CU sends RTSP PLAY request to the VAU for real-time video surveillance.

- The VAU chooses the CMU according to PUID.

- The VAU starts session request to the CMU.

- The CMU returns session response to the VAU. If the VAU provides video format message body, the CMU chooses codec that the PU supports and returns the message body to the VAU. If the VAU doesn’t offer message body info, the CMU returns the message body carrying all capacities the PU supports to the VAU.

- The VAU judges the media data format and decides whether to start the transcoding process. The VAU starts the connection between PUID-ChannelNo and CallID , and creates a RTSP Session NO.

- The VAU sends session control response message to the M_CU.

## 6.2	Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

### 6.2.1	Playback in platform

When users want to playback the surveillance video using mobile phones, they must log into the

MSP by the M_CU. The MSP performs authentication and returned the playback surveillance list.

After the users choose one surveillance point name, the M_CU sends a request for this surveillance

video to the VAU, and then VAU returns the media stream to the M_CU to play.

# Figure 6-2 Video Playback from the platform procedural flows

- The M_CU sends  http video search request(PUID-ChannelNo) to the MSP.

- The MSP sends http video search request(PUID-ChannelNo) to the CMU.

- The CMU returns videos list info to the MSP.

- The MSP returns the videos list info to the M_CU. The videos gets the unique Record-No in the CMU.

- The M_CU sends RTSP DESCRIBE (PUID-ChannelNo,Record-No)  to the VAU.

- The VAU returns response message to the M_CU according to parameter configuration.

- The M_CU sends RTSP SETUP request to the VAU.

- The VAU distributes a channel and returns RTSP 200OK response message to the M_CU.

- The M_CU sends RTSP PLAY request to the VAU for videos record (PUID-ChannelNo, Record-No).

- The VAU chooses the CMU according to PUID and Record-No.

- The VAU starts session request to the CMU. The request message carries message body including video format the VAU supports.

- The CMU returns session response to the VAU. If the VAU provides video format message body, the CMU chooses codec the PU supports and returns the message body to the VAU. If the VAU doesn’t offer message body info, the CMU returns the message body carrying all capacities the PU supports to the VAU.

- The VAU decides whether to start the transcoding process according to the media data format. The VAU starts the connection between PUID-ChannelNo ,RecordNo and CallID, and creates a RTSP Session NO.

- The VAU sends session response message to the M_CU .

- The M_CU sends RTSP  TEARDOWN to the VAU in order to remove the real-time session.

- The VAU sends the end request to the CMU to stop session.

- The CMU sends the end response to the VAU to stop session.

18. The VAU sends the end response to the M_CU to stop session.

### 6.2.2	Playback in PU

# Figure 6-3  Video Playback from the PU procedural flows

- The M_CU sends  http video search request(PUID-ChannelNo) to the MSP.

- The MSP sends http video search request(PUID-ChannelNo) to the CMU.

- The CMU sends SIP(INFO) request to the PU for videos list info(PUID-ChannelNo).

- The PU returns videos list info to the CMU.

- The CMU returns videos list info to the MSP.

- The MSP returns the videos list info to the M_CU. The videos get the unique Record-No in the CMU.

- The M_CU sends RTSP DESCRIBE (PUID-ChannelNo, Record-No) to the VAU.

- The VAU returns response message to the M_CU according to parameter configuration.

- The M_CU sends RTSP SETUP request to the VAU.

- The VAU distributes a channel and returns RTSP 200OK response message to the M_CU.

- The M_CU sends RTSP PLAY request to the VAU for videos record (PUID-ChannelNo,  Record-No).

- The VAU chooses the CMU according to PUID and  Record-No.

- The VAU starts session request to the PU. The request message carries message body including videos format the VAU supports.

- The PU returns session response to the VAU. If the VAU provides video format message body, the CMU chooses codec the PU supports and returns the message body to the VAU. If the VAU doesn’t offer message body info, the CMU returns the message body carrying all capacities the PU supports to the VAU.

- The VAU decides whether to start the transcoding process according to the media data format. The VAU starts the connection between PUID-ChannelNo, RecordNo and CallID, and creates a RTSP Session NO.

- The VAU sends session response message to the M_CU.

- The M_CU sends RTSP  TEARDOWN to the VAU in order to disconnect the real-time session.

- The VAU sends the disconnect request to the PU to stop session.

- The PU sends the disconnect response to the VAU to stop session.

20. The VAU sends the disconnect response to the M_CU to stop session.

## 6.3	PTZ control

The PTZ Control function is used to pan, tilt and zoom cameras, switch auxiliary peripherals (such as flashlight, fan and rain brush), and to preset the position of related physical devices.

# Figure 6-4  PTZ control procedural flows

- The M_CU sends PTZ control request to the VAU .The message body carries UserID and DestID (DestID  is PUID-ChannelNo).

- The VAU sends PTZ code to the CMU. The code is that the VAU matches routing table

according to DestID.

- The CMU sends PTZ control response to the VAU.

- The M_CU sends SIP INFO PTZ stop request to the VAU

- If the VAU receives the stop request from the M_CU within configuration time, the VAU sends PTZ stop request to the CMU, otherwise, the VAU sends PTZ stop request to the CMU automatically.

- The CMU sends PTZ stop response to the VAU.

- The M_CU sends RTSP TEARDOWN to the VAU in order to disconnect the real-time session.

- The VAU sends the disconnect request to the CMU to stop session.

- The CMU sends the disconnect response to the VAU to stop session.

- The VAU sends the disconnect response to the M_CU to stop session.

## 6.4 	Resource query

# Figure 6-5  USER resource query procedural flows

- USER log in the MSP using MSISDN or USERID and password.

- The MSP gets MSISDN-No  from MSISDN, then recognizes user type from MSISDN-No. At last the MSP translates user type and MSISDN-No  into UserID.

- The MSP queries UserID in the CMU.

- The CMU queries user resource list according to UserID.

- The CMU returns the list and user information to the MSP including PUID-ChannelNo, PUName, PuProperty.

- The MSP builds web page according to uesr order mobile service.

- The MSP returns the PSS-URL and mobile service order to user.

# Bibliography

[b-ITU-T H.Sup.1]	ITU-T H-series Recommendations – Supplement 1 (1999), Application profile – Sign language and lip-reading real-time conversation using low bit rate video communication.

_____________________