| Rapporteur Meeting of Question 21/16 |  | TD-12 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | STUDY GROUP 16 |
|  |  | Original: English |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |
| RAPPORTEUR MEETING DOCUMENT |  |  |
| Source: | Editors F.EMSarch |  |
| Title: | F.EMSarch "Requirements and architecture for energy management service" (New): Output draft (Changzhou, 26-29 September 2016) |  |
| Purpose | Proposal |  |

This document contains the editor’s output draft for F.EMSarch "Requirements and architecture for energy management service" based on the following contribution and discussions held at the Rapporteurs meeting in Changzhou, 26 - 29 September 2016.

| AVD-4797 | F.EMSarch: Proposal to update clauses 3, 4, 6, 7, 8 | MIIT, China Telecom | Q21/16 |
| --- | --- | --- | --- |

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 4 3.1 Terms defined elsewhere 4 3.2 Terms Defined within this document 5 4 Abbreviations 5 5 Convention 5 6 Overview 5 7 Functional architecture for user-side EMS 6 7.1 Supported energy metering devices 7 7.2 Supported energy control device 8 8 Scenarios for user-side EMS 9 8.1 User-side EMS for domestic users 10 8.2 User-side EMS for public buildings 11 8.3 User-side EMS for factory cases 12 9 General Requirements for user-side EMS 13 9.1 Requirements for device management 13 9.2 Requirements for data acquisition 13 9.3 Requirements for policies management 13 9.4 Requirements for energy consumption analysis 13 9.5 Requirement for energy-saving decisions implementation 14 10 Security Consideration 14 Bibliography 14

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 7-1 Framework of a user-side EMS 7 Figure 7-2 Ordinary electricity meter, ordinary gas meter and socket with metering 8 Figure 7-3 Electricity meter and gas meter supporting AMR 8 Figure 7-4 Conventional power switch and gas switch 9 Figure 7-5 Smart socket and gas switch gas meter supporting remote control 9 Figure 8-1 Scenarios of EMS for domestic user 10 Figure 8-2 Application area of the user-side EMS for public buildings 11 Figure 8-3 Scenario of the user-side EMS for public buildings 12

Draft new Recommendation F.EMSarch

Requirements and Architecture for Energy Management Service

AAP Summary

[To be provided before Consent]

Summary

[To be provided before Consent]

Keywords

[Mandatory material]

Scope

This Recommendation describes requirements, scenarios and functional architecture for user-side Energy Management Service (EMS) where energy consumption equipment with heterogeneous metering and control capacities coexist. This Recommendation covers the following:

- Scenarios of user-side EMS

- Requirements of user-side EMS

- Functional architecture for user-side EMS

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[TBD]

Definitions

  - 3.1 Terms defined elsewhere

This document uses the following terms defined elsewhere:

3.1.1	device [ITU-T Y.3021]:  This is the material element or assembly of such elements to perform a required function.

3.1.2	home energy management system(HEMS) [ITU-T Y.2070]:  A computer system comprising a software platform providing basic support services and a set of applications providing the functionality needed for the effective operation of home equipment, such as home appliances and storage batteries, so as to assure adequate security of energy supply at minimum cost.

3.1.2	smart object [ITU-T Y.2064]:  An object which is aware of its characteristic, context and situation. It shares and processes information, such as its identity, current location, physical properties and the information it senses from its surroundings, while performing object-to-object communications.

  - 3.2 Terms Defined within this document

This document defines the following terms

3.2.1	Domestic user: a family.

3.2.2	Public building: Public building is a general term for civil architecture which covers all of those buildings used for social activities except individual family. It mainly includes big buildings (e.g. office premises, shopping malls, hotels, hospitals, residential building for many families etc.), transportation hub (e.g. airports, bus stations, etc.) and sports & entertainment buildings (e.g. public gymnasiums and stadiums)

3.2.3	Energy control device: a device which can prevent or change the transmission of energy thus change the state(s) of one or multiple energy-consumption devices.

Abbreviations

| EMS | Energy Management Service |
| --- | --- |
| HEMS | Home Energy Management System |
| IoT AMR | Internet of Things Automatic Meter Reading |

Convention

In this Recommendation:

- The expression "is required to" indicates a requirement which must be strictly followed and from which no deviation is permitted if conformance to this Recommendation is to be claimed.

- The expression "is recommended" indicates a requirement which is recommended but not absolutely required. Thus this requirement need not be present to claim conformance.

Overview

There are growing concerns about energy efficiency and emissions reduction among people from all aspects of life, which raises the demand of energy management for customers. On the other hand, with the development of information technology (e.g., Internet of Things (IoT) [b-ITU-T Y.2064]), various smart objects, including sensors and actuators, can interconnect with each other to monitor and change the states of energy consumption equipment, which provides methods for energy management. Thus, user-side energy management service (EMS) is used to help consumers to improve the energy efficiency as well as reduce their energy bill.

The user-side EMS provides raw data as well as processed information of energy consumption to users by collecting and monitoring energy consumption data of various equipment. It also makes energy-saving decisions based on multiple factors such as user demands, supplier’s pricing strategies, and optimization policies. Some other factors, such as government subsidy, device status and environmental conditions can also affect the energy-saving decisions. For example, usage related information, such as the number of customers shopping in the mall, the occupancy rate of the hotel, the number of students enrolled in the school, and attendance at stadium, can affect the energy-decision, e.g., which lights and air-conditioners need to be turned on.

Recent years, various EMS have been developed and standardized. [b-ITU-T Y.2064] describes requirements and capabilities for saving energy by using smart objects in home networks. Home energy management system (HEMS) [b-ITU-T Y.4409] supports energy efficiency and reduction of energy consumption by monitoring and controlling devices. All these existing standards only consider the smart objects which can be monitored and controlled automatically and remotely.

However, not all existing energy consumption devices are smart objects. They are not only different in the type of consumed energy (e.g., electricity, gas or coal), but also different in the capacity of metering and control. For example, in practice, an electricity meter can measure the amount of electric energy consumed by one circuit, which may be wired with multiple electrical equipment located in a home or in a factory. It can also be wired with an individual electrical equipment. Thus, both coarse-grained and fine-grained data of consumed electric energy are collected manually or through automatic meter reading (AMR). On the other hand, many electrical equipment can only be turned on and off manually, e.g., light bulb with on-off switches. Some other electrical devices can be fine-tuned, e.g., temperature setting of air-conditioner. Furthermore smart appliances can be controlled automatically and remotely.

Considering the cost of adding metering unit for each individual energy consumption equipment and the difficulties of updating all traditional devices to smart objects, various heterogeneous devices with different capacities of metering and control must be supported in a practical user-side EMS system. In such EMS system, energy consumption data for both single device or device group are needed to collected, and these devices can be controlled by coarse-grained (on-off state) and fine-grained instructions. Furthermore, such operations are executed manually or automatically.

This document describes the requirements and scenarios of the user-side EMS as well as the functional architecture for user-side EMS with heterogeneous devices.

Functional architecture for user-side EMS

The functional architecture of user-side EMS is shown in Figure 7-1.

As shown in the Figure 7-1, the user-side EMS is interacted with EMS clients, supply-side EMS and various energy-consumption equipment. There are six modules of user-side EMS to interact with other systems and provide energy management for users.

- The device management module registers and de-registers various energy consumption equipment and other devices (i.e., meters and controllers). The relationship among the energy consumption equipment, metering and control devices are maintained. The attribute information of energy consumption equipment, meter devices, and control devices are also maintained.

- The metering data management module is responsible for recording real-time energy consumption data for each energy consuming device or device group. The consumed data of all types of energies, including gas, electricity, coal and other energy resources, are collected and submitted to the metering data management module manually or through AMR.

- The control management module is responsible for generating control instruction which is executed manually or automatically

- The strategy module is responsible for interacting with EMS clients and maintain user-side strategies. The EMS client collects the user demands, including the energy consumption demand, efficiency goals, and energy saving strategies, and sends them to this module.

- The supply-side price module keeps record of the pricing policies and energy plan provided by the supply-side EMS.

- The energy consumption data processing and optimization module performs the energy consumption statistics and make energy-saving decisions.  The statistic results are sent to supply-side EMS which can be used to help energy supplier make energy supply plan and pricing policies. The results also are sent to the EMS client for displaying.

Figure 7-1 Framework of a user-side EMS

In clauses 7.1 and 7.2, an outline of supported metering and control devices is described.

  - 7.1 Supported energy metering devices

The acquisition of energy consumption data provides the input data for user-side EMS, two steps are involved: energy consumption metering and data collection.

Metering devices can be classified based on the number of monitored devices and the way of metering data collection.

According to the number of monitored devices, there are two types of metering devices:

- Meters which measuring the energy consumption data of one device. For example, with the emergence of socket with metering, the consumed power of a specific device can be metered independently.

- Meters which measuring the energy consumption data of one device group (a set of devices). For example, one conventional electricity meter could only meter the total power consumption for one ordinary house, which means all the household electrical appliances are wired with the same circuit and considered as a whole.

According to the way of metering data collection, there are two types of metering devices:

- Meters whose data must be collected manually.

- Meters which supporting AMR.

Figure 7-2 below shows a conventional electricity meter, a conventional gas meter and a socket with metering, the measurement data of which is collected manually.

Figure 7-2 Ordinary electricity meter, ordinary gas meter and socket with metering

Two meters supporting remote meter reading are shown in Figure 7-3.

Figure 7-3 Electricity meter and gas meter supporting AMR

The shortcomings of collecting data manually include high cost of human resources and inefficiency of data collection. However, updating all meters to support AMR is very expensive. In a home, there are various household energy-consumption equipment such as lightbulbs, chargers, air conditioners/fans, refrigerator, electric/gas water heater, gas stove, etc. It is unnecessary to add meters for each device. Thus, in practice, one metering device is often used to measure the energy consumption data of one device group and most metering devices are still needed to collect data manually.

For  user-side EMS, the device diversity on metering capacity and data collection method must be fully taken into account.

  - 7.2 Supported energy control device

An energy control device can prevent or change the transmission of energy thus change the state(s) of one or multiple energy-consumption devices.

Energy control devices can be classified based on the number of controlled devices, the way of control and the granularity of control.

According to the number of controlled devices, there are two types of energy control devices:

- Energy control device which controls one device. For example, one power switch controls one light bulb, i.e., one-to-one mode.

- Energy control device which controls one set of devices. For example, one power switch controls one set of light bulbs, i.e., one-to-many mode.

- According to the way of control, there are two types of energy control devices: Energy control devices which are manually operated. For example, one conventional power switch and one gas switch shown in Figure 7-4.

- Energy control devices which can be operated automatically and remotely. For example, user controls on-off state of the smart sockets and smart appliances remotely as shown in Figure 7-5.

According to the granularity of control, there are two types of energy control devices:

- Energy control devices which control the “on” an “off” states of the energy-consumption equipment.

- Energy control devices which fine-tune the consumed energy of energy-consumption equipment. For example, adjusting the temperature setting of a room changes the consumed energy of the air conditioner in that room.

Figure 7-4 Conventional power switch and gas switch

Figure 7-5 Smart socket and gas switch gas meter supporting remote control

Scenarios for user-side EMS

In the domestic usage case, the user-side EMS is used to serve the individual families or houses which are the basic units of human society. User-side EMS could not only save energy but also enhance public awareness of energy saving.

In the case of user-side EMS for public buildings, the application area of user-side EMS is expanded from inside a building or house for a family to an architectural complex.

In the case of user-side EMS for factory, various types of energy resources, such as electricity, gas, coal and heat, are involved.

There exist great opportunities to achieve more energy efficiency in these scenarios.

The study of other scenarios is encouraged and welcomed.

  - 8.1	User-side EMS for domestic users

Figure 8-1 Scenarios of EMS for domestic user

The scenarios of user-side EMS for domestic user are shown in Figure 8-1.

In a family, there are two common types of energy consumption equipment, namely electric equipment and gas equipment. The electric equipments include lights, decorative lamps, television, washing machine, refrigerator, air conditioner, electric water heater, rice cooker/steamer, and many others. The typical gas equipment includes gas water heater, gas cooker and gas heater.

The user-side EMS collects and processes the energy consumption data of these devices to provide the raw data as well as processed information of energy consumption to home users. The EMS system helps users to understand how their own behaviours affect the usage of energy.

The EMS also provides users with guidelines or suggestions about how to improve the energy efficiency as well as save energy. The pricing policies of supply side (e.g., the power companies and gas companies) can affect these decisions. For instance, in order to reduce energy demand during peak hours, power companies usually introduce step-tariff, i.e., charging higher price at peak hours and lower price at off-peak hours to regulate energy consumption. Thus, if a washing machine has a timer or can be controlled remotely, the EMS may suggest doing the laundry at off-peak hours. And the user does this by setting the timer manually or the EMS starting the washing machine remotely at the right time.

Another typical scenario is related to the energy saving decision for bathing. For an electric water heater, companying with a socket with meter, the consumed energy required for a bath can be metered and submitted to the EMS manually. Similarly, the consumed gas required for a bath can also be collected. Then, The EMS can calculate the fee paid for one bath based on the consumed gas or electric power and recommend appropriate energy saving decision to user.

  - 8.2 User-side EMS for public buildings

Public building is a general term for civil architecture which covers all of those buildings used for social activities rather than individual family. It mainly includes big buildings (e.g. office premises, shopping malls, hotels, hospitals etc.), transportation hub (e.g. airports, bus stations, etc.) and sports buildings (e.g. public gymnasiums and stadiums). As shown in Figure 8-2, the application area of the user-side EMS for public building is expanding from inside a building or house for a family as the EMS for domestic users to an architectural complex.

Figure 8-2 Application area of the user-side EMS for public buildings

Figure 8-3 shows one scenario of the user-side EMS for public buildings. In order to create a comfortable environment, public buildings are generally equipped with adequate lighting and air-condition facilities. For example, in northern China, central heating system is often employed in winter. Typically, this central heating system is charged by the size of the building regardless of the amount of actual consumed energy. Ministry of Housing and Urban-Rural Development of China is now vigorously promoting the mandatory installation of meters for heating in new constructions, and also force to install meters in existing buildings, which means people need to pay according to the actual amount of consumed heat. Thus, reducing energy consumption and the cost will become an important task for public building management.

It is worth noting that the consumed energy for a building is always varying. For example, after normal working hours, some people may still stay in the buildings to work. In this case, the building manager can continue using the central heating system or using air conditioner in the appropriate rooms the building. The building manager can manually count which rooms are still in use and offer this information to the EMS which will make decision to save energy.

Figure 8-3 Scenario of the user-side EMS for public buildings

  - 8.3 User-side EMS for factory cases

A large portion of energy is consumed by factories such as steel, mining, petroleum, chemicals, metallurgy, tobacco, food processing, textile manufacturing, etc.

Due to lack of technologies and money, the energy consumption information of many factories, especially small and medium-sized enterprises, is still collected and managed manually. As for many large-scale and energy-intensive industries, the user-side EMS is still in its early stage,

The EMS for factory manages multiple energy resources with large-scale distribution. For example, in a coal mine, there are a large amount of devices (e.g., central pump, winches, shearers, coal feeders, belt conveyors and air compressors) which are located at different places and are powered by different resources (e.g., coal, electricity and gas). Diversified data must be collected.

What is more, the energy saving decisions are influenced by some policies such as government’s subsidy or other incentive mechanism, and mandatory regulations. The security of the EMS related data and information is also critical.

The EMS is helpful for finding the composition of energy use, predicting the trend of energy consumption. Besides, the accurate data collected by the EMS can be used by energy suppliers such as power companies to improve the efficiency, reliability, and sustainability of the production and distribution of electricity. It can also be used by regulators to make better energy related polices.

General Requirements for user-side EMS

Several basic requirements of the user-side EMS are given, including device management, data acquisition, policies management, energy consumption analysis, and decisions implementation.

  - 9.1 Requirements for device management

DM-01: The user-side EMS is required to register and deregister energy consumption equipment, metering devices and controlling devices.

DM-02: The user-side EMS is required to maintain attribute information of energy consumption equipment, meter devices, and control devices.

DM-03: The user-side EMS is required to associate the energy consumption equipment with the corresponding metering device(s) and control device(s).

  - 9.2 Requirements for data acquisition

DA-01: The user-side EMS is required to collect and store the energy consumption data for each energy consuming device or device group.

DA-02: The user-side EMS is required to provide the raw data as well as pre-processed data for further energy consumption calculation, optimization, and device control.

DA-03: The user-side EMS is required to support at least one kind of mechanism (i.e., manual or automatical) of metering data collection.

DA-04: The user-side EMS is recommended to implement a metering adaptation layer which supports various data collection mechanisms, such as remote and manual meter reading.

DA-05: The user-side EMS is recommended to include the consumption of electricity, natural gas, wood, coal, gasoline, diesel and other energy resources as energy consumption data, if applicable.DA-06: The user-side EMS is recommended to collect environmental data periodically.

  - 9.3 Requirements for policies management

PM-01: The user-side EMS is recommended to fetch and store energy consumption demands and optimization policies of users.

PM-02: The user-side EMS is recommended to fetch and store the pricing policies of supply side.

  - 9.4 Requirements for energy consumption analysis

EA-01: The user-side EMS is required to process the collected energy consumption data and provide the statistical energy consumption report to the user and the supplier if required.

EA-02: The user-side EMS is recommended to provide an energy efficiency evaluation. For domestic users and the public buildings, the evaluation is based on the amount of energy used, the number of people, and the environmental conditions. For factory, the evaluation takes the amount of production into consideration.

EA-03: The user-side EMS is recommended to provide data analysis such as statistical analysis, energy consumption prediction.

EA-04: The user-side EMS is recommended to generate an optimal energy saving solution based on all the data and information collected, the policies related, and the demands of user.

-EA-05: The user-side EMS is recommended to keep track of the change of the energy consumption for one device or device group after applying each control command.

  - 9.5 Requirement for energy-saving decisions implementation

ED-01: The user-side EMS is required to have the capability for users to implement the energy saving decisions manually.

ED-02: The user-side EMS is recommended to implement the energy saving decisions automatically.

ED-03: The user-side EMS is recommended to implement controlling adaptation layer which supports various control mechanisms, such as remote and manual control.

Security Consideration

It is recommended that the security requirements of [b-ITU-T Y.2201], [b-ITU-T Y.2701], and applicable X, Y and M series of ITU-T security Recommendations be taken into consideration; this included access control, authentication, data confidentiality, communications security, data integrity, availability and privacy.

Bibliography

[b-ITU-T Y.2064]	Recommendation ITU-T Y.2064 (2014), Energy saving using smart objects in home networks.

[b-ITU-T Y.4409]	Recommendation ITU-T Y.4409 (2015), Requirements and architecture of the home energy management system.

[b-ITU-T Y.3021]	Recommendation ITU-T Y.3021 (2012), Framework of energy saving for future networks.

[b-ITU-T Y.2201]	Recommendation ITU-T Y.2201 (2009),  Requirements and capabilities for ITU-T NGN.

[b-ITU-T Y.2701]	Recommendation ITU-T Y.2701 (2007),  Security requirements for NGN release 1.

_________________