| RAPPORTEUR MEETING OF QUESTION 21/16 |  |  |  | TD-13 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.VCDN.1-NV |  |  |  |
| Title: | H.VCDN.1-NV “Virtual content delivery network – Network virtualization” : Revised text (Geneva, 23 May-3 June 2016) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This TD contains the revised text of draft of new Recommendation H.VCDN.1-NV “Virtual Content Delivery Network – Network Virtualization”, after the following contributions and the outcome of the discussions in Q21/16.

| AVD-4789 | 21 | H.VCDN.1-NV: proposed updated Figure 6-1and the related content | ZTE |
| --- | --- | --- | --- |
| AVD-4790 | 21 | H.VCDN.1-NV: proposed functions and functional blocks in Resource Control Functions | ZTE |
| AVD-4791 | 21 | H.VCDN.1-NV: proposed content of loose coupled SDN-based network virtualization | ZTE |
| AVD-4792 | 21 | H.VCDN.1-NV proposed use case of vCDN nodes deployment | ZTE |
| AVD-4793 | 21 | H.VCDN.1-NV proposed use case of vCDN: smart traffic control | ZTE |
| AVD-4794 | 21 | H.VCDN.1-NV proposed use case of vCDN: quickly service deploying | ZTE |
| AVD-4795 | 21 | H.VCDN.1-NV proposed use case of vCDN: unified configuration and management | ZTE |

CONTENTS

Page

1 Scope 3 2 References 3 3 Definitions 3 3.1 Terms defined elsewhere 3 3.2 Terms defined within this document 4 4 Acronyms and abbreviations 4 5 Convention 4 6 Introduction 4 7 Functions and Functional blocks of network virtualization under VCDN architecture 5 7.1 Functions and Functional blocks in Recourse Control Functions 5 7.1.1 Virtual Network Application Functions 6 7.1.2 Virtual Network MANO Functions 6 7.2 Functional blocks in Recourse Functions 7 8 Network virtualization for VCDN 7 8.1 SDN-based network virtualization 7 8.1.1 Tight coupled SDN-based network virtualization 7 8.1.2 Loose coupled SDN-based network virtualization 9 9 Reference points 11 Appendix I Use cases of network virtualization supporting VCDN services 11 Bibliography 14

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 6-1: Functions related to network virtualization in VCDN architecture 5 Figure 7- 1: Functional blocks in Recourse Control Functions related to Virtual Network instance 6 Figure 8-1: tight coupled SDN-based network Virtualization solutions 8 Figure 8-2: Loose coupled SDN-based network Virtualization solutions 10 Figure A-1: vCDN nodes deploy dynamically 12 Figure A-2: smart traffic control in vCDN 13

Draft new Recommendation ITU-T H.VCDN.1-NV

Virtual content delivery network – Network virtualization

AAP Summary

[To be provided before Consent]

Summary

[To be provided before Consent]

Keywords

Content delivery network, virtualization, network virtualization, software-defined network.

Scope

This draft Recommendation describes the network virtualization requirement, Functional blocks and potential solutions related to Virtual Content Delivery Network (VCDN) service.

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[ITU-T Y.3011]	ITU-T Recommendation Y.3011 (2012), Framework of network virtualization for Future Networks.

[ITU-T Y.3012]	ITU-T Recommendation Y.3012 (2014), Requirements of network virtualization for future networks.

[ITU-T Y.3300]	ITU-T Recommendation Y.3300 (2014), Framework of software-defined networking.

[ITU-T Y.2019]	ITU-T Recommendation Y.2019 (2010), Content delivery functional architecture in NGN.

[TBD]

Definitions

Terms defined elsewhere

This document uses the following terms defined elsewhere

3.1.1	logically isolated network partition (LINP) [ITU-T Y.3011]: A network that is composed of multiple virtual resources which is isolated from other LINPs.

3.1.2	network virtualization [ITU-T Y.3011]: A technology that enables the creation of logically isolated network partitions over shared physical networks so that heterogeneous collection of multiple virtual networks can simultaneously coexist over the shared networks. This includes the aggregation of multiple resources in a provider and appearing as a single resource.

3.1.3	virtual resource [ITU-T Y.3011]: An abstraction of physical or logical resource, which may have different characteristics from the physical or logical resource and whose capability may be not bound to the capability of the physical or logical resource.

3.1.4	software-defined networking [ITU-T Y.3300]: A set of techniques that enables to directly program, orchestrate, control and manage network resources, which facilitates the design, delivery and operation of network services in a dynamic and scalable manner.

[TBD]

Terms defined within this document

This document defines the following terms

[TBD]

Acronyms and abbreviations

| LINP | Logically Isolated Network Partition |
| --- | --- |
| SDN | Software Defined Network |
| VCDN | Virtual Content Delivery Network |

Convention

[TBD]

Introduction

This section is going to describe overview of functional requirements, components and the possible approach of network virtualization in VCDN.

The target of legacy CDN is to push the content to the server node which is nearest to the end-user. It also provides the capability of locating the content, content backtracking, content delivery route, etc. But in real implementation, the efficiency of CDN will be affected by many factors, such as loading balance, bandwidth, network traffic and the underlay network topology. The current network is fixed, non-service aware and hard to replace. In some CDN solution, CDN node may have the capability to detect the network status periodically. The problem is the network information detected is unreliable, which reduce the CDN service efficiency. In the other hand, it is difficult for Service Provider to create and distribute a new service based on the fixed network hardware and configuration.

Therefore, as the fundamental element of CDN, network innovation will bring a great benefit to Service Provider or CDN operator to optimize their CDN implementation.  A flexible, programmable and service-aware network would give an opportunity to solve the problem just mentioned above and could reduce the cost of CDN equipments investment and maintenance.

Figure 6-1 shows the functional component that related to the network virtualization in VCDN.

Figure 6-1: Functions related to network virtualization in VCDN architecture

The Figure 6-1 provides the basic idea of network virtualization. The physical network resource, such as router and switches, in Network Functions can be abstracted in the Resource Function and be orchestrated and managed in Resource Control Functions (RCF). The orchestrator and management can be implemented in a VN MANO (virtual network management and orchestrator) Functions which could be integrated with RCF. Alternatively, the VN MANO Functions can be separated from RCF as a 3rd party Functions. The RCF can communicate with 3rd party MANO with a standard interface.

The network resource controlling may be based on the request from the Content Delivery Management Functional Block or dynamically modified according to the network information from under layer network.

The Resource Functions (RF) will provide abstract infrastructure resource based on the physical resource from Content Delivery Functions (CDF) and created the required virtual resource by the instruction from RCF.

[TBD]

Functions and Functional blocks of network virtualization under VCDN architecture

This section is going to describe the functional blocks details with is components, roles and possible relationship with other functional blocks.

Functions and Functional blocks in Recourse Control Functions

This clause is going to describe the detailed functional blocks which related to the virtual network creation, configuration and management in the RCF .

The Resource Control Functions handles the orchestration and management of the virtual network resource from the Resource Functions. It also creates the application instance to march the vCDN operator’s requirements. With the combination of those components, the RCF is able to control the virtual network resource creation, allocation and release.

The Figure 7-1 shows the Functions and Functional Blocks that contains in the RCF.

Figure 7- 1: Functional blocks in Recourse Control Functions related to Virtual Network instance

(Contributor note: VN=Virtual Network)

[Notes:] The architecture of Virtual Network Resource Control Functions in this Recommendation covers the instantiation of Virtual Network. But it could also cover the instantiation of Virtual Storage and Virtual Computing as well.

- 7.1.1 Virtual Network Application Functions

The Virtual Network Application (VNA) Functions enables the RCF to implements the CDN service logical operations as instances with virtualized network resource, e.g. content distribution with virtualized CDN nodes.

- 7.1.2 Virtual Network MANO Functions

The Virtual Network MANO (VN-MANO) Functions handles the lifecycle of vCDN service, VNAF and the virtualized network resource control corresponding with the vCDN service request. It is composed of Orchestrator, VN application management and VN resource control functional blocks.

It is noted that the VNMANOF is alternatively existed out of the RCF as a 3rd party Functions.

- 7.1.2.1 Virtual Network Orchestrator Functional Block

The Orchestrator is responsible for the lifecycle management of VN service, the coordination of VN applications management in vCDN and the orchestration of VN resource.

- 7.1.2.2 Virtual Network Application Management Functional Block

The Virtual Network Application Management (VNAM) functional block is responsible for the instantiation and lifecycle management of the VN application related to vCDN service. It is noted that the VNAM functional block is possibly composed of many independent instances.

- 7.1.2.3 Virtual Network Resource Control Functional Block

The Virtual Network Resource Control (VNRC) functional block is responsible for the controlling and managing the Virtual Network Resource from the Resource Functions.

[TBD]

Functional blocks in Recourse Functions

This clause is going to describe the detailed functional blocks which related to the network resource abstraction, modification and implementation in RF.

[TBD]

Network virtualization for VCDN

This section is going to describe the potential solution of network virtualization in VCDN. The convergence of SDN approach and network virtualization in VCDN and other possible approaches are considered. More reference point or functional components may be reference from the current Recommendation or draft in SG13.

SDN-based network virtualization

Tight coupled SDN-based network virtualization

The Tight coupled network virtualization means most of functions in CDN, especially the transport and control related functions, can be integrated into VCDN and the less of service logic are remained in CDN functions. VCDN functions needs to receive the request from CDN functions and to created its own service logic and control the resource usage directly. The benefit of adopting the tight coupled network virtualization is to improve the CDN distribution and delivery efficient. Data transport and process can be directly executed under the control of the virtualized network elements in CDN.

SDN-based solution provides an opportunity to realize the VCDN functions with its own functional architecture. Figure 6-1 shows how the SDN functional architecture can be integrated with CDN functions under the VCDN scope.

Figure 8-1: tight coupled SDN-based network Virtualization solutions

VCDN

Resource Functions

CDN

VCDN Service

Function

SDN control layer

SDN application layer

CDN

CDN application

CDN transport network (Physical Resource)

SDN application

SDN Resource layer

Data transport and processing

VCDN

Resource control

Functions

Function integration

Media stream

Content Distribution, Delivery control

Content Distribution, Delivery

Orchestrator

SDN application

Basically, there are three parts in CDN are recommended to be integrated with SDN architecture: CDN application is integrated with SDN application. CDN control functions, i.e. Content Distribution control function and Content Delivery control function are integrated with SDN controller. Content Distribution, Content Delivery and the physical network resource, such as switcher, router, gateway or any device which supports SDN transport protocols, are integrated with SDN Resource Layer (more precisely, with data transport and processing function).

With the integration, when the SDN application layer receives the service request from CDN application, it creates the resource acquiring information and service orchestrating information based on the request. Then that information will be used to create the resource reservation request. The request is then sent from application layer to the SDN control layer.

SDN control layer allocates the CDN resource required according to the above-mentioned resource reservation requirement. It also generates a content route table for data transporting, according to the network load balance and routing policy. Then SDN control layer generates the L2/L3 (data link layer/network layer) forwarding table based on the content routing table and sends this table to SDN Resource layer.

The data transport and processing function in SDN resource layer is responsible for the data transporting for content distribution and content delivery. When the request of content distribution and content delivery are received, SDN Resource layer invokes the related resource from CDN physical network to execute the functions of distribution and delivery for implementing the service application, according to the L2/L3 forwarding table.

From the above conceptual progress, CDN is able to request, allocation, occupy and released the network resource dynamically by taking advantage of tight coupled SDN-based network virtualization.

[TBD]

Loose coupled SDN-based network virtualization

The loose coupled network virtualization means the major functions of CDN, i.e. the application, distribution and delivery, can be migrated into vCDN and only a few functions are still remained in CDN functions, e.g. user request redirect. When vCDN receives the request redirected from CDN, it implements its own service logic and control the resource usage directly under vCDN software environment. In this case, the vCDN node is almost independent from the real CDN node. The benefit of adopting the loose coupled network virtualization is to improve the CDN functions deploy in a flexible way. CDN nodes can be created as a software module in a general hardware and operation system, where is running many other application program in parallel. It is easily managed by Operators and can reduce the hardware and software investment.

Figure 8-2 shows how CDN functions are deployed in the SDN functional architecture under the vCDN scope.

vCDN

SDN control layer

SDN application layer

CDN functions

CDN application

SDN Resource layer

Data transport and processing

Function migration

Media stream

Content Distribution Functions

Content Delivery Functions

Orchestrator

CDN Function (SDN application)

Content Cache/Storage Functions

Request Function

Operation Function

Control

Orchestrate

Loading balance

User terminal device

Service request

Request message

Resource request

Resource reservation

Orchestrate feedback

Media access

Content Distribution Functions

Figure 8-2: Loose coupled SDN-based network Virtualization solutions

- SDN Application layer –CDN Function (SDN application)

CDN logical functions could be migrated into SDN application layer as a SDN application. To implement CDN service logical operations, CDN Function needs to implement at least 2 functional modules:

- Request Function

This functional block is used to add the integrated CDN service logical functions into SDN application layer. Moreover, it receives the CDN service request and then sends the request of resource requiring and service orchestrating to Orchestrate Function in Orchestrator in SDN control layer.

- Operation Function

The major CDN functions, e.g. CDN application, content distribution and delivery, content storage and load balance, can be migrated into that logical functional module, The Operation Function handles the logical procedure of CDN service. When the feedback information of resource reservation is received, the Operation Function implements the logical operation based on the content distribution or media accessing request.

- SDN control layer – Orchestrator functions

The Orchestrator provides control and management of the network resource, as well as coordination of request from SDN application layer (Referenced from [ITU-T Y.3300]). To process the request from SDN application layer, the Orchestrator is recommended to implement 2 functional modules:

- Orchestrate Function

The Orchestrate Function can be further composed of 2 functional blocks:

- Recourse coordination: It is used to receive the request of resource requiring and service orchestrating and to process the service orchestrating according to the SDN network information. Then it sends the request of required resource reservation to the Control Function.

- Orchestrate feedback: when the resource reservation is completed, it returns the feedback to CDN Function with the resource reservation information.

- Control Function

The Control Function is used to allocate the resource required by CDN service, according to the resource reservation request. After the resource allocation, the Control Function will determine the routing path for content distribution and delivery, combining with the condition of load balance and routing status in the network. It also creates the data link layer/network layer (L2/L3) forwarding tables based on the control policy of SDN controller layer and then sends those tables to Data transport and process Function. Moreover, it also needs to return the information of resource reservation to the Orchestrate Function.

- SDN Resource layer – data transport and processing functions

The data transport and processing function entity involves many network devices which support SDN transport protocols. They are used for storing data based on the allocated resource related to the CDN Function. Furthermore, it executes data transporting correspond with the logical operation in CDN Function, according to the L2/L3 forwarding tables determined by Control Function.

[TBD]

[TBD]

Reference points

This section is going to describe the potential reference point between different functional blocks that mentioned in this Recommendation.

[TBD]

Appendix I

Use cases of network virtualization supporting VCDN services

(This Appendix does not form an integral part of this Recommendation.)

The legacy telecom service, such as IPTV, is usually deployed on the Telecom Operator’s dedicate network. CDN is also a distribute network overlay with the legacy transport network. But in current market, CDN is expected to be designed for the integrated service in an open network environment. The integrated services may be run on the different platforms or networks with their own service logic. According to the features of the integrated service, network virtualization is a prospective approach to improve the CDN capability to use the network resource with higher efficiency and flexibility by ignoring the difference of hardware/software.

So the network virtualization for vCDN is considered to have the following use cases.

I.1 virtualized CDN node deploy dynamically and flexibly

In general, CDN nodes deployment depends on many factors such as geography location, target user density, etc. Usually the deployment purpose and node device should be determined by CDN operators. Once the CDN nodes are deployed, the configuration and maintenance for CDN nodes are usually keep in stable. But in that situation, if one CDN node capability is exceeded its limitation by the rapid increasing of user request in a peak time, it is difficult to add a new CDN node in time. Also a deployed CDN node resource may not be used efficiently in the normal time. vCDN approach is able to create a new logical CDN node is network device supporting network virtualization by Resource control request and to release the resource when the peak time is passed.

Figure A-1: vCDN nodes deploy dynamically

As the Figure A-1shows, a vCDN node can be created and deployed by the service request. The functionality of CDN node can be virtualized and programmed on the virtual machine in a server located in a data centre or any device which supports NFV/SDN functions.

I.2 smart traffic control

In general, the quality of media content distribution and delivery in CDN is usually restricted to bandwidth, traffic congestion and network jitter. The routing path and transport control cannot be easily predicted in the legacy CDN because of the lack of sensing for the low layer network status. In vCDN approach, the traffic control can be centralized in a transport controller, e.g. a SDN controller, and the profile of network can be obtained by vCDN management node. Before the content transmission, the vCDN will plan a shortest (may not the shortest in geography) and efficient distribution/delivery path, according the profile of network status. Moreover, the traffic control may adjust the bandwidth for media transmission in the peak time and release the bandwidth after.

Figure A-2: smart traffic control in vCDN

As the Figure A-2 shows, the media content data can be distributed to the different edge nodes with an optimized path while the traffic congestion is detected. In addition, a virtualized switch/router can be created for adjust the bandwidth temporary.

I.3 quickly service deploying

In the current market, with the development of transport network, service platform and terminal device, more and more services are designed rapidly with multiple service types. But in the traditional platform, the service is usually running in a closed ecosystem. The service designing and deploying may require couple of months.  It may require to be run on the special hardware and software as well. The service configuration and maintenance are difficult because the network resource that service required cannot be easily reserved and allocated while the service needs to be distributed over large scaled network.

In the vCDN, a virtualized CDN node provides a network resource abstraction with the common hardware/software resource. A new service can be programmed and installed into Operator’s service platform quicker than the legacy way. Also, the visualization of network resource, information collection and network policy would help Operator to deploy the new service into each node in a very short period (usually couple of months before, but only a few days now). The configuration will also be automatically completed in a short time by taking advantage of network resource virtualization.

I.4 unified configuration and management

In the traditional CDN ecosystem,  many CDN equipments from different manufactors may be deployed in a Telecom Operator’s premises side-by-side. Those CDN nodes are usually designed for different purposes and rely on the dedicated hardware or software. Any changes such as system/equipment updated, new service creation, may affect the operation for each nodes in the whole CDN topology correspondently. Therefore, the differentiation of dedicated hardware/software increases the complexity of configuration, management and operation investments.

The vCDN approach provides an opportunity to separate the logical control layer from the dedicated physical equipment and be redesigned as a unified control centre. The unified control centre can interact with network elements in L2/L3 layer, such as switch or router, by through a standard interface. Moreover, with the virtualization of CDN functions, the unified control centre is able to configure and manage the logical CDN nodes for different purposes based on the resource allocated in a general infrastructure, even the resource may physically come from different Data Centres. The resource can be added, adjusted, shared and migrated according to the CDN operator’s purpose and time allocation.

Bibliography

[TBD]	Placeholder.

____________________