INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2012/M26638

July 2012, Stockholm, Sweden

| Source | Kwangwoon University (KWU) and Simon Fraser University (SFU) |
| --- | --- |
| Status | Proposal |
| Title | 3D-HEVC – Improvement of the rate control for 3D multi-view video coding |
| Author | Woong Lim (limwoong@kw.ac.kr) Jonghoon Yoo (mmyjh@kw.ac.kr) Hyomin Choi (hyomin06@kw.ac.kr) Junghak Nam (qejixfyza@kw.ac.kr) Hyunho Jo (idjhh@kw.ac.kr) Donggyu Sim (dgsim@kw.ac.kr) Ivan V. Bajić (ibajic@ensc.sfu.ca) |

Abstract

In the last meeting in Stokholm, we proposed rate control algorithm for multi-view video coding [1]. The contribution presents a rate control scheme for multi-view video coding based on the unified rate-quantization (URQ) model for HEVC described in JCTVC-H0213 [2] and JCTVC-I0094 [3]. In the contribution, there were two primary algorithms for extended view. The first, initial frame level QP setting of the first frame of a GOP of extended view. The second, inter-view MAD prediction by using depth map. However, there was a problem in the results for some sequences that occrus sudden high bits genereation. This problem doesn’t affect much the whole performace of rate control to saturate the target bit rate. But, in the scope of frame-wise actual genereated bits, it causes the bit fluctuation. To remove this problem, we propose the initial QP setting for the first frame of GOP of extended views. The proposed rate control scheme is implemeted on 3D-HTM4.0.1. To evaluate the proposed multi-view rate control scheme, 3D-HTM4.0.1, which uses hierarchical QP setting, is used for the constant bit rate (CBR) case. In the last contribution, we found out that the performance of our proposed multi-view rate control was over 99.11% of bitrate accuracy and 2.1dB of PSNR degradation. By using the newly fixed rate control scheme, the average bitrate accuracy is 99.2% with about 1.22dB average PSNR degradation.

- Introduction

A rate control algorithm for HEVC, described in JCTVC-H0213, was adopted in the reference HM software at the 8th San Jose meeting in February 2012. Some improvements were reported in JCTVC-I0094. While the approach from JCTVC-H0213 and JCTVC-I0094 can be applied to HEVC-based multi-view encoding, some modifications are necessary in order to adapt the rate control procedure to the multi-view scenario. And we proposed rate control scheme for multi-view video coding with consideration of the following two main issues.

First, we considered to set the initial QP value of the first frame of the extended views. Because the extended view has at least one reference frame all the time, the QP initialization can be conducted with the QP of the inter-view reference frame at the frame level rate control. Furthermore, for stable generation of bits, we propose inter-view MAD prediction for extended views. In single view encoding, rate control scheme uses the temporal MAD factor to predict the complexity of texture for the current frame. In single-view coding, to predict the complexity of texture, there is no other choice but to use the previously encoded frame along temporal axis to calculate temporal MAD. But in multi-view coding, the previously encoded frame(s) in neighbouring view(s) can also be employed. In addition, depth map can be used to find the corresponding position of the current unit block in the neighbouring view. However, with the proposed multi-view rate control scheme, there was a problem that sudden high generated bits occurs at some frames as shown in fig. 1.

(a)

(b)

Figure 1. Frame-wise (a) accumulated occupancy of virtual buffer and (b) actual generated bits for ‘Newspaper’ sequence (view 2, R2)

To remove this problem, we fixed frame level QP initialization of the first frame of GOP of extended view that reference the QP from the base view and pre-defined view layer QP offset.

- Initial QP decision for the first frame in an extended view

At the beginning of encoding, the first frame of an extended view is an inter-view frame. This means that the first frame of extended view is not coded as an intra-frame but an inter-frame. At frame level, the QPs between base view and extended view are redundant ant to maintain the similar quality between base view and extended view, ViewLayerQPOffset is pre-defined in the configuration. We can simply use this ViewLayerQPOffset for initialization of frame level QP setting of the first frame of GOP of the extended view.

(3)

ViewLayerQPOffset indicates the QP offset between view layers.  and  are QP of the first frame of extended view and base view respectively. By referencing the QP of the first frame of GOP from the base view as above, the bit rate and the quality of the extended view can be stable.

- Experimental results

The proposed rate control for multi-view video coding is estimated under the CBR condition. Table 1 shows the delta bitrate to show the overall performance. The proposed rate control is evaluated with the same condition of the common random access condition of the current 3D multi-view coding. To show the difference of the number of bits, we use % kbps error defined by formula (10).

(10)

Table 1. Performance of view-wise actual generated bits of 3D-HTM rate control and the existing 3D-HTM

| sequence | target total | Target rate V0 | Target rate V1 | Target rate V2 | view0 | view1 | view2 | % kbps error view0 | % kbps error view1 | % kbps error view2 | overall kbps error(%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 1527.00 | 876.00 | 311.00 | 340.00 | 872.84 | 310.65 | 340.75 | 0.36 | 0.11 | 0.22 | 0.18 |
|  | 797.00 | 482.00 | 151.00 | 164.00 | 479.50 | 148.60 | 163.41 | 0.52 | 1.59 | 0.36 | 0.69 |
|  | 448.00 | 278.00 | 81.00 | 89.00 | 276.02 | 79.31 | 87.67 | 0.71 | 2.09 | 1.49 | 1.12 |
|  | 265.00 | 168.00 | 46.00 | 51.00 | 166.92 | 45.75 | 50.13 | 0.64 | 0.55 | 1.71 | 0.83 |
| Kendo | 1495.00 | 844.00 | 314.00 | 337.00 | 832.41 | 307.95 | 331.78 | 1.37 | 1.93 | 1.55 | 1.53 |
|  | 759.00 | 451.00 | 149.00 | 159.00 | 446.39 | 147.66 | 157.78 | 1.02 | 0.90 | 0.77 | 0.94 |
|  | 423.00 | 257.00 | 81.00 | 85.00 | 256.20 | 80.56 | 84.66 | 0.31 | 0.54 | 0.41 | 0.37 |
|  | 250.00 | 155.00 | 46.00 | 49.00 | 154.86 | 45.96 | 48.99 | 0.09 | 0.08 | 0.02 | 0.07 |
| Newspaper | 1685.00 | 934.00 | 383.00 | 368.00 | 924.76 | 379.81 | 360.92 | 0.99 | 0.83 | 1.92 | 1.16 |
|  | 839.00 | 491.00 | 180.00 | 168.00 | 486.02 | 175.68 | 164.10 | 1.02 | 2.40 | 2.32 | 1.57 |
|  | 453.00 | 270.00 | 95.00 | 88.00 | 266.27 | 92.56 | 86.02 | 1.38 | 2.57 | 2.25 | 1.80 |
|  | 260.00 | 157.00 | 54.00 | 49.00 | 152.44 | 52.37 | 47.77 | 2.90 | 3.01 | 2.51 | 2.85 |
| Gtfly | 4949.00 | 3670.00 | 629.00 | 650.00 | 3650.33 | 622.29 | 649.56 | 0.54 | 1.07 | 0.07 | 0.54 |
|  | 2091.00 | 1611.00 | 235.00 | 245.00 | 1602.14 | 234.06 | 242.14 | 0.55 | 0.40 | 1.17 | 0.61 |
|  | 976.00 | 755.00 | 107.00 | 114.00 | 751.92 | 107.22 | 113.44 | 0.41 | 0.20 | 0.49 | 0.35 |
|  | 474.00 | 366.00 | 52.00 | 56.00 | 364.24 | 52.23 | 56.47 | 0.48 | 0.44 | 0.84 | 0.22 |
| PoznanHall2 | 1366.00 | 809.00 | 271.00 | 286.00 | 802.45 | 257.47 | 291.30 | 0.81 | 4.99 | 1.85 | 1.08 |
|  | 570.00 | 344.00 | 111.00 | 115.00 | 341.25 | 111.43 | 115.51 | 0.80 | 0.39 | 0.44 | 0.32 |
|  | 300.00 | 178.00 | 60.00 | 62.00 | 181.27 | 62.00 | 63.34 | 1.83 | 3.33 | 2.16 | 2.20 |
|  | 167.00 | 100.00 | 33.00 | 34.00 | 99.54 | 33.09 | 34.27 | 0.46 | 0.28 | 0.80 | 0.06 |
| PoznanStreet | 3913.00 | 2339.00 | 707.00 | 867.00 | 2332.93 | 695.76 | 854.78 | 0.26 | 1.59 | 1.41 | 0.75 |
|  | 1355.00 | 890.00 | 209.00 | 256.00 | 885.74 | 196.04 | 249.00 | 0.48 | 6.20 | 2.73 | 1.79 |
|  | 605.00 | 415.00 | 86.00 | 104.00 | 413.51 | 86.70 | 104.72 | 0.36 | 0.81 | 0.70 | 0.01 |
|  | 302.00 | 212.00 | 42.00 | 48.00 | 211.53 | 41.53 | 47.74 | 0.22 | 1.12 | 0.54 | 0.40 |
| Dancer | 6539.00 | 4593.00 | 1002.00 | 944.00 | 4578.18 | 988.16 | 925.57 | 0.32 | 1.38 | 1.95 | 0.72 |
|  | 2736.00 | 1996.00 | 381.00 | 359.00 | 1990.06 | 381.46 | 360.73 | 0.30 | 0.12 | 0.48 | 0.14 |
|  | 1256.00 | 917.00 | 174.00 | 165.00 | 914.53 | 175.06 | 165.68 | 0.27 | 0.61 | 0.41 | 0.06 |
|  | 592.00 | 429.00 | 83.00 | 80.00 | 427.83 | 83.49 | 80.64 | 0.27 | 0.59 | 0.80 | 0.01 |
|  |  |  |  |  |  |  | Avg | 0.70 | 1.43 | 1.16 | 0.80 |

Table 1 shows the percentage error of the proposed rate control given the target bitrates produced by the existing 3D-HTM. The average percentage error across all views and sequences is about 0.80%, meaning that the accuracy is better than 99%.

However, rate control comes with some cost in terms of PSNR degradation. To show the PSNR degradation from the proposed rate control, we computed the ΔPSNR as

(11)

Table 2. Performance of view-wise quality of 3D-HTM rate control and the existing 3D-HTM

| sequence | PSNR V0 | PSNR V1 | PSNR V2 | view0 | view1 | view2 | Δ PSNR V0 | Δ PSNR V1 | Δ PSNR V2 | average Δ PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 42.51 | 41.36 | 41.12 | 43.10 | 42.29 | 41.96 | 0.60 | 0.93 | 0.84 | 0.79 |
|  | 39.80 | 38.69 | 38.52 | 41.07 | 39.99 | 39.74 | 1.27 | 1.30 | 1.22 | 1.26 |
|  | 36.37 | 35.39 | 35.30 | 38.51 | 37.40 | 37.19 | 2.13 | 2.01 | 1.89 | 2.01 |
|  | 33.87 | 32.80 | 32.58 | 35.68 | 34.67 | 34.47 | 1.81 | 1.87 | 1.89 | 1.85 |
| Kendo | 41.97 | 40.64 | 40.59 | 43.62 | 42.75 | 40.17 | 1.65 | 2.11 | -0.42 | 1.11 |
|  | 39.52 | 38.38 | 38.33 | 41.46 | 40.43 | 40.17 | 1.95 | 2.05 | 1.84 | 1.94 |
|  | 37.09 | 36.22 | 36.09 | 38.99 | 38.01 | 37.81 | 1.90 | 1.79 | 1.72 | 1.80 |
|  | 34.68 | 33.92 | 33.78 | 36.38 | 35.47 | 35.32 | 1.70 | 1.55 | 1.54 | 1.60 |
| Newspaper | 40.17 | 39.18 | 38.80 | 41.38 | 40.25 | 39.87 | 1.21 | 1.07 | 1.07 | 1.12 |
|  | 37.35 | 36.16 | 35.92 | 38.98 | 37.85 | 37.48 | 1.63 | 1.68 | 1.57 | 1.63 |
|  | 34.87 | 33.79 | 33.58 | 36.46 | 35.34 | 35.04 | 1.59 | 1.55 | 1.46 | 1.54 |
|  | 31.96 | 30.86 | 30.56 | 33.93 | 32.86 | 32.55 | 1.97 | 2.00 | 1.99 | 1.98 |
| Gtfly | 39.44 | 38.74 | 38.76 | 40.19 | 39.68 | 39.64 | 0.75 | 0.94 | 0.88 | 0.86 |
|  | 36.61 | 36.26 | 36.15 | 37.78 | 37.43 | 37.38 | 1.17 | 1.16 | 1.23 | 1.19 |
|  | 34.60 | 34.46 | 34.42 | 35.60 | 35.36 | 35.31 | 1.00 | 0.90 | 0.89 | 0.93 |
|  | 33.03 | 32.89 | 32.78 | 33.46 | 33.29 | 33.23 | 0.44 | 0.40 | 0.45 | 0.43 |
| PoznanHall2 | 41.54 | 41.43 | 41.31 | 42.13 | 41.86 | 41.70 | 0.59 | 0.43 | 0.39 | 0.47 |
|  | 40.65 | 40.41 | 40.33 | 41.15 | 40.86 | 40.77 | 0.50 | 0.45 | 0.44 | 0.46 |
|  | 39.54 | 39.04 | 38.97 | 39.76 | 39.44 | 39.40 | 0.21 | 0.39 | 0.44 | 0.35 |
|  | 37.49 | 36.87 | 37.01 | 38.05 | 37.71 | 37.70 | 0.56 | 0.83 | 0.68 | 0.69 |
| PoznanStreet | 39.19 | 38.33 | 37.73 | 39.87 | 39.09 | 38.57 | 0.67 | 0.76 | 0.84 | 0.76 |
|  | 36.36 | 35.90 | 35.06 | 37.94 | 37.34 | 36.61 | 1.58 | 1.43 | 1.55 | 1.52 |
|  | 34.26 | 33.82 | 32.90 | 35.95 | 35.53 | 34.63 | 1.70 | 1.72 | 1.73 | 1.72 |
|  | 31.82 | 31.57 | 30.62 | 33.89 | 33.66 | 32.68 | 2.07 | 2.09 | 2.06 | 2.07 |
| Dancer | 37.86 | 36.83 | 37.03 | 38.76 | 37.95 | 38.13 | 0.89 | 1.12 | 1.10 | 1.04 |
|  | 34.78 | 34.12 | 34.27 | 35.89 | 35.34 | 35.54 | 1.11 | 1.21 | 1.27 | 1.20 |
|  | 32.30 | 31.78 | 32.00 | 33.38 | 32.95 | 33.15 | 1.08 | 1.17 | 1.16 | 1.13 |
|  | 30.46 | 30.04 | 30.23 | 31.10 | 30.75 | 30.93 | 0.65 | 0.71 | 0.70 | 0.69 |
|  |  |  |  |  |  | Avg | 1.23 | 1.27 | 1.16 | 1.22 |

Table 2 shows the average PSNR degradation is about 1.22dB.

Figures 2 to 8 show the accumulated occupancy of virtual buffer for base view of each sequence.

(a)

(b)

(c)

(d)

Figure 2. The accumulated occupancy of virtual buffer for ‘Balloons’ sequence. (a)~(d) : rate1 ~ rate4

(a)

(b)

(c)

(d)

Figure 3. The accumulated occupancy of virtual buffer for ‘Newspaper’ sequence. (a)~(d) : rate1 ~ rate4

(a)

(b)

(c)

(d)

Figure 4. The accumulated occupancy of virtual buffer for ‘Kendo’ sequence. (a)~(d) : rate1 ~ rate4

(a)

(b)

(c)

(d)

Figure 5. The accumulated occupancy of virtual buffer for ‘GTFly’ sequence. (a)~(d) : rate1 ~ rate4

(a)

(b)

(c)

(d)

Figure 6. The accumulated occupancy of virtual buffer for ‘PoznanHall2’ sequence. (a)~(d) : rate1 ~ rate4

(a)

(b)

(c)

(d)

Figure 7. The accumulated occupancy of virtual buffer for ‘PoznanStreet’ sequence. (a)~(d) : rate1 ~ rate4

(a)

(b)

(c)

(d)

Figure 8. The accumulated occupancy of virtual buffer for ‘UndoDancer’ sequence. (a)~(d) : rate1 ~ rate4

According to the above figures, general fluctuation of the accumulated occupancy of the virtual buffer for the base view is reasonable, and the deviation from the target occupancy is much smaller than without rate control. There is some more fluctuation in some sequences with rate control than without rate control scheme, however, most of all the results seems less fluctuation than the results without rate control. The results for the extended views are presented in the additional Excel file included with this document.

- Conclusion

We proposed a rate control scheme for 3D multi-view video coding. The proposed rate control is based on the existing rate control in HEVC. Specifically, the HEVC scheme is applied to the base view directly. In addition, we fixed initial setting of QP for the first frame of GOP of extended view. By using this rate control for 3D multi-view video coding, the average bitrate accuracy was over 99%, with about 1.22dB average PSNR degradation. It is almost the same accuracy for target bitrate but highly improved the quality of reconstructed video from the results of the last meeting which is with about 2.1dB degradation. And with our proposed rate control scheme, most sudden spikes were removed and it caused less fluctuation of the generated bits per frame.

- Reference

- W. Lim, H. Choi, J. Nam, H. Jo, D. Sim and I. V. Bajić, “3D-HEVC – Rate control for 3D multi-view video coding,” ISO/IEC JTC1/SC29/WG11 document M25966, Stockholm, Sweden, July, 2012

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Rate control based on unified RQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT-VC H0213 (m23088), San José, CA, USA, Feb. 2012

- H. Choi, J. Nam, J. Yoo, D. Sim, and I. V. Bajić, “Improvement of the rate control based on pixel-based URQ model for HEVC,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0094 (m24333), Geneva, Switzerland, Apr.-May. 2012.

- Patent rights declaration(s)

KWU and SFU do not have any current or pending patent rights relating to the technology described in this contribution.