| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0046_proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-B0046 based on MV-HEVC Working Draft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Jewon Kang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8457 jewonk@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides text for the reference picture list construction aspect of JCT3V-B0046 based on MV-HEVC working draft. Changes are highlighted.

## General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if (layer_id) |  |
| inter_view_ref_start_position | u(v) |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

## General slice header semantics

inter_view_ref_start_position specifies the starting position of the inter-view reference pictures in reference picture list 0 after reference picture list initialization. inter_view_ref_start_position is in the range of 0 to min (num_ref_idx_l0_active_minus1, NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr), inclusive. When not present, inter_view_ref_start_position is inferred to be equal to -1.

## Decoding process

F.8.3.4.2	Initialization process for reference picture lists

This process is invoked when decoding a P or B slice header.

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && cIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(8-8)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumIvCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetIvCurr[ i ]
}

a list of positions of the changed index is defined as idxPosList and derived as follows:

iNumTemp = NumPocStCurrBefore + NumPocStCurrAfter +NumPocLtCurr;
if (inter_view_ref_start_position>=0) {

for ( i = 0; i < inter_view_ref_start_position; i ++)	
		idxPosList [ i ] = i;

for ( ; i < (inter_view_ref_start_position + NumPocIvCurr); i ++)	
		idxPosList[ i ] = iNumTemp + (i - inter_view_ref_start_position);
	for ( ; i <= num_ref_idx_l0_active_minus1; i ++)	
		idxPosList[ i ] = i - NumPocIvCurr;
}
else 
	for (i=0; i< iNumTemp+ NumIvCurr ; i++)
		idxPosList [ i ] = i;

The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(8-9)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[idxPosList[ rIdx ] ]