- I.7.3.8.5.1	Intra mode extension syntax

| intra_mode_ext( x0 , y0 , log2PbSize ) { | Descriptor |
| --- | --- |
| if( log2PbSize < 6 ) |  |
| dim_not_present_flag[ x0 ][ y0 ] | ae(v) |
| if ( !dim_not_present_flag[ x0 ][ y0 ] ) |  |
| depth_intra_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ) |  |
| { |  |
| wedge_diff_tab_idx[ x0 ][ y0 ] | ae(v) |
| if (IsDMM1SignBitExist[ x0 ][ y0 ]) |  |
| wedge_diff_tab_idx_sign[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

- I.7.4.9.5.1 Intra mode extension semantics

wedge_diff_tab_idx[ x0 ][ y0 ] specifies the index of the differential wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL. The variables DMM1InitValue is derived from the value of log2PbSize as defined in Table I-x..

Table I-x DMM1InitValue

| log2PbSize | value |
| --- | --- |
| 2 | 23 |
| 3 | 255 |
| 4 | 327 |
| 5 | 480 |

wedge_diff_tab_idx_sign[ x0 ][ y0 ] specifies the sign of the differential wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

The variable IsDMM1SignBitExist[ x0 ][ y0 ] is derived as specified in the following:

IsDMM1SignBitExist[ x0 ][ y0 ] = (wedge_diff_tab_idx[ x0 ][ y0 ] > DMM1InitValue) ? 0 : 1  (I-xx1)

The variable WedegeFullTabIdx[ x0 ][ y0 ] is derived as specified in the following:

WedegeFullTabIdx[ x0 ][ y0 ] = DMM1InitValue + (1 - 2*wedge_diff_tab_idx_sign[ x0 ][ y0 ])* wedge_diff_tab_idx[ x0 ][ y0 ]    (I-xx2)

- I.8.4.4.2.7 Specification of intra prediction mode INTRA_DMM_WFULL

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nTbS − 1, specifying a binary partition pattern is derived as.

wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ WedegeFullTabIdx [ xTb ][ yTb ] ]	(I-63)

- I.8.4.4.3 Segmental depth intra coding process

Depending on predModeIntra the array wedgePattern[ x ][ y ] with x, y =0..nTbS − 1 specifying the binary segmentation pattern is derived as follows:

If predModeIntra is equal to INTRA_DMM_WFULL, the following applies:

wedgePattern = WedgePatternTable[ Log2( nTbS ) ][ WedegeFullTabIdx[ xTb ][ yTb ] ]

Table I-12 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| cu_extension( ) intra_mode_ext( ) | depth_intra_mode_flag | Table I-17 | 0 | 1 | 2 |
|  | wedge_diff_tab_idx | Table I-13 | 0 | 1 | 2 |
|  | depth_dc_flag | Table I-18 | 0 | 1 | 2 |
|  | depth_dc_abs | Table I-14 | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Table I-15 |  | 0..2 | 3..5 |
|  | ic_flag | Table I-16 |  | 0 | 1 |
|  | dbbp_flag | Table I-19 | 0 | 1 | 2 |
|  | sdc_flag | Table I-20 | 0 | 1 | 2 |
|  | dim_not_present_flag | Table I-21 | 0..2 | 3..5 | 6..8 |

Table I-13 – Values of initValue for wedge_diff_tab_idx ctxIdx

| Initialization variable | ctxIdx of wedge_diff_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table I-22 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| cu_extension( ) | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | ic_flag | FL | cMax = 1 |
|  | dbbp_flag | FL | cMax = 1 |
|  | sdc_flag | FL | cMax = 1 |
| intra_mode_ext( ) | dim_not_present_flag | FL | cMax = 1 |
|  | depth_intra_mode_flag | FL | cMax = 1 |
|  | wedge_diff_tab_idx | FL | cMax = wedgeDiffTabIdxBits[ log2PbSize ] (defined in Table I-23) |
|  | depth_dc_flag | FL | cMax = 1 |
|  | depth_dc_abs | UEG0 | [Ed. (GT) To be specified] |
|  | depth_dc_sign_flag | FL | cMax = 1 |

Table I-23 –Values of wedgeDiffTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeDiffTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 |  |
| Value | 6 | 9 | 10 | 10 |  |

Table I-24 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_diff_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| iv_res_pred_weight_idx | 0, 1 Table I-25 | 2 | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| dbbp_flag | 0 | na | na | na | na | na |
| depth_intra_mode_flag | 0 | na | na | na | na | na |
| sdc_flag | 0 | na | na | na | na | na |
| dim_not_present_flag | 0, 1,2 Table I-25 | na | na | na | na | na |