| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-J0050 |
| --- | --- |

| Title: | IC improvement for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil, Seocho-gu Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 junghak.nam@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

The newly added parts compared to 3D-HEVC working draft 3 are highlighted in green and the removed parts are marked with strikethrough.

Method 1: removal of chroma IC

- I.8.5.3.3    Decoding process for inter prediction samples

- I.8.5.3.3.1  General

Inputs to this process are:

a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

a variable nCbS specifying the size of the current luma coding block,

two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

the luma motion vectors mvL0 and mvL1,

when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

the reference indices refIdxL0 and refIdxL1,

the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

- The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC.

If DispDerivedDepthFlag[ xC + xB ][ yC + yB ] is equal to 1, the following applies:

For each luma sample location ( xL = 0..nPbW −1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSamplesL, the corresponding prediction luma sample value predSamplesL[ xL ][ yL ] is set equal to DispDerivedDepthVal[ xC + xB ][ yC + yB ].

Otherwise, the following ordered steps apply:

  - Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and when ChromaArrayType is not equal to 0, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr, and predSamplesL1Cr be (nPbW / SubWidthC)x(nPbH / SubHeightC) arrays of predicted chroma sample values.

For X being each of 0 and 1, when predFlagLX is equal to 1, the following applies:

When predFlagLX is equal to 1, the following applies:

The variable resPredFlag is derived as specified in the following:

        - resPredFlag = 	( iv_res_pred_weight_idx  !=  0 )  &&  RpRefPicAvailFlagLX  &&
	RefRpRefAvailFlagLX[ RefViewIdx[ xP ][ yP ] ]	(I-185)

If resPredFlag is equal to 1, the bilinear sample interpolation and residual prediction process as specified in subclause I.8.5.3.3.7 is invoked with the luma locations ( xCb, yCb ), ( xBl, yBl ), the size of the current luma coding block nCbS, the width and the height of the current luma prediction block nPbW, nPbH, the prediction list utilization flags predFlagL0 and predFlagL1, the reference indices refIdxL0 and refIdxL1, the motion vectors mvL0, mvL1, when ChromaArrayType is not equal to 0, the motion vectors mvCL0, and mvCL1, and the prediction list indication X, and the outputs are the array predSamplesLXL, and when ChromaArrayType is not equal to 0, the arrays predSamplesLXCb, and predSamplesLXCr.

Otherwise (resPredFlag is equal to 0 ), the following applies:

The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.3.3.2 with refIdxLX as input.

If DepthFlag is equal to 0, the array predSamplesLXL and when ChromaArrayType is not equal to 0, the arrays, predSamplesLXCb, and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in subclause 8.5.3.3.3 with the luma locations ( xCb, yCb ) and ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH, the motion vectors mvLX and, when ChromaArrayType is not equal to 0, mvCLX, and the reference arrays refPicLXL, refPicLXCb, and refPicLXCr as inputs.

Otherwise (DepthFlag is equal to 1), the array predSamplesLXL, and when ChromaArrayType is not equal to 0, the arrays predSamplesLXCb, and predSamplesLXCr are derived by invoking the full sample interpolation process specified in subclause I.8.5.3.3.5 with the luma locations ( xCb, yCb ), ( xBl, yBl ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vector mvLX and the reference arrays with refPicLXL given as input.

Depending on ic_flag, the array predSamplesL is derived as specified in the following:

If ic_flag is equal to 0, the following applies:

The inside the current luma prediction block, predSamplesL[ xL + xBl ][ yL + yBl ] with xL = 0..nPbW − 1 and yL = 0..nPbH − 1, are derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the luma prediction block width nPbW, the luma prediction block height nPbH, and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 0 as inputs.

Otherwise (ic_flag is equal to 1), the following applies:

The inside the current luma prediction block, predSamplesL[ xL + xBl ][ yL + yBl ] with xL = 0..nPbW − 1 and yL = 0..nPbH − 1, are derived by invoking the illumination compensated sample prediction process specified in subclause I.8.5.3.3.6, with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, the luma location ( xBl, yBl ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvL0, mvL1 and cIdx equal to 0 given as input.

Depending on ic_flag and nPbW, tThe arrays predSampleCb, and predSampleCr are derived as specified in the following:

If ic_flag is equal to 0 or nPbW is not greater than 8, the following applies:

When ChromaArrayType is not equal to 0, the prediction samples inside the current chroma component Cb prediction block, predSamplesCb[ xC + xBl / SubWidthC ][ yC + yBl / SubHeightC ] with xC = 0..nPbW / SubWidthC − 1 and yC = 0..nPbH / SubHeightC − 1 , are derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the prediction block width nPbW set equal to nPbW / SubWidthC, the prediction block height nPbH set equal to nPbH / SubHeightC, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 as inputs.

When ChromaArrayType is not equal to 0, the prediction samples inside the current chroma component Cr prediction block, predSamplesCr[ xC + xBl / SubWidthC ][ yC + yBl / SubHeightC ] with xC = 0..nPbW / SubWidthC − 1 and yC = 0..nPbH / SubHeightC − 1 , are derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the prediction block width nPbW set equal to nPbW / SubWidthC, the prediction block height nPbH set equal to nPbH / SubHeightC, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 as inputs.

Otherwise (ic_flag is equal to 1 and nPbW is greater than 8), the following applies:

When ChromaArrayType is not equal to 0, the prediction samples inside the current chroma component Cb prediction block, predSamplesCb[ xC + xBl / SubWidthC ][ yC + yBl / SubHeightC ] with xC = 0..nPbW / SubWidthC − 1 and yC = 0..nPbH / SubHeightC − 1 , are derived by invoking the illumination compensated sample prediction process specified in subclause I.8.5.3.3.6, with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, with the chroma location ( xBl / SubWidthC, yBl / SubHeightC ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW / SubWidthC, nPbHCb set equal to nPbH / SubHeightC, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 1 given as input.

When ChromaArrayType is not equal to 0, the prediction samples inside the current chroma component Cr prediction block,predSamplesCr[xC + xBl / SubWidthC ][ yC + yBl / SubHeightC ] with xC = 0..nPbW / SubWidthC − 1 and yC = 0..nPbH / SubHeightC − 1 , are derived by invoking the illumination compensated sample prediction process specified in subclause I.8.5.3.3.6, with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, with the chroma location ( xBl / SubWidthC, yBl / SubHeightC), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW / SubWidthC, nPbHCr set equal to nPbH / SubHeightC, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 2 given as input.

Method 2: Offset model of chroma IC (version 1)

- I.8.5.3.3.6  Illumination compensated sample prediction process

Inputs to this process are:

a location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

the size of current luma coding block nCbS,

a location ( xBl, yBl ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

the width and height of this prediction block, nPbW and nPbH,

two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

two prediction list utilization flags, predFlagL0 and predFlagL1,

two reference indices, refIdxL0 and refIdxL1,

two motion vector mvL0 and mvL1,

a colour component index cIdx.

Outputs of this process are:

the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows:

The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

The variable offset1 is derived as follows:

If shift1 is greater than 0, offset1 set equal to 1  <<  ( shift1 − 1 ).

Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

The variable offset2 is set equal to 1  <<  ( shift2 − 1 ).

The variable bitDepth is derived as follows:

If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause I.8.5.3.3.6.1 is invoked with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as inputs, and the outputs are the flags puIcFlagL0 and puIcFlagL1 and the variables icWeightL0 and icWeightL1 specifying weights for illumination compensation, the variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as follows:

For X in the range of 0 to 1, inclusive, the following applies:

When predFlagLX is equal to 1 the following applies:

      - clipPredVal = 
Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLX[ x ][ y ] + offset1 )  >>  shift1 )	(I-191)

      - If cIdx is equal to 0, the following applies:

      - predValX = !puIcFlagLX ? clipPredVal : 
( Clip3( 0, ( 1  <<  bitDepth ) − 1, ( clipPredVal * icWeightLX )  >>  5 ) + icOffsetLX )	(I-192)

Otherwise (cIdx is equal to 1 or 2), the following applies:

      - predValX = !puIcFlagLX ? clipPredVal : 
( Clip3( 0, ( 1  <<  bitDepth ) − 1, clipPredVal + icOffsetLX ) )	(I-192)

If predFlagL0 is equal to 1 and predFlagL1 is equal to 1, the following applies:

- predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predVal0 + predVal1 + offset2 )  >>  shift2 )	(I-193)

Otherwise (predFlagL0 is equal to 0 or predFlagL1 is equal to 0), the following applies:

- predSamples[ x ][ y ] = predFlagL0 ? predVal0 : predVal1	(I-194)

- I.8.5.3.3.6.1  Derivation process for illumination compensation mode availability and parameters

Inputs to this process are:

a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top left sample of the current picture,

the size of the current luma coding block nCbS,

prediction list utilization flags, predFlagL0 and predFlagL1,

reference indices refIdxL0 and refIdxL1,

motion vectors mvL0 and mvL1,

a bit depth of samples bitDepth,

a variable cIdx specifying colour component index.

Outputs of this process are:

flags puIcFlagL0 and puIcFlagL1 specifying whether illumination compensation is enabled.

variables icWeightL0 and icWeightL1 specifying weights for illumination compensation

variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation

The variables puIcFlagL0 and PuIcFlagL1 are set equal to 0, the variables icWeightL0 and icWeightL1 are set equal to 1, and the variables icOffsetL0 and icOffsetL1 are set equal to 0.

The variables subWidth and subHeight, and the location ( xC, yC ) specifying the top left sample of the current luma or chroma coding block is derived as follows:

- subWidth = ( cIdx = = 0 ) ? 1 : SubWidthC 		(I-195)

- subHeight = ( cIdx = = 0 ) ? 1 : SubHeightC 		(I-196)

- ( xC, yC ) =  ( xCb / subWidth , yCb / subHeight ) 		(I-197)

The variable availFlagCurAboveRow specifying the availability of above neighbouring row samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xN, yN ) set equal to ( xCb, yCb − 1 ) as the input and the output is assigned to availFlagCurAboveRow.

The variable availFlagCurLeftCol specifying the availability of left neighbouring column samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xN, yN ) set equal to ( xCb − 1, yCb ) as the input and the output is assigned to availFlagCurLeftCol.

When availFlagCurAboveRow is equal to 0 and availFlagCurLeftCol is equal to 0 the whole derivation process of this subclause terminates.

For X being replaced by 0 and 1, when predFlagLX is equal to 1, the variable puIcFlagLX is derived by the following ordered steps.

  - The variable refPicLX specifying the reference picture from reference picture list X is set equal to RefPicListX[ refIdxLX ] .

If ViewIdx( refPicLX ) is not equal to ViewIdx, the variable puIvPredFlagLX specifying whether inter-view prediction from list X is utilized is set equal to 1, otherwise (predFlagLX is equal to 0 or ViewIdx( RefPicListX[ refIdxLX ] )  is equal to ViewIdx ), puIvPredFlagLX is set equal to 0.

If puIvPredFlagLX is equal to 0, the variable puIcFlagLX is set equal to 0, otherwise (puIvPredFlagLX is equal to 1) the following applies:

The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as specified in the following:

        - xRLX = xC + ( mvLX[ 0 ]  >>  ( 2 + ( cIdx ? 1 : 0 ) ) )	(I-198)

        - yRLX = yC + ( mvLX[ 1 ]  >>  ( 2 + ( cIdx ? 1 : 0 ) ) )	(I-199)

The variable puIcFlagLX is derived as follows:

        - puIcFlagLX = availFlagCurAboveRow  | |  availFlagCurLeftCol	(I-200)

Depending on the colour component cIdx, the variable curRecSamples specifying the reconstructed picture samples before deblocking filter of the current picture is derived as specified in the following:

    - curRecSamples = ( !cidx ) ? SL : ( ( icdx == 1 ) ? SCb : SCr )	(I-201)

- For X being replaced by 0 and 1, when puIcFlagLX is equal to 1, the variables icWeightLX, and icOffsetLX are derived by the following ordered steps:

  - Depending on the colour component cIdx, the variable refRecSamples specifying the reconstructed picture samples of the reference picture is derived as specified in the following:

If cIdx is equal to 0, refRecSamples is set equal to reconstructed picture sample array SL of picture refPicLX.

Otherwise, if cIdx is equal to 1, refRecSamples is set equal to the reconstructed chroma sample array SCb of picture refPicLX.

Otherwise (cIdx is equal to 2), refRecSamples is set equal to the reconstructed chroma sample array SCr of picture refPicLX.

The lists curNeighSampleListLX and refNeighSampleListLX specifying the neighbouring samples in the current picture and the reference picture are derived as specified in the following:

The variable numNeighSamplesLX specifying the number of elements in curNeighSampleListLX and in refNeighSampleLX is set equal to 0.

For curNbColFlag in the range of !availFlagCurAboveRow to availFlagCurLeftCol, inclusive, the following applies:

For i in the range of 0 to ( nCbS / ( curNbColFlag ? subHeight : subWidth ) )− 1, inclusive the following applies:

          - xOff = curNbColFlag ? −1: i		(I-202)

          - yOff = curNbColFlag ? i : −1		(I-203)

          - xP = Clip3( 0, ( pic_width_in_luma_samples / subWidth ) – 1, xRLX + xOff )	(I-204)

          - yP = Clip3( 0, ( pic_height_in_luma_samples / subHeight ) – 1, yRLX + yOff )	(I-205)

          - refNeighSampleListLX[ numNeighSamplesLX ] = refRecSamples[ xP ][ yP ]	(I-206)

          - curNeighSampleListLX[ numNeighSamplesLX++ ] = 
	curRecSamples[ xC + xOff][ yC + yOff ]	(I-207)

Depending on the colour component cIdx, the derivation process for illumination compensation parameters is derived as follows:

If cIdx is equal to 0, the following applies:

- The derivation process for illumination compensation parameters as specified in subclause I.8.5.3.3.6.2 is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample, and the number of neighbouring samples numNeighSamlesLX as inputs and the illumination parameters icWeightLX, and icOffsetLX as outputs.

Otherwise (cIdx is equal to 1 or 2), the following applies:

- The derivation process for illumination compensation parameters as specified in subclause I.8.5.3.3.6.3 is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample, and the number of neighbouring samples numNeighSamlesLX as inputs and the illumination parameter icOffsetLX as outputs.

- I.8.5.3.3.6.2  Derivation process for luma illumination compensation parameters

Inputs to this process are:

a list curSampleList specifying the current samples,

a list refSampleList specifying the reference samples,

a variable numSamples specifying the number of elements in curSampleList and refSampleList,

a bit depth of samples, bitDepth,

Outputs of this process are:

a variable icWeight specifying a weight for illumination compensation,

a variable icOffset specifying a offset for illumination compensation.

The variable precShift is set equal to Max( 0, bitDepth − 12 ).

The variables sumRef, sumCur, sumRefSquare and sumProdRefCur are set equal to 0 and the following applies for i ranging from 0 to numSamples / 2 − 1, inclusive:

- sumRef  +=  refSampleList[ 2 * i ]		(I-208)

- sumCur  +=  curSampleList[ 2 * i ]		(I-209)

- sumRefSquare  +=  ( refSampleList[ 2 * i ] * refSampleList[ 2 * i ] )  >>  precShift	(I-210)

- sumProdRefCur  +=  ( refSampleList[ 2 * i ] * curSampleList[ 2 * i ] )  >>  precShift	(I-211)

The variable avgShift and avgOffset are derived as follows:

- avgShift = Log2( numSamples / 2)		(I-212)

- avgOffset = 1  <<  ( avgShift − 1 )		(I-213)

The variables numerDiv and denomDiv are derived as follows:

- denomDiv= ( ( sumRefSquare + ( sumRefSquare  >>  7 ) )  <<  avgShift )
		 − ( sumRef * sumRef )  >>  precShift	(I-214)

- numerDiv= Clip3( 0, 2 * denomDiv, ( ( sumProdRefCur + ( sumRefSquare  >>  7 ) )  <<  avgShift )
		 − ( sumRef * sumCur )  >>  precShift )	(I-215)

The variables shiftNumer and shiftDenom are derived as follows:

- shiftDenom = Max( 0, Floor( Log2( Abs( denomDiv ) ) ) − 5 )	(I-216)

- shiftNumer = Max( 0, shiftDenom − 12 )		(I-217)

The variables sNumerDiv and sDenomDiv are derived as follows:

- sDenomDiv = denomDiv  >>  shiftDenom		(I-218)

- sNumerDiv = numerDiv  >>  shiftNumer		(I-219)

The value of variable divCoeff is derived from Table I-10 depending on sDenomDiv and the variables icWeight, and icOffset are derived as follows:

  - icWeight = ( sNumerDiv * divCoeff )  >>  ( shiftDenom − shiftNumer + 10 )	(I-220)

- icOffset = ( sumCur − ( ( icWeight * sumRef )  >>  5 ) + avgOffset )  >>  avgShift	(I-221)

Table I-10 – Specification of divCoeff depending on sDenomDiv

| sDenomDiv | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| divCoeff | 0 | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 |
| sDenomDiv | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| divCoeff | 2521 | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 |
| sDenomDiv | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| divCoeff | 1260 | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 |
| sDenomDiv | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| divCoeff | 840 | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 |
| sDenomDiv | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |  |
| divCoeff | 630 | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 |  |

- I.8.5.3.3.6.3  Derivation process for chroma illumination compensation parameters

Inputs to this process are:

a list curSampleList specifying the current samples,

a list refSampleList specifying the reference samples,

a variable numSamples specifying the number of elements in curSampleList and refSampleList,

a bit depth of samples, bitDepth,

Outputs of this process are:

a variable icOffset specifying a offset for illumination compensation.

The variables sumRef and sumCur are set equal to 0 and the following applies for i ranging from 0 to numSamples / 2 − 1, inclusive:

- sumRef  +=  refSampleList[ 2 * i ]		(I-208)

- sumCur  +=  curSampleList[ 2 * i ]		(I-209)

The variable avgShift and avgOffset are derived as follows:

- avgShift = Log2( numSamples / 2)		(I-2xx)

- avgOffset = 1  <<  ( avgShift − 1 )		(I-2xx)

- icOffset = ( sumCur − sumRef  + avgOffset )  >>  avgShift	(I-2xx)

Method 2: Offset model of chroma IC (version 2)

- I.8.5.3.3.6  Illumination compensated sample prediction process

Inputs to this process are:

a location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

the size of current luma coding block nCbS,

a location ( xBl, yBl ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

the width and height of this prediction block, nPbW and nPbH,

two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

two prediction list utilization flags, predFlagL0 and predFlagL1,

two reference indices, refIdxL0 and refIdxL1,

two motion vector mvL0 and mvL1,

a colour component index cIdx.

Outputs of this process are:

the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows:

The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

The variable offset1 is derived as follows:

If shift1 is greater than 0, offset1 set equal to 1  <<  ( shift1 − 1 ).

Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

The variable offset2 is set equal to 1  <<  ( shift2 − 1 ).

The variable bitDepth is derived as follows:

If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause I.8.5.3.3.6.1 is invoked with the luma location ( xCb, yCb ), the size of the current luma coding block nCbS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as inputs, and the outputs are the flags puIcFlagL0 and puIcFlagL1 and the variables icWeightL0 and icWeightL1 specifying weights for illumination compensation, the variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..( nPbW ) − 1 and y = 0..( nPbH ) − 1 are derived as follows:

For X in the range of 0 to 1, inclusive, the following applies:

When predFlagLX is equal to 1 the following applies:

      - clipPredVal = 
Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLX[ x ][ y ] + offset1 )  >>  shift1 )	(I-191)

      - If cIdx is equal to 0, the following applies:

      - predValX = !puIcFlagLX ? clipPredVal : 
( Clip3( 0, ( 1  <<  bitDepth ) − 1, ( clipPredVal * icWeightLX )  >>  5 ) + icOffsetLX )	(I-192)

Otherwise (cIdx is equal to 1 or 2), the following applies:

      - predValX = !puIcFlagLX ? clipPredVal : 
( Clip3( 0, ( 1  <<  bitDepth ) − 1, clipPredVal + icOffsetLX ) )	(I-192)

If predFlagL0 is equal to 1 and predFlagL1 is equal to 1, the following applies:

    - predSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predVal0 + predVal1 + offset2 )  >>  shift2 )	(I-193)

Otherwise (predFlagL0 is equal to 0 or predFlagL1 is equal to 0), the following applies:

- predSamples[ x ][ y ] = predFlagL0 ? predVal0 : predVal1	                                         (I-194)

- I.8.5.3.3.6.2  Derivation process for illumination compensation parameters

Inputs to this process are:

a list curSampleList specifying the current samples,

a list refSampleList specifying the reference samples,

a variable numSamples specifying the number of elements in curSampleList and refSampleList,

a bit depth of samples, bitDepth,

Outputs of this process are:

a variable icWeight specifying a weight for illumination compensation,

a variable icOffset specifying a offset for illumination compensation.

The variable precShift is set equal to Max( 0, bitDepth − 12 ).

The variables sumRef, sumCur, sumRefSquare and sumProdRefCur are set equal to 0 and the following applies for i ranging from 0 to numSamples / 2 − 1, inclusive:

- sumRef  +=  refSampleList[ 2 * i ]		(I-208)

- sumCur  +=  curSampleList[ 2 * i ]		(I-209)

The variable avgShift and avgOffset are derived as follows:

- avgShift = Log2( numSamples / 2)		(I-212)

- avgOffset = 1  <<  ( avgShift − 1 )		(I-213)

If cIdx is equal to 0, the following applies:

- sumRefSquare  +=  ( refSampleList[ 2 * i ] * refSampleList[ 2 * i ] )  >>  precShift	(I-210)

- sumProdRefCur  +=  ( refSampleList[ 2 * i ] * curSampleList[ 2 * i ] )  >>  precShift	(I-211)

The variable avgShift and avgOffset are derived as follows:

- avgShift = Log2( numSamples / 2)		(I-212)

- avgOffset = 1  <<  ( avgShift − 1 )		(I-213)

The variables numerDiv and denomDiv are derived as follows:

- denomDiv= ( ( sumRefSquare + ( sumRefSquare  >>  7 ) )  <<  avgShift )
		 − ( sumRef * sumRef )  >>  precShift	(I-214)

- numerDiv= Clip3( 0, 2 * denomDiv, ( ( sumProdRefCur + ( sumRefSquare  >>  7 ) )  <<  avgShift )
		 − ( sumRef * sumCur )  >>  precShift )	(I-215)

The variables shiftNumer and shiftDenom are derived as follows:

- shiftDenom = Max( 0, Floor( Log2( Abs( denomDiv ) ) ) − 5 )	(I-216)

- shiftNumer = Max( 0, shiftDenom − 12 )		(I-217)

The variables sNumerDiv and sDenomDiv are derived as follows:

- sDenomDiv = denomDiv  >>  shiftDenom		(I-218)

- sNumerDiv = numerDiv  >>  shiftNumer		(I-219)

The value of variable divCoeff is derived from Table I-10 depending on sDenomDiv and the variables icWeight, and icOffset are derived as follows:

- icWeight = ( sNumerDiv * divCoeff )  >>  ( shiftDenom − shiftNumer + 10 )	(I-220)

- icOffset = ( sumCur − ( ( icWeight * sumRef )  >>  5 ) + avgOffset )  >>  avgShift	(I-221)

Otherwise (cIdx is equal to 1 or 2), the following applies:

- icWeight = 1                                                   	(I-2xx)

- icOffset = ( sumCur − sumRef  + avgOffset )  >>  avgShift	(I-2xx)

Table I-10 – Specification of divCoeff depending on sDenomDiv

| sDenomDiv | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| divCoeff | 0 | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 |
| sDenomDiv | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| divCoeff | 2521 | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 |
| sDenomDiv | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| divCoeff | 1260 | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 |
| sDenomDiv | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| divCoeff | 840 | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 |
| sDenomDiv | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |  |
| divCoeff | 630 | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 |  |