#### Picture parameter set RBSP syntax (no change)

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| long_term_ref_pics_present_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |

| short_term_ref_pic_set_pps_flag | u(1) |
| --- | --- |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |

| if( aps_sample_adapt ive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| --- | --- |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count | u(8) |
| byte_align( ) |  |
| sao_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count | u(8) |
| byte_align( ) |  |
| alf_param( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else |  |
| pic_order_cnt_lsb | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### Short-term reference picture set syntax (no change)

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |

### Adaptation parameter set RBSP semantics

…

short_term_ref_pic_set_pps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active picture parameter set. short_term_ref_pic_set_pps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active picture parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by ceil( log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets – 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active picture parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_pps_flag )
		StRpsIdx = short_term_ref_pic_set_idx
	else													(7-x)
		 StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to max_num_ref_frames – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics shall be inferred to be equal to 0.

delta_poc_lsb_lt_minus1[ i ] is used to determine the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. delta_poc_lsb_lt_minus1[ i ] shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

The variable DeltaPocLt[ i ] is derived as follows.

if (i  = =  0) 
		DeltaPocLt[ i ] = delta_poc_lsb_lt_minus1[ i ] + 1					(7-x)
	else
		DeltaPocLt[ i ] = delta_poc_lsb_lt_minus1[ i ] + 1 − DeltaPocLt[ i – 1 ]

The value of DeltaPocLt[ i ] shall be in the range of 0 to MaxPicOrderCntLsb, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

…

### Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, idr_pic_id no_output_of_prior_pics_flag and pic_order_cnt_lsb, shall be the same in all slice headers of a coded picture.

…

short_term_ref_pic_set_pps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active picture parameter set. short_term_ref_pic_set_pps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active picture parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by ceil( log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets – 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active picture parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_pps_flag )
		StRpsIdx = short_term_ref_pic_set_idx
	else													(7-x)
		 StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to max_num_ref_frames – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics shall be inferred to be equal to 0.

delta_poc_lsb_lt_minus1[ i ] is used to determine the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. delta_poc_lsb_lt_minus1[ i ] shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

The variable DeltaPocLt[ i ] is derived as follows.

if (i  = =  0) 
		DeltaPocLt[ i ] = delta_poc_lsb_lt_minus1[ i ] + 1					(7-x)
	else
		DeltaPocLt[ i ] = delta_poc_lsb_lt_minus1[ i ] + 1 − DeltaPocLt[ i – 1 ]

The value of DeltaPocLt[ i ] shall be in the range of 0 to MaxPicOrderCntLsb, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

...