| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G355 |
| --- | --- |

| Title: | Joint coding of splitting flag and inter modes in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Zhang Ping Wu | Tel: Email: | +86-13528803560 zhang.wen4@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution is successor to JCTVC-F418 which focused on CAVLC only. It has resolved parsing dependency problem meanwhile maintained coding gain. According test results, -0.1%/-0.4%/-0.6%/-0.6% BD-rate gain was achieved comparing to HM 4.0 for AI-LC/RA-LC/LB-LC/LP-LC configurations.

Introduction

Specifically, 4 coding types (contexts) were defined as shown in table 1.

| CU-depth | Prediction mode-top CU | Prediction mode-left CU | Coding Type |
| --- | --- | --- | --- |
| CU size > 8x8 (min) | SKIP | SKIP | 0 |
|  | INTER | SKIP |  |
|  | SKIP | INTER |  |
|  | INTRA | SKIP |  |
|  | SKIP | INTRA |  |
|  | CU Splitting | SKIP |  |
|  | SKIP | CU Splitting |  |
|  | INTER | INTER |  |
|  | INTRA | INTER | 1 |
|  | INTER | INTRA |  |
|  | CU Splitting | INTER |  |
|  | INTER | CU Splitting |  |
|  | INTRA | INTRA |  |
|  | CU Splitting | INTRA |  |
|  | INTRA | CU Splitting |  |
|  | CU Splitting | CU Splitting |  |
| CU size==8x8(min) | SKIP | SKIP | 2 |
|  | INTER | SKIP |  |
|  | SKIP | INTER |  |
|  | INTRA | SKIP |  |
|  | SKIP | INTRA |  |
|  | INTER | INTER |  |
|  | INTRA | INTER | 3 |
|  | INTER | INTRA |  |
|  | INTRA | INTRA |  |

Table 1 Coding types of CU splitting flag and inter modes

Depend on distribution of each coding type, 31 explicit codeword were defined and signaled per slice using UVLC. Which means the decoder didn't need to gather statistics of previous slices. But at encoder side, such information was still helpful and used in RDO progress. Since codeword was signaled per slice, it should be updated for each slice and for each of 4 coding types if existed. Regarding AMP had been added into HM 4.0 meanwhile inter 4x4 was disabled, the case summary of CU splitting flag and inter modes was modified as shown in table 2.

| PredMode and CU splitting | PartMode | CU size > 8x8 (min) | CU size == 8x8(min) |
| --- | --- | --- | --- |
| MODE_SKIP | PART_2Nx2N | 0 (SKIP_2Nx2N) | 0 (SKIP_2Nx2N) |
| MODE_MRG | PART_2Nx2N | 5 (MRG_2Nx2N) | 5 (MRG_2Nx2N) |
| MODE_INTER | PART_2Nx2N | 1(INTER_2Nx2N) | 1 (INTER_2Nx2N) |
|  | PART_2NxN | 3 (INTER_2NxN) | 3 (INTER_2NxN) |
|  | PART_Nx2N | 4 (INTER_Nx2N) | 4 (INTER_Nx2N) |
|  | PART_2NxnU | 7 (INTER_AMP) | -- |
|  | PART_2NxnD |  |  |
|  | PART_nLx2N |  |  |
|  | PART_nRx2N |  |  |
|  | PART_NxN | -- | 7 (INTER_NxN) disabled |
| MODE_INTRA | PART_2Nx2N | 2 (INTRA_2Nx2N) | 2(INTRA_2Nx2N) |
|  | PART_NxN | -- | 6(INTRA_NxN) |
| SPLIT |  | 6(SPLIT) | -- |
| PCM | PART_2Nx2N | disabled | disabled |

Table 2 Case summary of CU splitting flag and inter modes

Apart from above changes, swapping adaption process in HM 4.0 was incorporated as complementary part to explicit codeword signaling.

WD Text

The current WD

Simulation Results

The proposed algorithm was implemented based on HM 4.0 and tested using NT server 2003 32 bits OS. So, please be noted that the anchor used was a little bit different to BBC‘s as a known problem of ticket #203 (slideshow LDPLC QP==32). For cross check results, please refer to JCTVC-G741 by Samsung and JCTVC-xxx by Qualcomm.

|  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V |  |
| Class A | 0.0% | 0.0% | 0.0% |  |
| Class B | 0.0% | -0.1% | 0.0% |  |
| Class C | -0.1% | -0.1% | -0.1% |  |
| Class D | -0.1% | -0.1% | -0.1% |  |
| Class E | 0.0% | 0.0% | 0.0% |  |
| Class F | -0.1% | -0.1% | -0.1% |  |
| Overall | -0.1% | -0.1% | -0.1% |  |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access LC |  |  |  |  |
|  | Y | U |  | V |  |
| Class A | -0.2% | 0.0% |  | 0.1% |  |
| Class B | -0.3% | -0.2% |  | -0.2% |  |
| Class C | -0.7% | -0.7% |  | -0.6% |  |
| Class D | -0.6% | -0.7% |  | -0.6% |  |
| Class E |  |  |  |  |  |
| Class F | -0.3% | -0.4% |  | -0.4% |  |
| Overall | -0.4% | -0.4% |  | -0.3% |  |
|  | -0.4% | -0.4% |  | -0.3% |  |
| Enc Time[%] | 99% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |
|  | Low delay B LC |  |  |  |
|  | Y | U | V |  |
| Class A |  |  |  |  |
| Class B | -0.3% | -0.2% | -0.2% |  |
| Class C | -0.7% | -0.6% | -0.9% |  |
| Class D | -0.8% | -1.1% | -1.0% |  |
| Class E | -1.2% | -0.7% | -1.3% |  |
| Class F | -0.3% | -0.6% | -0.5% |  |
| Overall | -0.6% | -0.6% | -0.7% |  |
|  | -0.6% | -0.6% | -0.8% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low delay P LC |  |  |  |
|  | Y | U | V |  |
| Class A |  |  |  |  |
| Class B | -0.3% | -0.1% | -0.3% |  |
| Class C | -0.7% | -0.7% | -0.7% |  |
| Class D | -0.8% | -1.0% | -1.3% |  |
| Class E | -1.0% | -0.9% | -0.8% |  |
| Class F | -0.4% | -0.5% | -0.4% |  |
| Overall | -0.6% | -0.6% | -0.7% |  |
|  | -0.6% | -0.6% | -0.7% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

- References

JCTVC-F418 Joint Coding of CU splitting flag and inter modes based on HM 3.1 [W. Zhang (ZTE)].

F. Bossen, “Common test conditions and software reference configurations”, Document no JVCVC-F900, Torino, July 2011.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).