| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0571 |
| --- | --- |

| Title: | A Comparison of HM-5.0 ALF with H0068’s Proposed Single-Shape ALF for Some Video Sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Hsan Guermazi Wang (PoLin) Lai Felix C. A. Fernandes | Email: | faouzi@ebriskvideo.com hsan@ebriskvideo.com wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | eBrisk Video Inc. Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the advantages of using HM-5.0’s two ALF filter shapes. Using both test-set and non-test-set video sequences, it is shown that a single-shape ALF solution doesn’t allow enough adaptivity to handle all kinds of video content. More specifically, compared to HM-5.0, the proposed single shape in [2] yields large coding efficiency losses in several test-set and non-test-set video sequences.

# Introduction

In the 7th JCT-VC meeting held at Geneva, Switzerland, ALF using two shapes, Snowflake5x5 and Cross9x9 [1], both having 9 coefficients, was adopted. The two shapes in [1] are depicted in Figure 1 below.

Figure 1 ALF shapes in HM-5.0

In [2], different single-shape solutions have been proposed, and the shape (depicted in Figure 2) that yields the best complexity-performance trade-offs is considered in our experiments.

Figure 2: Proposed ALF shape in [2]

# Simulation Results

The BD-Rate performance of the proposed single shape is shown in Table 1 for those test-set video sequences where the single shape achieves large losses. Even that many single-shape solutions have been tested, some losses are still observed in some test-set video sequences, as illustrated in Table 1 below. For example, the average BD-Rate loss for the Vidyo1 and Vidyo3 sequences for LDP/LD is 0.5%.

| Sequence | IO |  |  | LD-P |  |  | LD |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BQSquare | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.5% | 0.6% | -0.8% | -0.3% | 0.5% | -0.1% | 0.1% |
| Vidyo1 | -0.2% | -0.1% | -0.1% | 0.5% | 0.1% | 0.2% | 0.3% | -0.2% | 0.0% |  |  |  |
| Vidyo3 | -0.2% | -0.1% | 0.0% | 0.9% | -0.9% | -1.1% | 0.3% | -0.2% | 0.2% |  |  |  |

Table 1 Single shape performance in some test-set sequences

Following the conditions in [3], experiments were also conducted using the HM-5.0 software for some non-test-set sequences. Table 2 compares the BD-Rate performance of the HM-5.0 shapes with that of the single shape proposed in [2].

| Sequence | IO |  |  | LDP |  |  | LD |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ShuttleStart | -0.1% | -0.2% | -0.2% | 0.0% | -0.5% | 0.3% | -0.2% | -1.0% | -0.4% | -0.1% | -0.3% | -0.2% |
| Crew | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | 0.2% | -0.1% | 0.4% | -0.1% |
| Jets | -0.1% | 0.0% | 0.0% | -0.3% | 0.5% | 1.7% | 0.2% | 1.1% | -0.8% | 0.0% | 0.0% | 0.5% |
| SpinCalendar | -0.4% | -0.2% | -0.7% | 0.4% | 0.1% | -0.5% | 0.3% | 0.0% | -0.7% | 0.0% | -0.2% | -0.2% |
| BlueSky | 0.3% | -0.1% | -0.3% | 1.5% | 0.5% | 0.3% | 1.2% | 0.5% | 0.5% | 1.2% | 0.2% | -0.1% |
| RiverBed | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | -0.2% |
| Station2 | -0.1% | 0.0% | 0.0% | 1.2% | -0.1% | -0.3% | 0.7% | 0.4% | 0.1% | 0.3% | -0.3% | 0.0% |
| Sunflower | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% | 0.1% |
| walkingCouple | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.2% |

Table 2 BD-Rate performance of ALF using the single shape of [2] for some non-test-set video sequences

The obtained results show the incapability of the single-shape solution to maintain good coding efficiency for a variety of video sequences. For example, in the BlueSky and Station2 video sequences, the single-shape solution of [2] degrades the coding efficiency for LDP/LD/RA by an average of 1%.

# Conclusions

Compared to the HM-5.0 ALF, the proposed single-shape ALF of [2] degrades noticeably the coding efficiency for some test-set and non-test-set video sequences.

# References

- “CE8.d.1: Snowflake5x5 and cross9x9 for luma and chroma ALF shapes”, JCTVC-G208, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- “CE8.b: ALF with single filter type”, JCTVC-H0068, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- “Common test conditions and software reference configurations”, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

eBrisk and Samsung Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).