INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2012/M23485

February 2012, San Jose, USA

| Source | InterDigital Communications LLC |
| --- | --- |
| Status | Input Document to MPEG |
| Title | Downsampling filters for anchor generation for scalable extensions of HEVC |
| Author | Jie Dong, Yong He, Yan Ye |

# Abstract

This document provides the information on the downsampling filters used to generate downsampled test sequences for anchor generation for scalable extentions of HEVC.

# Introduction

After the Geneva meeting in Nov. 2011, the AHG on scalable extentions of HEVC started the discussion on anchor generation and various parameters associated with the anchors. The approach used to compete spatial scalability is simulcast, where one input video is coded at three resolutions: the original one, the 1.5X downsampled one, and the 2.0X downsampled one. Therefore, two filters to perform 1.5X and 2.0X downsampling are required.

One solution for the downsampling filters is to use the ones in the JSVM software, which are Cosine windowed Sinc function with cut-off frequency at 0.8π in the downsampled domain. Each filter is composed of 16 12-tap polyphase filters. To preserve more high frequency detail, the AHG also suggested increasing the cut-off frequency to 0.9π and 0.95π and keeping other design elements unchanged. After some discussions on the reflector, the consensus was to generate two sets of downsampled video sequences using both the JSVM filters (0.8π cut-off frequency) and the new filters (0.9π cut-off frequency). Then, two sets of simulcast bitstreams are generated. A plan to evaluate these two sets of downsampling filters is still under discussion.

We generated the coefficients of the new filters based on the given design elements and then use the two sets of filters to downsample the test video sequences.

# Description of Downsampling Filters

According to the fundamentals of digital signal processing, ideal impulse response hn is shown in (1),

hn=aMNSincπnaN,   n=-∞, …, +∞                                       (1)

where a is the cut-off frequency with the value in [0, 1], Sinc(.) is defined in (2), and NM is equal to the downsampling ratio, i.e., 1.5 for 1.5X downsampling and 2 for the 2.0X case.

Sincx=1,           x=0sin⁡(x)x,   x≠0                                                    (2)

hn, which has unlimited length, is clipped by an L-point Cosine window, as shown in (3),

wn=cosπnL-1, n=-L-12, …, L-12                                                (3)

and the downsampling filter is the product of the ideal impulse response hn and the window wn, as in (4).

fn=hnwn=aMNSincπnaNcosπnL-1,  n=-L-12, …, L-12                    (4)

fn obtained from (4) consists of real-valued coefficients. To make the new filters have the comparable magnitudes to the JSVM ones, the filter coefficients in fn are multiplied by 128 and rounded to the nearest integers. The impulse and frequency responses of the JSVM filters and the new filters with the design elements as spcified in Section 2 are shown in Figs. 1 and 2, respectively. The filter coefficients are shown in TABLE 1 toTABLE 6.

(a)                                                                            (b)

Fig. 1 Impulse responses of (a) the 1.5X downsampling filter and (b) the 2X downsampling filter

(a)                                                                            (b)

Fig. 2 Frequency responses of (a) the 1.5X downsampling filter and (b) the 2X downsampling filter

TABLE 1 JSVM 1.5X downsampling filter

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [2 | 0 | -10 | 0 | 40 | 64 | 40 | 0 | -10 | 0 | 2 | 0]/128 |
| 1 | [2 | 1 | -9 | -2 | 37 | 64 | 42 | 2 | -10 | -1 | 2 | 0] /128 |
| 2 | [2 | 1 | -9 | -3 | 34 | 64 | 44 | 4 | -10 | -1 | 2 | 0] /128 |
| 3 | [2 | 1 | -8 | -5 | 31 | 63 | 47 | 6 | -10 | -2 | 3 | 0] /128 |
| 4 | [1 | 2 | -8 | -6 | 29 | 62 | 49 | 8 | -10 | -2 | 3 | 0] /128 |
| 5 | [1 | 2 | -7 | -7 | 26 | 61 | 52 | 10 | -10 | -3 | 3 | 0] /128 |
| 6 | [1 | 2 | -6 | -8 | 23 | 60 | 54 | 13 | -10 | -4 | 3 | 0] /128 |
| 7 | [1 | 2 | -6 | -9 | 20 | 59 | 56 | 15 | -10 | -4 | 3 | 1] /128 |
| 8 | [1 | 2 | -5 | -9 | 18 | 57 | 57 | 18 | -9 | -5 | 2 | 1] /128 |
| 9 | [1 | 3 | -4 | -10 | 15 | 56 | 59 | 20 | -9 | -6 | 2 | 1] /128 |
| 10 | [0 | 3 | -4 | -10 | 13 | 54 | 60 | 23 | -8 | -6 | 2 | 1] /128 |
| 11 | [0 | 3 | -3 | -10 | 10 | 52 | 61 | 26 | -7 | -7 | 2 | 1] /128 |
| 12 | [0 | 3 | -2 | -10 | 8 | 49 | 62 | 29 | -6 | -8 | 2 | 1] /128 |
| 13 | [0 | 3 | -2 | -10 | 6 | 47 | 63 | 31 | -5 | -8 | 1 | 2] /128 |
| 14 | [0 | 2 | -1 | -10 | 4 | 44 | 64 | 34 | -3 | -9 | 1 | 2] /128 |
| 15 | [0 | 2 | -1 | -10 | 2 | 42 | 64 | 37 | -2 | -9 | 1 | 2] /128 |

TABLE 2 New 1.5X downsampling filter with 0.9π cut-off

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [ 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0] /128 |
| 1 | [ 0 | 5 | -4 | -11 | 33 | 76 | 40 | -9 | -7 | 5 | 0 | 0] /128 |
| 2 | [ 0 | 5 | -3 | -12 | 28 | 75 | 44 | -7 | -8 | 5 | 1 | 0] /128 |
| 3 | [-1 | 4 | -2 | -13 | 25 | 75 | 48 | -5 | -9 | 5 | 1 | 0] /128 |
| 4 | [-1 | 4 | -1 | -13 | 22 | 73 | 52 | -3 | -10 | 4 | 1 | 0] /128 |
| 5 | [-1 | 4 | 0 | -13 | 18 | 72 | 55 | -1 | -11 | 4 | 2 | -1] /128 |
| 6 | [-1 | 4 | 1 | -13 | 14 | 70 | 59 | 2 | -12 | 3 | 2 | -1] /128 |
| 7 | [-1 | 3 | 1 | -13 | 11 | 68 | 62 | 5 | -12 | 3 | 2 | -1] /128 |
| 8 | [-1 | 3 | 2 | -13 | 8 | 65 | 65 | 8 | -13 | 2 | 3 | -1] /128 |
| 9 | [-1 | 2 | 3 | -12 | 5 | 62 | 68 | 11 | -13 | 1 | 3 | -1] /128 |
| 10 | [-1 | 2 | 3 | -12 | 2 | 59 | 70 | 14 | -13 | 1 | 4 | -1] /128 |
| 11 | [-1 | 2 | 4 | -11 | -1 | 55 | 72 | 18 | -13 | 0 | 4 | -1] /128 |
| 12 | [ 0 | 1 | 4 | -10 | -3 | 52 | 73 | 22 | -13 | -1 | 4 | -1] /128 |
| 13 | [ 0 | 1 | 5 | -9 | -5 | 48 | 75 | 25 | -13 | -2 | 4 | -1] /128 |
| 14 | [ 0 | 1 | 5 | -8 | -7 | 44 | 75 | 28 | -12 | -3 | 5 | 0] /128 |
| 15 | [ 0 | 0 | 5 | -7 | -9 | 40 | 76 | 33 | -11 | -4 | 5 | 0] /128 |

TABLE 3 New 1.5X downsampling filter with 0.95π cut-off

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [-1 | 5 | -3 | -13 | 36 | 81 | 36 | -13 | -3 | 5 | -1 | 0] /128 |
| 1 | [-1 | 5 | -2 | -14 | 32 | 80 | 40 | -12 | -4 | 5 | -1 | 0] /128 |
| 2 | [-1 | 4 | -1 | -14 | 28 | 80 | 44 | -11 | -5 | 5 | -1 | 0] /128 |
| 3 | [-1 | 4 | 1 | -15 | 23 | 78 | 48 | -9 | -7 | 6 | 0 | 0] /128 |
| 4 | [-1 | 3 | 1 | -15 | 19 | 77 | 53 | -7 | -8 | 6 | 0 | 0] /128 |
| 5 | [-1 | 3 | 2 | -14 | 16 | 75 | 57 | -5 | -9 | 5 | 0 | -1] /128 |
| 6 | [-1 | 3 | 3 | -14 | 11 | 73 | 60 | -2 | -10 | 5 | 1 | -1] /128 |
| 7 | [-1 | 2 | 4 | -13 | 7 | 70 | 64 | 1 | -11 | 5 | 1 | -1] /128 |
| 8 | [-1 | 2 | 4 | -12 | 4 | 67 | 67 | 4 | -12 | 4 | 2 | -1] /128 |
| 9 | [-1 | 1 | 5 | -11 | 1 | 64 | 70 | 7 | -13 | 4 | 2 | -1] /128 |
| 10 | [-1 | 1 | 5 | -10 | -2 | 60 | 73 | 11 | -14 | 3 | 3 | -1] /128 |
| 11 | [-1 | 0 | 5 | -9 | -5 | 57 | 75 | 16 | -14 | 2 | 3 | -1] /128 |
| 12 | [ 0 | 0 | 6 | -8 | -7 | 53 | 77 | 19 | -15 | 1 | 3 | -1] /128 |
| 13 | [ 0 | 0 | 6 | -7 | -9 | 48 | 78 | 23 | -15 | 1 | 4 | -1] /128 |
| 14 | [ 0 | -1 | 5 | -5 | -11 | 44 | 80 | 28 | -14 | -1 | 4 | -1] /128 |
| 15 | [ 0 | -1 | 5 | -4 | -12 | 40 | 80 | 32 | -14 | -2 | 5 | -1] /128 |

TABLE 4 JSVM 2.0X downsampling filter

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [0 | -4 | -7 | 11 | 38 | 52 | 38 | 11 | -7 | -4 | 0 | 0] /128 |
| 1 | [0 | -4 | -7 | 9 | 37 | 51 | 40 | 13 | -6 | -7 | 2 | 0] /128 |
| 2 | [0 | -3 | -7 | 8 | 35 | 51 | 41 | 14 | -5 | -7 | 1 | 0] /128 |
| 3 | [0 | -2 | -8 | 6 | 33 | 51 | 42 | 16 | -5 | -7 | 2 | 0] /128 |
| 4 | [0 | -2 | -8 | 5 | 32 | 50 | 43 | 18 | -4 | -8 | 2 | 0] /128 |
| 5 | [0 | -2 | -8 | 4 | 30 | 50 | 45 | 19 | -3 | -8 | 1 | 0] /128 |
| 6 | [0 | -1 | -8 | 2 | 28 | 49 | 46 | 21 | -2 | -8 | 1 | 0] /128 |
| 7 | [0 | -1 | -8 | 1 | 26 | 49 | 47 | 23 | -1 | -8 | 0 | 0] /128 |
| 8 | [0 | 0 | -8 | 0 | 24 | 48 | 48 | 24 | 0 | -8 | 0 | 0] /128 |
| 9 | [0 | 0 | -8 | -1 | 23 | 47 | 49 | 26 | 1 | -8 | -1 | 0] /128 |
| 10 | [0 | 1 | -8 | -2 | 21 | 46 | 49 | 28 | 2 | -8 | -1 | 0] /128 |
| 11 | [0 | 1 | -8 | -3 | 19 | 45 | 50 | 30 | 4 | -8 | -2 | 0] /128 |
| 12 | [0 | 2 | -8 | -4 | 18 | 43 | 50 | 32 | 5 | -8 | -2 | 0] /128 |
| 13 | [0 | 2 | -7 | -5 | 16 | 42 | 51 | 33 | 6 | -8 | -2 | 0] /128 |
| 14 | [0 | 1 | -7 | -5 | 14 | 41 | 51 | 35 | 8 | -7 | -3 | 0] /128 |
| 15 | [0 | 2 | -7 | -6 | 13 | 40 | 51 | 37 | 9 | -7 | -4 | 0] /128 |

TABLE 5 New 2.0X downsampling filter with 0.9π cut-off

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [2 | -3 | -9 | 6 | 39 | 58 | 39 | 6 | -9 | -3 | 2 | 0] /128 |
| 1 | [2 | -3 | -9 | 4 | 38 | 58 | 43 | 7 | -9 | -4 | 1 | 0] /128 |
| 2 | [2 | -2 | -9 | 2 | 35 | 58 | 44 | 9 | -8 | -4 | 1 | 0] /128 |
| 3 | [1 | -2 | -9 | 1 | 34 | 58 | 46 | 11 | -8 | -5 | 1 | 0] /128 |
| 4 | [1 | -1 | -8 | -1 | 31 | 57 | 48 | 13 | -8 | -5 | 1 | 0] /128 |
| 5 | [1 | -1 | -8 | -2 | 29 | 56 | 49 | 15 | -7 | -6 | 1 | 1] /128 |
| 6 | [1 | 0 | -8 | -3 | 26 | 55 | 51 | 17 | -7 | -6 | 1 | 1] /128 |
| 7 | [1 | 0 | -7 | -4 | 24 | 54 | 52 | 19 | -6 | -7 | 1 | 1] /128 |
| 8 | [1 | 0 | -7 | -5 | 22 | 53 | 53 | 22 | -5 | -7 | 0 | 1] /128 |
| 9 | [1 | 1 | -7 | -6 | 19 | 52 | 54 | 24 | -4 | -7 | 0 | 1] /128 |
| 10 | [1 | 1 | -6 | -7 | 17 | 51 | 55 | 26 | -3 | -8 | 0 | 1] /128 |
| 11 | [1 | 1 | -6 | -7 | 15 | 49 | 56 | 29 | -2 | -8 | -1 | 1] /128 |
| 12 | [0 | 1 | -5 | -8 | 13 | 48 | 57 | 31 | -1 | -8 | -1 | 1] /128 |
| 13 | [0 | 1 | -5 | -8 | 11 | 46 | 58 | 34 | 1 | -9 | -2 | 1] /128 |
| 14 | [0 | 1 | -4 | -8 | 9 | 44 | 58 | 35 | 2 | -9 | -2 | 2] /128 |
| 15 | [0 | 1 | -4 | -9 | 7 | 43 | 58 | 38 | 4 | -9 | -3 | 2] /128 |

TABLE 6 New 2.0X downsampling filter with 0.95π cut-off

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [2 | -2 | -9 | 3 | 40 | 61 | 40 | 3 | -9 | -2 | 2 | 0] /128 |
| 1 | [2 | -1 | -9 | 1 | 37 | 61 | 42 | 5 | -10 | -2 | 2 | 0] /128 |
| 2 | [2 | -1 | -9 | 0 | 35 | 61 | 44 | 7 | -10 | -3 | 2 | 0] /128 |
| 3 | [2 | 0 | -9 | -2 | 32 | 60 | 46 | 9 | -9 | -3 | 2 | 0] /128 |
| 4 | [2 | 0 | -8 | -3 | 30 | 59 | 48 | 11 | -9 | -4 | 2 | 0] /128 |
| 5 | [1 | 1 | -8 | -4 | 27 | 59 | 50 | 13 | -9 | -4 | 2 | 0] /128 |
| 6 | [1 | 1 | -7 | -6 | 25 | 57 | 52 | 15 | -8 | -5 | 2 | 1] /128 |
| 7 | [1 | 1 | -7 | -6 | 22 | 57 | 54 | 17 | -8 | -6 | 2 | 1] /128 |
| 8 | [1 | 1 | -6 | -7 | 20 | 55 | 55 | 20 | -7 | -6 | 1 | 1] /128 |
| 9 | [1 | 2 | -6 | -8 | 17 | 54 | 57 | 22 | -6 | -7 | 1 | 1] /128 |
| 10 | [1 | 2 | -5 | -8 | 15 | 52 | 57 | 25 | -6 | -7 | 1 | 1] /128 |
| 11 | [0 | 2 | -4 | -9 | 13 | 50 | 59 | 27 | -4 | -8 | 1 | 1] /128 |
| 12 | [0 | 2 | -4 | -9 | 11 | 48 | 59 | 30 | -3 | -8 | 0 | 2] /128 |
| 13 | [0 | 2 | -3 | -9 | 9 | 46 | 60 | 32 | -2 | -9 | 0 | 2] /128 |
| 14 | [0 | 2 | -3 | -10 | 7 | 44 | 61 | 35 | 0 | -9 | -1 | 2] /128 |
| 15 | [0 | 2 | -2 | -10 | 5 | 42 | 61 | 37 | 1 | -9 | -1 | 2] /128 |