| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012 | Document: JCTVC-I0116 M24255 |
| --- | --- |

| Title: | Temporal merging candidate derivation with reduced complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution proposes to fix the reference index for temporal merging candidate (TMVP) of the merging candidate list to 0. Such a simplification increases the cache efficiency of buffering reference block for merge motion estimation, and facilitates low-complexity TMVP derivation on both encoder and decoder side by leveraging the motion data down-sampling in the HEVC. To compensate the slight loss (0.1%) in LB-Main and LP-Main, it is also proposed to add a simple pruning process (maximum one additional MVP comparison) for the TMVP; the TMVP is pruned against the first available spatial merging candidate before appending to the merging candidate list. Adding both changes together leads to a slight gain (0.1% to 0.2%) in various sequence classes and configurations when compared to the HM6.0.

# Introduction

In the current HM6.0 design [2], the reference index derivation for the temporal merging candidate ( TMVP) of merge mode depends availability of the bottom right spatial neighboring PU (A1, see Fig. 1). The reference index for TMVP of merge mode is set to that of the bottom right spatial neighboring PU A1 if the following conditions are all true:

- A1 is not in different slice or outside picture

- The current PU is the first PU of a CU

- A1 is not intra-coded

- A1 and the current PU belongs to different parallel merge regions

Otherwise, the reference index for the TMVP is set to 0.

Fig. 1  derivation of reference index for temporal merging candidates

There are several disadvantages of deriving the TMVP reference index of merge mode based on the availability of spatial neighbors.

- For the merge motion estimation on the encoder side in which the rate-distortion cost for merging candidates are calculated, the TMVP oftentimes goes beyond the major search window and the reference block for TMVP needs to be loaded separately. Changing the reference index for TMVP from PU to PU will require loading reference blocks from different reference pictures from PU to PU, which inevitably reduces cache efficiency and increases memory bandwidth for merge estimation.

- Motion data are down-sampled in the HEVC in which each 16x16 region has one set of motion data.  To reduce complexity a smart encoder or decoder may take advantage of the motion data down-sampling by pre-calculating TMVPs on 16x16 grid for entire LCU instead of computing them on-the-fly for individual PUs (whose minimum size is 4x4). Having reference index of TMVP changing from PU to PU magnifies the number of combinations for the TMVP scaling based on the temporal distances, and makes such an implementation of TMVP derivation with reduced complexity difficult.

# Algorithm description

To resolve the problems above, we proposed to fix the reference picture index of TMVP for merge mode to 0 regardless of the availability of spatial neighbors. As shown in Fig. 2 the reference index for PUs of a CU is fixed to 0 for the TMVP derivation of merge mode. Such a simplification will increase the cache efficiency for merge motion estimation, and facilitate low-complexity TMVP derivation on both encoder and decoder side by leveraging the motion data down-sampling in the HEVC.

Fig. 2  Proposed derivation of reference index for temporal merging candidates

Fixing the TMVP reference picture index to 0 in merge mode can lead to a slight coding efficiency

degradation  (up to 0.1%). The is due to fact that the TMVP is added to the merging candidate list without

pruning in HM6.0, but fixing the TMVP reference index to 0 increases the likelihood that the TMVP is

duplicated relative to one of the spatial merging candidates in the list.

Fig. 3  Diagram of merging candidate list derivation process after adding pruning process for TMVP

To resolve the quality issue, we further propose to change the merging candidate list derivation process

by introducing a simple pruning process for the TMVP.

As shown in Fig. 3, the TMVP (if available) is pruned against the first spatial merging candidate (i.e. merge_idx = 0) in the list before adding it to the list. The TMVP pruning process works as follows:

- If TMVP is available but none of spatial merging candidates are available, the TMVP is directly added to the list and set to be the first merging candidate in the merging candidate list.

- If TMVP is available and at least one of spatial merging candidates is available, the TMVP is appended to the list only if the TMVP has different motion data from the first merging candidate in the list. Otherwise, the TMVP is discarded.

This pruning process requires only one motion data comparison operation in the worst case, it won’t introduce additional latency. The derivation of TMVP requires more cycles than the derivation of spatial merging candidates, the first merging candidate in the list is already available when the TMVP is being derived, the simple TMVP pruning can be absorbed by the following processing steps.

# Test Settings and Conditions

The simulations of this document have used HM6.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

In Table 1 summarizes the experiments results for fixing the TMVP reference index to 0 for the merging candidate list derivation, slight loss (0.1%) is seen in LB-Main and LP-Main configuration. No loss has been observed in other configurations. Please refer to HM-6.0-TI-anchor_vs _TMVP-ref0.xls for detail.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.3% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% |
| Class D | 0.1% | 0.7% | 1.1% | 0.1% | -0.3% | -0.3% |
| Class E | 0.1% | 0.5% | 0.5% | 0.0% | 0.2% | 0.3% |
| Overall | 0.1% | 0.2% | 0.5% | 0.0% | 0.0% | 0.0% |
|  | 0.1% | 0.3% | 0.6% | 0.0% | 0.1% | 0.0% |
| Class F | -0.1% | 0.0% | 0.1% | 0.0% | -0.8% | 0.2% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.1% | 0.4% | 0.0% | 0.2% | -0.4% |
| Class E | 0.1% | 0.0% | 0.5% | 0.0% | 0.7% | -0.5% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.2% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | -0.2% |
| Class F | 0.0% | -0.2% | -0.2% | -0.1% | 0.6% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1 experimental results on fixing the TMVP reference index to 0

In Table 2 summarizes the experiments results for adding the simple TMVP pruning process to the merging candidate list derivation, slight gain of 0.1% is seen in LB-Main, LP-Main, LB-HE10 and LP-HE10 configurations. In Class E sequences the average gain amounts to 0.3% in LB-HE10 and 0.4% in LP-Main.  Please refer to HM-6.0-TI-anchor_vs _TMVP-pruning0.xls for detail.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.3% | 0.1% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.4% | 0.0% | -0.2% | -0.6% |
| Class E | -0.1% | -0.3% | 0.0% | -0.3% | -0.4% | 0.2% |
| Overall | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.1% | 0.1% | -0.1% | -0.2% | -0.2% |
| Class F | -0.2% | -0.6% | -0.3% | 0.0% | -0.4% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | -0.1% |
| Class C | -0.1% | -0.5% | -0.5% | -0.1% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.6% | 0.0% | 0.2% | -0.4% |
| Class E | -0.4% | -0.2% | -0.2% | -0.2% | 0.6% | 0.4% |
| Overall | -0.1% | -0.2% | -0.3% | -0.1% | 0.1% | -0.1% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | 0.2% | -0.1% |
| Class F | -0.1% | -0.1% | -0.4% | -0.1% | 0.5% | -0.9% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2 experimental results on adding simple pruning process for TMVP

In Table 2 provides the experiments results for fixing the TMVP reference index to 0 and adding the simple TMVP pruning process to the merging candidate list derivation. Slight gain (0.1% To 0.2%) has been observed in various classes of sequences and various configurations. Please refer to HM-6.0-TI-anchor_vs _TMVP-pruning0&ref0.xls for detail.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | 0.2% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.1% | -0.1% | 0.0% | 0.2% | -0.2% |
| Class D | 0.1% | 0.4% | 0.6% | 0.1% | -0.2% | 0.2% |
| Class E | -0.1% | -0.1% | 0.2% | -0.2% | -0.3% | 0.0% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class F | -0.2% | -0.4% | -0.3% | 0.0% | -0.8% | 0.4% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | -0.4% | -0.1% | -0.1% | 0.0% | -0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.0% | 0.6% | -0.5% |
| Class E | -0.1% | 0.0% | 0.3% | -0.2% | 0.4% | 0.0% |
| Overall | 0.0% | -0.1% | 0.1% | -0.1% | 0.2% | -0.2% |
|  | 0.0% | -0.1% | 0.1% | -0.1% | 0.2% | -0.1% |
| Class F | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3 experimental results on fixing the TMVP reference index to 0 and adding simple pruning process for TMVP

# Conclusion and recommendation

Fixing the reference index for TMVP lowers the memory bandwidth requirements of  merge motion estimation, and enables the derivation of temporal merging candidates (TMVP) with reduced complexity by leveraging the motion data down-sampling to pre-calculate TMVPs on 16x16 grids instead of individual PUs. The changes lead to slight coding gain and no complexity increase. It is recommended to adopt both changes into the spec.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1100, San Jose, CA, USA, February 2012.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) Test Model 6 (HM 6) Encoder Description” JCT-VC Document, JCTVC-G1003, San Jose, CA, USA, February 2012.

.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text

Replace 8.5.2.1.1 Derivation process for luma motion vectors for merge mode with (changes marked in yellow, for tool 1 which fixes TMVP refIdx = 0)

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPSW and nPSH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause 8.5.2.1.2 is invoked with luma location ( xP, yP ), the variable singleMCLFlag, the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows.

- If the following conditions are true, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available

- the PartIdx is equal to 0

- PredMode is not MODE_INTRA

- (xP >> (log2_parallel_merge_level_minus2 + 2)) ! = 
((xP − 1) >> (log2_parallel_merge_level_minus2 + 2))	| |
(yP >> (log2_parallel_merge_level_minus2 + 2)) ! = 
((yP + nPSH − 1) >> (log2_parallel_merge_level_minus2 + 2)

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause 8.5.2.1.7 is invoked with luma location ( xP, yP ), refIdxLX equal to 0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-97)

predFlagLXCol = availableFlagLXCol	(8-98)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause 8.5.2.1.3 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-99)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-100)

refIdxLX = refIdxLXN	(8-101)

predFlagLX = predFlagLXN	(8-102)

Replace 8.5.2.1.1 Derivation process for luma motion vectors for merge mode with (changes marked in yellow, for tool 1 which fixes TMVP refIdx = 0 and tool 2 which adds simple TMVP pruning))

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPSW and nPSH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause 8.5.2.1.2 is invoked with luma location ( xP, yP ), the variable singleMCLFlag, the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows.

- If the following conditions are true, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available

- the PartIdx is equal to 0

- PredMode is not MODE_INTRA

- (xP >> (log2_parallel_merge_level_minus2 + 2)) ! = 
((xP − 1) >> (log2_parallel_merge_level_minus2 + 2))	| |
(yP >> (log2_parallel_merge_level_minus2 + 2)) ! = 
((yP + nPSH − 1) >> (log2_parallel_merge_level_minus2 + 2)

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause 8.5.2.1.7 is invoked with luma location ( xP, yP ), refIdxLX equal to 0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-97)

predFlagLXCol = availableFlagLXCol	(8-98)

If availableFlagCol  is equal to 1 and all the following conditions are true, availableFlagCol is set to equal to 0.

- If availableFlagN is equal to 1, with N being the first available spatial merging candidate replaced by in an order of A1, B1, B0, A0 or B2.

- If  predFlagLXCol is equal to predFlagLXN,  refIdxLXCol is equal to refIdxLXN, mvLXCol[0]  is equal  to mvLXN[0] and mvLXCol[1] is equal to  mvLXN[1], with X being replaced by 0 and 1

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause 8.5.2.1.3 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-99)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-100)

refIdxLX = refIdxLXN	(8-101)

predFlagLX = predFlagLXN	(8-102)