| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0340r1 |
| --- | --- |

| Title: | Signalling of long-term reference pictures in the SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang Ying Chen 5775 Morehouse Dr San Diego, CA 92121, USA Chong Soon Lim (Panasonic) Sachin Deshpande (Sharp) | Tel: Email: | 1-858-658-5804 aramasub@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com 1-858-845-6589 cheny@qualcomm.com chongsoon.lim@sg.panasonic.com +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Qualcomm Incorporated, Sharp, and Panasonic |  |  |

_____________________________

# Abstract

This document proposes to enable the inclusion of candidate long-term reference pictures, as part of the reference picture set signalling in the sequence parameter set.

Revision 1 of this document includes a slightly changed syntax for signalling of long-term reference pictures in the SPS, and the corresponding changed semantics and decoding process, after review of the original version of this document, as described in Section 3 in JCTVC-I0340r1.doc. The presentation slides were not updated to include the changed syntax.

# Introduction

The current long-term reference picture (LTRP) signaling is present only in the slice header. Thus, for any particular LTRP, the POC information used to identify the LTRP must be included in all slice headers, as part of the reference picture set (RPS) information, since the LTRP is firstly identified as a long term reference until the LTRP becomes no longer needed for inter prediction reference. However, an LTRP may stay in the DPB for a long period, thus the current LTRP signaling requires the POC information of the LTRP to be repeated in all slice headers of pictures within the long period.

In many scenarios wherein LTRPs are pre-selected or the bitstream is multi-pass encoded, reference pictures that would be used for LTRPs and would stay in the DPB for long periods are known at the beginning of the bitstream. In these scenarios, signaling of candidate LTRPs in the SPS can save a lot of bits to be transmitted, because in this case, instead of transmitting directly and repeatedly the POC information of the LTRPs in the slice header, only the indices to the candidate LTRPs included in the SPS need to be transmitted.

Thus we propose to enable the inclusion of candidate long-term reference pictures in the SPS for reference picture set signaling. The proposed syntax and semantics changes are provided in the remainder of this document.

# Proposal

## SPS changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| long_term_ref_pic_poc_len_delta | ue(v) |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| long_term_ref_pic_poc_sps[ i ] | i(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| ... |  |
| } |  |

long_term_ref_pic_poc_len_delta plus log2_max_pic_order_cnt_lsb_minus4 plus 4 specifies the length, in bits, of the long_term_ref_pic_poc_sps[ i ] syntax element. The value of long_term_ref_pic_poc_len_delta shall be in the range of 0 to 32 − log2_max_pic_order_cnt_lsb_minus4 − 4, inclusive.

num_long_term_ref_pics_sps specifies the number of long-term reference pictures that are specified in the sequence parameter set. The value of num_long_term_ref_pics_sps shall be in the range of 0 to 32, inclusive.

long_term_ref_pic_poc_sps[ i ] specifies the picture order count of the i-th long-term reference picture specified in the sequence parameter set. The number of bits used to represent long_term_ref_pic_poc_sps[ i ] shall be equal to long_term_ref_pic_poc_len_delta + log2_max_pic_order_cnt_lsb_minus4 + 4.

used_by_curr_pic_lt_sps_flag[ i ] equal to 0 specifies that the i-th long-term reference picture specified in the sequence parameter set is not used for reference by the current picture.

## Slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps; i++ ) |  |
| long_term_idx_sps[ i ] | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

num_long_term_sps specifies the number of long-term reference pictures that are specified in the active sequence parameter set and that are to be included in the long-term reference picture set of the current picture. If num_long_term_sps is not present, the value is inferred to be equal to 0. The value of num_long_term_sps shall be in the range of 0 to Min( num_long_term_ref_pics_sps, max_dec_pic_buffering[ max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] ), inclusive.

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to max_dec_pic_buffering[ max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] – num_long_term_ref_pics_sps, inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

long_term_idx_sps[ i ] specifies the index, to the list of long-term reference pictures specified in the active sequence parameter set, of the i-th long-term reference picture inherited from the referred sequence parameter set to the long-term reference picture set of the current picture. The value of long_term_idx_sps[ i ] shall be in the range of 0 to num_long_term_ref_pics_sps - 1, inclusive.

.

# Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause 8.3.3. The process may result in marking one or more reference pictures as "unused for reference".

….

- Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-5)

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ )
	if( delta_poc_msb_present_flag[ i ] )
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] )* MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] ) * MaxPicOrderCntLsb
	else
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb
for( i = 0, ; i < num_long_term_sps; i++ )
	if( used_by_curr_pic_lt_sps_flag[ long_term_idx_sps[ i ] ] )
		PocLtCurr[ j++ ] = long_term_ref_pic_poc_sps[ long_term_idx_sps[ i ] ]
	else
		PocLtFoll[ k++ ] = long_term_ref_pic_poc_sps[ long_term_idx_sps[ i ] ]

NumPocLtCurr = j
NumPocLtFoll = k

# Revised signalling of LTRPs in the SPS

## SPS syntax and semantics changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| ... |  |
| } |  |

num_long_term_ref_pics_sps specifies the number of long-term reference pictures that are specified in the sequence parameter set. The value of num_long_term_ref_pics_sps shall be in the range of 0 to 32, inclusive.

lt_ref_pic_poc_lsb_sps[ i ] specifies the least significant bits of the picture order count of the i-th long-term reference picture specified in the sequence parameter set. The number of bits used to represent lt_ref_pic_poc_lsb_sps[ i ] shall be equal to log2_max_pic_order_cnt_lsb_minus4 + 4.

used_by_curr_pic_lt_sps_flag[ i ] equal to 0 specifies that the i-th long-term reference picture specified in the sequence parameter set is not used for reference by the coded picture that refers to the sequence parameter set and includes the i-th long-term reference pictures specified in the sequence parameter set in the reference picture set.

## Slice header syntax and semantics changes

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| if( num_long_term_ref_pics_sps ) |  |
| num_long_term_sps | ue(v) |
| for( i = 0; i < num_long_term_sps; i++ ) |  |
| long_term_idx_sps[ i ] | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture and that are directly signalled in the slice header. The value of num_long_term_pics shall be in the range of 0 to max_dec_pic_buffering[ max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] – num_long_term_sps, inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

num_long_term_sps specifies the number of long-term reference pictures that are specified in the active sequence parameter set and that are to be included in the long-term reference picture set of the current picture. If num_long_term_sps is not present, the value is inferred to be equal to 0. The value of num_long_term_sps shall be in the range of 0 to Min( num_long_term_ref_pics_sps, max_dec_pic_buffering[ max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] – num_long_term_pics ), inclusive.

long_term_idx_sps[ i ] specifies the index, to the list of long-term reference pictures specified in the active sequence parameter set, of the i-th long-term reference picture inherited from the referred sequence parameter set to the long-term reference picture set of the current picture. The value of long_term_idx_sps[ i ] shall be in the range of 0 to num_long_term_ref_pics_sps - 1, inclusive.

...

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when DeltaPocLt[ i ] is equal to DeltaPocLt[ j ] for any value of j in the range of 0 to num_long_term_pics − 1, inclusive, and not being equal to i. When not present, the value of delta_poc_msb_present_flag[ i ] is inferred to be 0.

## Change on the decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause 8.3.3. The process may result in marking one or more reference pictures as "unused for reference".

….

- Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-5)

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ )
	if( delta_poc_msb_present_flag[ i ] )
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] )* MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] ) * MaxPicOrderCntLsb
	else
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb
for( i = 0, ; i < num_long_term_sps; i++ )
	if( used_by_curr_pic_lt_sps_flag[ long_term_idx_sps[ i ] ] )
		PocLtCurr[ j++ ] = lt_ref_pic_poc_lsb_sps[ long_term_idx_sps[ i ] ]
	else
		PocLtFoll[ k++ ] = lt_ref_pic_poc_lsb_sps[ long_term_idx_sps[ i ] ]

NumPocLtCurr = j
NumPocLtFoll = k

# Description of experiments

The test case 2.7 in the common conditions for reference picture marking and list construction, described in JCTVC-H0725[1], is used to determine the signalling performance of the proposed method. The test condition describes two scenes alternating at fixed time intervals, and scene changes occur at pre-determined time points. The first case considers scene intervals of 10s – 30s – 10s – 30s and the second case considers scene intervals of 20s – 60s – 20s – 60s. A comparison of the bit-counts of the syntax elements related to long-term reference picture signalling in the proposed method and the CD [2] is presented below. Results shown below include the bit-counts for two full scene-cycles (80s in Test 1 and 160 s in Test 2). The proposed method provides, on an average, 65% reduction in the bit-count of the related syntax elements. The syntax elements used to compute the bit-counts are

- For CD: num_long_term_pics, delta_poc_lsb_lt, delta_poc_msb_present_flag, delta_poc_msb_cycle_lt_minus1, used_by_curr_pic_lt_flag

- For proposed method: all the elements used for the CD bit-count, and long_term_ref_pic_poc_len_delta, num_long_term_ref_pic_sps, long_term_ref_pic_poc_sps, and used_by_curr_lt_sps_flag.

|  | CD | Proposed method |
| --- | --- | --- |
| Test 1: 10s – 30s – 10s – 30s |  |  |
| LTRP signalling-related bit-count in SPS | 0 | 94 |
| LTRP signalling-related bit-count in slice header | 63771 | 22482 |
| Total LTRP signalling-related bit-count | 63771 | 22576 |
| % reduction |  | 65 % |
| Test 2: 20s – 60s – 20s – 60s |  |  |
| LTRP signalling-related bit-count in SPS | 0 | 100 |
| LTRP signalling-related bit-count in slice header | 127479 | 44994 |
| Total LTRP signalling-related bit-count | 127479 | 45094 |
| % reduction |  | 65 % |

# References

[1] Y-K. Wang, M. M. Hannuksela, T. K. Tan, R. Sjöberg, and Yan Ye, “Common conditins for reference picture marking and list construction proposals,” JCT-VC documents JCTVC-H0725, San Jose, February 2012.

[2] B. Bross, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCT-VC document JCTVC-H1003_dK, San Jose, February, 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).