| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0347 |
| --- | --- |

| Title: | Simplification of inter-RPS prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5804 aramasub@qualcomm.com 1-858-845-6589 cheny@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current working draft of HEVC, inter prediction between RPS candidates in SPS is enabled. A simplification of the current syntax design for inter-RPS prediction is proposed in this document.

# Introduction

In the current HEVC WD, prediction among the short-term RPS candidates inside an SPS is enabled by the highlighted syntax design as shown in the short-term reference picture set syntax table copied below.

We received complaints from implementers about the complexity of the syntax and associated decoding process.

In some cases, when the RPS of a picture A is used to predict the RPS of a picture B, most likely the picture A will be added into the RPS of the picture B.

In most cases, used_by_curr_pic_flag can be predicted from the flags associated with the reference RPS. It is less efficient and not clean to still signal this flag for inter-RPS prediction.

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

# Proposal

In this document, the following modifications are proposed:

- delta_rps_sign and abs_delta_rps_minus1 are merged into one syntax element;

- used_by_curr_pic_flag for each reference picture in the inter-predicted RPS is not signalled and always derived to be the same value as that flag of the corresponding picture in the reference RPS;

- An inter-predicted RPS can include not only the pictures in a reference RPS, but also the current picture of a reference RPS.

## Short-term reference picture set syntax

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| delta_rps_minus1 | ie(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| add_predictor_rps_pic_flag | u(1) |
| num_ref_pics_drops_from_begin | ue(v) |
| num_ref_pics_drops_from_end | ue(v) |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

## Short-term reference picture set semantics

delta_rps_minus1 specifies the POC difference between picA and picB, where picA is associated with DeltaPoc[ RIdx ] and picB is associated with DeltaPoc[ idx ]. Note picA is called the current picture of DeltaPoc[ RIdx ] and picB is called the current picture of DeltaPoc[ idx ].

The variable DeltaRPS is derived as follows.

DeltaRPS = sign (delta_rps_minus1) ? (delta_rps_minus1 + 1) : (delta_rps_minus1 - 1)

add_predictor_rps_pic_flag equal to 1 indicates that the previous picture in decoding order using the reference RPS (with index equal to RIdx = idx - delta_idx_minus1-1) is included in the RPS with index equal to idx. add_predictor_rps_pic_flag equal to 0 indicates the previous picture in decoding order of the reference RPS (with index equal to RIdx ) is not included in the RPS with index equal to idx.

num_ref_pics_drops_from_begin specifies the number of reference pictures starting from the beginning of the reference RPS (with index equal to RIdx) that are not used to predict the current RPS (with index equal to idx). Let RNPP and RNNP be  num_positive_pics and num_negative_pics of the reference RPS, respectively. The value of num_ref_pics_drops_from_begin shall be in the range of 0 to RNPP + RNNP - 1, inclusive.

num_ref_pics_drops_from_end specifies the number of reference pictures starting from the end of the reference RPS (with index equal to RIdx) that are not used to predict the current RPS (with index equal to idx). num_ref_pics_drops_from_end is in the range of 0 to RNPP + RNNP -1- num_ref_pics_drops_from_begin, inclusive.

Note that it is assumed that the reference RPS (short-term) has been sorted in a way that all the entries are ordered in the ascending order of POC values.

## Decoding processes

When inter_ref_pic_set_prediction_flag is equal to 1, the variables DeltaPocS0[ idx ][ i0 ], UsedByCurrPicS0[ idx ][ i0 ], DeltaPocS1[ idx ][ i1 ], UsedByCurrPicS1[ idx ][ i1], NumNegativePics[ idx ] and NumPositivePics[ idx ] are derived as follows.

for( j = num_ref_pics_drops_from_begin, i0 = 0, i1 = 0, insertFlag = add_predictor_rps_pic_flag; j <= NumDeltaPocs[ RIdx ]- num_ref_pics_drops_from_end; j++ )
{
	if  ((DeltaPocSorted[ RIdx ][ j ]>DeltaRPS | | 
		j= = NumDeltaPocs[ RIdx ]- num_ref_pics_drops_from_end)  && insert_flag) {
		DPoc =  DeltaRPS
		currFlag = 1 
		insertFlag = 0
		if (j< NumDeltaPocs[RIdx]) j--	
	}
	else {
		DPoc = DeltaPocSorted[ RIdx ][ j ] + DeltaRPS
		currFlag = UsedByCurrPic[ Ridx ][ j ]
	}
	if( DPoc < 0 ) {
		DeltaPocS0[ idx ][ i0 ] = DPoc
		UsedByCurrPicS0[ idx ][ i0 ] = currFlag used_by_curr_pic_flag[ j ] 
		i0++
	} else {
		DeltaPocS1[ idx ][ i1 ] = DPoc
		UsedByCurrPicS1[ idx ][ i1 ] = currFlag used_by_curr_pic_flag[ j ] 
		i1++
	} 
}
NumNegativePics[ idx ] = i0

NumPositivePics[ idx ] = i1

Regardless the value of inter_ref_pic_set_prediction_flag, the following process is used to derive a sorted list of delta POC values and the associated used_by_curr_pic_sx_flag values of the entries in the RPS. The arrays DeltaPocSorted[ idx ] and UsedByCurrPic[ idx ][ ] are derived as follows.

for(j = 0, i = NumNegativePics[ idx ] - 1; i >= 0; i--, j++) {

DeltaPocSorted[ idx ][ j ] = DeltaPocS0[ idx ][ i ]
UsedByCurrPic[ idx ][ j ] = UsedByCurrPicS0[ idx ][ i ]

}

for(j = NumNegativePics[ idx ], i = NumNegativePics[ idx ] ; i < NumPositivePics[ idx ] + NumNegativePics[ idx ]; i++, j++) {

DeltaPocSorted[ idx ][ j ] = DeltaPocS1[ idx ][ i ]
UsedByCurrPic[ idx ][ j ] = UsedByCurrPicS1[ idx ][ i ]

}

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).