| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0050r1 |
| --- | --- |

| Title: | Modified deblocking filter for the ref_idx framework |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yuwen He, Jie Dong, Xiaoyu Xiu 9710 Scranton Rd, #250 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4803 {yan.ye,yuwen.he,jie.dong,xiaoyu.xiu}@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

The current SHVC software SMuC v0.1.1 supports both IntraBL based and ref_idx based signalling of inter layer prediction. In addition to the basic signalling difference, it is observed that different deblocking filter schemes are used between IntraBL and ref_idx. This contribution examines the impact on coding efficiency when different deblocking filter schemes are used, and shows that the difference in deblocking filter handling especially impacts the chroma components. Simulation results show that a small deblocking filter modification for the ref_idx branch can improve the R-D performance of the chroma components for almost all test cases.

# Introduction

The current SHVC software SMuC v0.1.1 supports both IntraBL based and ref_idx based inter layer prediction signalling. The ref_idx based signalling requires HLS-only changes by placing the inter layer reference picture in the DPB, whereas IntraBL uses a flag at the CU level to signal inter layer prediction. In addition to this basic signalling difference, it is observed that different deblocking schemes are used for these signalling methods.

- In the IntraBL branch, inter layer predicted blocks are treated as intra blocks, and deblocking filter boundary strength is always set to 2.

- In the ref_idx branch, inter layer predicted blocks are treated as inter blocks, and deblocking filter boundary strength is set to 0 or 1 most of the time, that is, unless the current block’s neighboring block is intra coded.

It is observed that the chroma components are especially negatively impacted due to the weaker boundary strength values being applied to inter layer predicted blocks in the ref_idx branch, as the single layer HEVC deblocking filter design by-passes chroma deblocking altogether unless boundary strength is 2. Table 1 shows the performance comparison between the IntraBL branch in SMuC v0.1.1 and the ref_idx branch in InterDigital’s TE-B2 setting [1]. It is observed that on average, while the performance of the ref_idx branch is same as or better than that of IntraBL for the luma component, the performance of the ref_idx branch lags behind that of IntraBL for the chroma components. For example, the largest EL-only BD rate for the chroma components can be above 3%.

Table 1. Ref_idx (TE-B2 setting 2 + zero MV) vs. IntraBL (SMuC v0.1.1) performance comparison

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.1% | 1.5% | 1.6% |  |  |  |  |  |  |
| Class B | -0.3% | 1.6% | 1.4% | -0.5% | 0.5% | 0.2% |  |  |  |
| Overall (EL+BL) | -0.2% | 1.6% | 1.4% | -0.5% | 0.5% | 0.2% |  |  |  |
| Overall (EL) | -0.5% | 3.2% | 2.9% | -1.9% | 2.4% | 1.2% |  |  |  |
| Enc Time[%] | 186.2% |  |  | 174.9% |  |  |  |  |  |
| Dec Time[%] | 85.3% |  |  | 96.0% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.8% | 1.0% |  |  |  | 0.0% | 1.1% | 1.4% |
| Class B | -0.2% | 1.7% | 1.9% | -0.6% | 0.2% | 0.4% | -0.4% | 1.5% | 2.3% |
| Overall (EL+BL) | -0.2% | 1.4% | 1.7% | -0.6% | 0.2% | 0.4% | -0.2% | 1.4% | 2.1% |
| Overall (EL) | -0.3% | 2.5% | 2.7% | -1.8% | 0.4% | -0.1% | -0.4% | 2.7% | 3.5% |
| Enc Time[%] | 92.4% |  |  | 92.3% |  |  | 99.9% |  |  |
| Dec Time[%] | 92.8% |  |  | 93.2% |  |  | 99.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.7% | 0.5% |  |  |  | 0.3% | 0.7% | 0.6% |
| Class B | 0.0% | 1.5% | 1.4% | -0.3% | 0.5% | 0.3% | 0.0% | 0.1% | 0.2% |
| Overall (EL+BL) | 0.1% | 1.3% | 1.1% | -0.3% | 0.5% | 0.3% | 0.1% | 0.3% | 0.3% |
| Overall (EL) | 0.1% | 2.4% | 2.0% | -1.5% | 1.9% | 0.8% | 0.0% | 0.8% | 0.6% |
| Enc Time[%] | 98.1% |  |  | 87.3% |  |  | 95.5% |  |  |
| Dec Time[%] | 96.7% |  |  | 84.3% |  |  | 95.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# The proposed boundary strength modification for ref_idx

In order to improve coding efficiency for the chroma components, the following modification is applied to the Bs calculation process in the ref_idx branch:

- If either block P or block Q is predicted using inter layer reference, then the boundary strength is set to 2

The following criterion is used to determine whether a block (P or Q) is predicted using inter layer reference:

- If the block is single list predicted, and if its reference picture (either L0 or L1) is the inter layer reference picture, then the block is considered to be predicted using inter layer reference; or

- If the block is bi-predicted, and if the reference pictures on both lists (both L0 and L1) are the inter layer reference picture, then the block is considered to be predicted using inter layer reference.

The remaining part of the boundary strength calculation and deblocking filter process is not changed.

# Simulation results

The proposed scheme is implemented on top of the TE2 software in [1]. Table 2 summarizes the performance of the proposed scheme in comparison to the ref_idx branch (“setting 2 + zero MV” in [1]). Timing is not accurate since the computing farm does not consist of homogenous cores. Except one case (RA 1.5x) where the chroma performance has a small loss, the chroma performance is improved for all other cases, and by up to -3.0% for AI 2x. Change to the luma performance is negligible. Finally, Table 3 summarizes the performance of the proposed scheme in comparison to the IntraBL branch in SMuC v0.1.1.

Table 2. Performance of the proposed deblocking filter modification in comparison to the ref_idx branch (setting 2 + zero MV)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.1% | -1.5% | -1.6% |  |  |  |  |  |  |
| Class B | 0.0% | -1.5% | -1.4% | 0.1% | -0.1% | -0.1% |  |  |  |
| Overall (EL+BL) | 0.0% | -1.5% | -1.4% | 0.1% | -0.1% | -0.1% |  |  |  |
| Overall (EL) | -0.1% | -3.0% | -2.8% | 0.2% | -0.3% | -0.2% |  |  |  |
| Enc Time[%] | 99.4% |  |  | 97.4% |  |  |  |  |  |
| Dec Time[%] | 99.6% |  |  | 98.8% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -1.0% | -0.7% |  |  |  | -0.1% | -0.8% | -0.6% |
| Class B | 0.0% | -0.7% | -0.4% | 0.0% | 0.5% | 0.7% | 0.0% | 0.2% | 0.3% |
| Overall (EL+BL) | -0.1% | -0.8% | -0.5% | 0.0% | 0.5% | 0.7% | 0.0% | -0.1% | 0.0% |
| Overall (EL) | -0.1% | -1.5% | -0.9% | 0.0% | 1.3% | 2.0% | -0.1% | -0.4% | -0.2% |
| Enc Time[%] | 91.0% |  |  | 92.4% |  |  | 92.4% |  |  |
| Dec Time[%] | 100.0% |  |  | 95.9% |  |  | 95.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -1.3% | -1.1% |  |  |  | -0.1% | -1.1% | -1.0% |
| Class B | -0.1% | -1.2% | -1.0% | 0.0% | -0.5% | -0.3% | 0.0% | -0.3% | -0.3% |
| Overall (EL+BL) | -0.1% | -1.3% | -1.1% | 0.0% | -0.5% | -0.3% | -0.1% | -0.5% | -0.5% |
| Overall (EL) | -0.1% | -2.3% | -1.9% | -0.1% | -1.1% | -0.4% | -0.1% | -1.2% | -1.1% |
| Enc Time[%] | 116.5% |  |  | 109.9% |  |  | 115.5% |  |  |
| Dec Time[%] | 114.0% |  |  | 109.2% |  |  | 115.1% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3. Performance of the proposed deblocking filter modification in comparison to the IntraBL branch (SMuC v0.1.1)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.2% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | -0.3% | 0.1% | 0.0% | -0.5% | 0.4% | 0.1% |  |  |  |
| Overall (EL+BL) | -0.3% | 0.0% | 0.0% | -0.5% | 0.4% | 0.1% |  |  |  |
| Overall (EL) | -0.6% | 0.1% | 0.0% | -1.7% | 1.8% | 0.8% |  |  |  |
| Enc Time[%] | 174.7% |  |  | 160.5% |  |  |  |  |  |
| Dec Time[%] | 81.1% |  |  | 89.9% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | 0.3% |  |  |  | 0.0% | 0.2% | 0.8% |
| Class B | -0.3% | 1.0% | 1.5% | -0.6% | 0.7% | 1.1% | -0.4% | 1.7% | 2.5% |
| Overall (EL+BL) | -0.2% | 0.6% | 1.1% | -0.6% | 0.7% | 1.1% | -0.3% | 1.3% | 2.0% |
| Overall (EL) | -0.4% | 1.0% | 1.8% | -1.8% | 1.7% | 1.8% | -0.4% | 2.1% | 3.3% |
| Enc Time[%] | 89.1% |  |  | 93.3% |  |  | 100.4% |  |  |
| Dec Time[%] | 96.9% |  |  | 97.2% |  |  | 103.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.7% | -0.6% |  |  |  | 0.2% | -0.4% | -0.4% |
| Class B | 0.0% | 0.3% | 0.3% | -0.3% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.3% | -0.2% |
| Overall (EL) | -0.1% | 0.0% | 0.1% | -1.6% | 0.7% | 0.3% | -0.1% | -0.5% | -0.5% |
| Enc Time[%] | 105.4% |  |  | 99.0% |  |  | 103.5% |  |  |
| Dec Time[%] | 107.7% |  |  | 98.3% |  |  | 105.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

In this contribution, the deblocking filter boundary strength calculation process is modified for the EL blocks coded using inter layer reference picture. Specifically, Bs is set to 2 if one of the blocks P and Q in EL is predicted using inter layer reference. Simulations show that this modification can reduce the chroma performance gap between ref_idx and IntraBL for most test cases.

In conclusion, the chroma performance gap between ref_idx and IntraBL is primarily because of the difference in the deblocking process. The inter layer prediction signalling in the ref_idx branch is as efficient as, or slightly more efficient than the IntraBL signalling.

# Patent rights declaration(s)

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

X. Xiu, Y. He, Y. He, Y. Ye, TE A2: Inter layer reference picture placement, Document no. JCTVC-L0051, January 2013.