| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0181 |
| --- | --- |

| Title: | Proposed VPS extension semantics and editorial cleanups to syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Ye-Kui Wang Sachin Deshpande | Email: jill@vidyo.com yekuiw@qti.qualcomm.com sdeshpande@sharplabs.com |  |
| Source: | Vidyo, Qualcomm, Sharp |  |  |

_____________________________

# Abstract

The output document from the Shanghai meeting JCTVC-K1007 contains the working design of the syntax for the NAL unit header and VPS extension for the scalability and multiview extensions, but lacks semantics for many of the syntax elements in the VPS extension.  This contribution proposes semantics for those VPS extension syntax elements for which the semantics were missing in JCTVC-K1007.  A correction for an error in the VPS syntax of JCTVC-K1007 is also proposed.  Additionally, some purely editorial changes to the syntax table are proposed which simplify the semantics.  Changes from JCTVC-K1007 are highlighted.

# Video parameter set extension syntax and semantics

## Video parameter set extension syntax

An editorial change is proposed to include a length 16 array of 1-bit scalability_mask_flag syntax elements rather than a single 16-bit field, to simplify referencing of individual bits.

The loop over the dimension_id[ i ][ j ] syntax element is corrected to avoid referencing the no longer present num_dimensions_minus1 syntax element, with some additional editorial changes which simplify the semantics.  The changes relatives to JCTVC-K1007 are highlighted in the table below.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| for( i = 0, numScalabilityTypes = 0 ; i <16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| numScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < numScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

## Video parameter set extension semantics

Additional semantics are proposed, which had been missing in JCTVC-K1007.

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

avc_base_layer_flag equal to 1 specifies that the base layer conforms to Rec. ITU-T H.264 | ISO/IEC 14496-10, equal to 0 specifies that it conforms to this specification.

When avc_base_layer_flag equal to 1, in the Rec. ITU-T H.264 | ISO/IEC 14496-10 conforming base layer, after applying the Rec. ITU-T H.264 | ISO/IEC 14496-10 decoding process for reference picture lists construction the output reference picture lists refPicList0 and refPicList1 (when applicable) does not contain any pictures for which the TemporalId is greater than TemporalId of the coded picture.  All sub-bitstreams of the Rec. ITU-T H.264 | ISO/IEC 14496-10 conforming base layer, that can be derived using the sub-bitstream extraction process as specified in Rec. ITUT H.264 | ISO/IEC 14496-10 subclause G.8.8.1 with any value for temporal_id as the input shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Rec. ITUT H.264 | ISO/IEC 14496-10 Annexes A, G and H.

scalability_mask_flag [ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table XX are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table XX. Mapping of ScalabiltyId to scalability dimensions

| scalability_mask_flag index | Scalability dimension | ScalabiltyId mapping |
| --- | --- | --- |
| 0 | spatial | DependencyId |
| 1 | quality | QualityId |
| 2 | depth | DepthFlag |
| 3 | multiview | ViewId |
| 4 - 15 | Reserved | Reserved |

dimension_id_len_minus1[ j ] plus1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

vps_nuh_layer_id_present_flag specifies whether the layer_id_in_nuh[ i ] syntax is present.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. When not present, the value of layer_id_in_nuh[ i ] is inferred to be equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

The following applies:

for( i = 0; i <= vps_max_layers_minus1; i++ )

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  !=  0 )  &&  scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

The following applies:

- For each layer indexed by i = 0..vps_max_layers_minus1 + 1

- DependencyId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

- QualityId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ]

- DepthFlag[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][2 ]

- ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 3 ]

num_direct_ref_layers[ i ] specifies the number of layers the i-th layer directly references. [Ed.(YK): Add the exact meaning of a layer directly referencing another layer.]

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly references.

# Patent rights declaration(s)

Vidyo may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).