| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-1103 |
| --- | --- |

| Title: | Description of Core Experiment SCE 3: Combined Inter and Inter-Layer Prediction in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | X. Li E. François P. Lai D. Kwon A. Saxena | Tel: Email: | lxiang@qti.qualcomm.com edouard.francois@crf.canon.fr Polin.Lai@mediatek.com d-kwon@ti.com asaxena@sta.samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of Core Experiment 3 on Combined Inter and Inter-layer Prediction in SHVC.

# Introduction

This document defines Core Experiment 3 (CE3) on combined prediction in SHVC, to be performed for the 13th JCT-VC meeting. The CE consists of proposals which use combined base layer and motion compensated enhancement layer information to predict enhancement layer. Intra coding is not addressed in this CE.

# Participants

| Company | Name & Email |
| --- | --- |
| Canon | E. François (edouard.francois@crf.canon.fr), C. Gisquet (Christophe.gisquet@crf.canon.fr), G. Laroche (guillaume.laroche@crf.canon.fr), P. Onno (patrice.onno@crf.canon.fr) |
| Qualcomm | X. Li (lxiang@qualcomm.com), V. Seregin (vseregin@qti.qualcomm.com), K. Rapaka (krapaka@qti.qualcomm.com), J. Chen (cjianle@qti.qualcomm.com) |
| MediaTek | P. Lai (Polin.Lai@mediatek.com), T.-D. Chuang (peter.chuang@mediatek.com), Y.-W. Huang (yuwen.huang@mediatek.com), S. Liu (shan.liu@mediatek.com), S.-T. Hsiang (shih-ta.hsiang@mediatek.com), X. Zhang (Ximin.zhang@mediatek.com), Z. Chen (Zhenzhong.chen@mediatek.com) |
| TI | D. Kwon (d-kwon@ti.com) M. Budagavi (madhukar@ti.com) |
| Samsung | A. Saxena (asaxena@sta.samsung.com), E. Alshina (elena_a.alshina@samsung.com), T. Lee (tammy.lee@samsung.com); C. Kim (dionism@samsung.com), A.Alshin (alexander_b.alshin@samsung.com) |
| Sharp | T. Yamamoto (yamamoto.tomoyuki@sharp.co.jp) |
| Intel | Y.-J. Chiu (yi-jen.chiu@intel.com), W. Zhang (wenhao.zhang@intel.com), X. Cai (xiaoxia.cai@intel.com) |
| Nokia | J.Lainema (Jani.Lainema@nokia.com), K. Ugur (kemal.ugur@nokia.com), A. Aminlou (ext-alireza.aminlou@nokia.com) |
| LG | J. Park (jy.park@lge.com) |
| Vidyo | W. Jang (wonkapjang@gmail.com) |
| ETRI | Jung Won Kang (jungwon@etri.re.kr), Jiho Lee (jinosoul@etri.re.kr), Hahyun Lee (hanilee@etri.re.kr) |
| Gachon University | Woo-Jin Han (hurumi@gmail.com) |

# Crosschecking assignments

| Tests | Technique | Proponent | Cross-checkers |
| --- | --- | --- | --- |
| 3.1 | Adaptive predictor compensation | T.-D. Chuang (peter.chuang@mediatek.com) |  |
| 3.2 | Combined inter mode | V. Seregin (vseregin@qti.qualcomm.com) |  |
| 3.3 | Generalized residual prediction | X. Li (lxiang@qti.qualcomm.com), J.Lainema (Jani.Lainema@nokia.com) E. François (edouard.francois@crf.canon.fr) | W. Zhang (wenhao.zhang@intel.com) E. Alshina (elena_a.alshina@samsung.com) |
| 3.4 | Generalized combined prediction | T.-D. Chuang (peter.chuang@mediatek.com) |  |
| 3.5 | Generalized residual prediction with MC at BL | E. François (edouard.francois@crf.canon.fr) |  |
| 3.6 | Generalized residual prediction with shorter MC filter | T. Tsukuba (tsukuba.takeshi@sharp.co.jp) | K. Sato (Kazushi.Sato@jp.sony.com) |
| 3.7 | Difference domain inter prediction | J. Park (jy.park@lge.com) |  |
| 3.8 | Difference domain inter prediction | Wonkap Jang (wonkap@vidyo.com) |  |
| 3.9 | RefIdx based differential coding | J.Lainema (Jani.Lainema@nokia.com) | E. François (edouard.francois@crf.canon.fr) X. Li (lxiang@qti.qualcomm.com) |
| 3.10 | Base Mode with Residual Prediction | E. François (edouard.francois@crf.canon.fr) |  |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The SHVC common test conditions defined in JCTVC-L1109 shall be used for this CE.

The CE shall test following cases.

|  | RA | LD-P |
| --- | --- | --- |
| Scalability | 2x, 1.5x, and SNR | 2x, 1.5x, and SNR |

Since the CE only addresses changes to inter pictures, only RA and LD-P tests are required.

  - Software

Reference software SM-1.0 will be used as the anchor and base software in this CE. The anchor depends of the SM1.0 framework (CU-level or Ref-idx) used by the proponent. The proponent must use as anchor the SM1.0 framework in which his proposal is implemented.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, and complexity. Complexity assessment is defined in 4.3.1.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed either with or without output of decoded YUV data for both the Anchors and the Test. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

To evaluate memory bandwidth requirement and computational complexity related to up-sampling and MC interpolation filter, methodologies defined in JCTVC-L0440 shall be used.

# Tools to test

## Mixed Inter-Inter-layer Prediction

### Test 3.1 Adaptive predictor compensation by using reconstructed BL texture (JCTVC-0072, MediaTek)

JCTVC-L0072 proposes an adaptive predictor compensation (APC) that uses reconstructed base layer (BL) texture to refine enhancement layer (EL) sample predictors. It only applied on inter-coded CU. A CU-level enabling flag is signaled for APC. If APC is applied, the EL predictor will be as follow.

PredEL’ = (PredEL + RecBL_cur) >> 1

The complexity and bandwidth requirement data will be provided. The results (include BD-rate, complexity, and bandwidth requirement) of combining general residual prediction will be provided as well. The different weighting for EL predictor and BL texture will be investigated.

### Test 3.2 Combined Inter Mode (JCTVC-L0206, Qualcomm)

The combined prediction mode is applied to inter predicted CUs. The prediction in this mode is defined as an average of two predictions which are done in intra-BL mode and enhancement layer inter prediction mode. Let Pb denote intra-BL prediction and Pe denote enhancement layer inter prediction. The final prediction P for the CU is defined as

P=(Pe+Pb+1)/2

This method is applied only to the luma component. The enhancement layer inter prediction is used for chroma components when the combined prediction mode is enabled. The combined_mode_flag is used to indicate the usage of the mode at CU level.

Bandwidth requirement and complexity of the method will be studied, some parameters can be adjusted to address the complexity issues.

## Residual Prediction

### Test 3.3 Generalized Residual Prediction (JCTVC-L0078, JCTVC-L0190, JCTVC-L0038, JCTVC-L0100)

In this technique, inter predicted residue of the collocated BL block is used to predict that of the current block. This method is applied to inter CUs, and skip mode CUs. The framework of this method is shown in Figure 1 where the case of uni-prediction is illustrated.

Let Be and Bb denote the current block in the EL picture and its collocated BL block, respectively. Let Pe0 denote the temporal prediction for the block Be obtained by using the motion vector MVe0 (where sub-index 0 refers to reference list 0). Similarly, Pb0 represents the temporal prediction for the block Bb obtained by using the same motion vector MVe0 in the BL reference picture.

Then, the inter predicted residue of the BL block Rb0 is obtained as

Rb0=Bb-Pb0,

Considering the temporal prediction Pe0 for block Be, the final uni-prediction P for block Be is

P=Pe0+w⋅Bb-Pb0,

where w is a the weighting factor, which takes the values 0, 0.5, or 1.

When extending this method to the bi-predictional case, the final prediction P for Be is formed as

P=(Pe0+w⋅Bb-Pb0 + Pe1+w⋅Bb-Pb1+1)/2,

where sub-index 1 indicates reference list 1.

Moreover, another weighting mode can be applied for uni-prediction as follows

P=Bb+0.5⋅Pe0-Pb0.

When extending to bi-predictional case, the final prediction P for Be is formed as

P=Bb+((w⋅Pe0-Pb0+w⋅Pe1-Pb1+1)/2

To distinguish the two weighting 0.5, the first one is called res-0.5 while the second is diff-0.5. The weighting factor w is signaled at CU level as a weighting index. The weighting index 0, 1, 2 and 3 are used to indicate the weighting factors 0, res-0.5, 1 and diff-0.5, respectively. A truncated unary code in regular CABAC mode is used for the signaling.

In addition, approaches to reduce memory bandwidth, such as using shorter MC interpolation filter like bilinear filter, using combined up-sampling MC interpolation filter, constraining block size in GRP mode, constraining GRP to Uni-Prediction, may also apply.

Figure 1 Inter residual prediction for uni-prediction

### Test 3.4 Generalized combined prediction (JCTVC-L0074, MediaTek)

JCTVC-L0074 provides the combined results of combining adaptive predictor compensation (APC) and generalized residual prediction (GRP). In this test, the generalized combined prediction (GCP) is evaluated. A CU-level enabling flag is signaled for GCP. If GCP is applied, the EL predictor will be as follow.

PredEL’= (1-a) * PredEL + a* RecBL_cur + b * (RecBL_cur – RecBL_ref)

Results with fixed factors a and b will be evaluated. In addition, results with CU level signaling of weighting factors will be provided. The complexity and bandwidth requirement data will also be provided.

### Test 3.5 Generalized Residual Prediction with MC at BL resolution (JCTVC-L0104, Canon)

JCTVC-L0104 proposes a simplification of the Generalized Residual Prediction which consists in directly accessing the temporal reference BL data instead of the upsampled temporal reference BL data to generate the second order residual, as illustrated in the next figure.

The simplification will also be evaluated in the Diff Inter mode implementation of GRP. Combinations with Test 3.3 on the limitations of block sizes, Prediction types will be also evaluated.

### Test 3.6 Generalized Residual Prediction with shorter-tap MC filter (JCTVC-L0265, Sharp)

Generalized Residual Prediction, such as JCTVC-L0038, JCTVC-L0100 or JCTVC-L0078, is simplified by applying shorter-tap MC filter to create motion-compensated up-sampled base-layer reconstruction. As the shorter-tap MC filter for luma, bilinear filter is used for B_SLICE and DCT-IF based 4-tap filter is used for P_SLICE. As the shorter-tap MC filter for chroma, bilinear filter is used irrespective of slice type.

## Diff inter prediction

### Test 3.7 Inter prediction based on difference picture (JCTVC-0136, LG)

JCTVC-0136 proposes inter coding in differential domain. When inter difference mode is applied, the reconstruction samples can be represented as below.

Rec = Up(Rec_BL) + MC(Diff, MV) + Residual, where Diff = Ref_EL-Up(Ref_BL)

In the proposed method, prediction and reconstruction values in D-domain are clipped to keep the range the same as the normal pixel values. The weight for difference signal is introduced in the crosscheck document (JCTVC-L0081) of JCTVC-L0136. The difference weighting method for difference signal will also be investigated. The complexity and bandwidth requirement data will be provided.

### Test 3.8 Inter prediction based on difference picture (JCTVC-L0184, Vidyo)

JCTVC-L0184 provided results for inter prediction based on difference coding. In the contribution, a CU-based signalling was used, and a separate upsampling filter was applied for difference-picture creation. We plan to test combining the upsampling filter, using weights for the difference prediction, turning off extra ME to lower complexity, and doing difference prediction on the fly. The complexity and bandwidth data will be provided for a subset of the above mentioned tests.

### Test 3.9 RefIdx based differential coding (JCTVC-L0213, Nokia)

JCTVC-L0213 proposes a differential coding approach based on RefIdx signaling. The method creates new reference pictures using a differential process, includes those together with a base reference to reference picture lists and applies HEVC version 1 weighted prediction to perform motion compensated prediction utilizing the new reference pictures. Picture below illustrates the process.

This test will evaluate coding performance and complexity of the approach proposed in JCTVC-L0213. In addition, results for different configurations based on different weighting and placement of reference pictures in the reference picture lists may be provided.

## Base mode with residual prediction

### Test 3.10 Base Mode with Residual Prediction (JCTVC-L0103, Canon)

JCTVC-L0103 proposes a Base Mode, applied at the CU-level and consisting in an 8x8-based Base Layer syntax inheriting, combined with Generalized Residual Prediction. The MC filters used for BL and EL motion compensation are bilinear.

Test 3.4 will be based on this design and will evaluate the usage of Generalized Residual prediction combined with the Base Mode. Evaluation of using directly the decoded Base Layer residual in place of the GRP residual will also be made.

Combinations with Test 3.2 on the limitations to Uni-Prediction will be also evaluated.

# Time-line

2013-Feb-06:	CE-3 description finalized and uploaded. Release of the Excel template for Complexity evaluation

2013-Feb-06:	Release of reference software SM1.0 from AHG ‘SHVC software’

2013-Feb-13:	Release of reference software SM1.0 with complexity assessment modules (from Samsung/InterDigital)

2013-Feb-27:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators

2013-Mar-06:	Proponents provide complexity analysis results to cross-verifiers(s) and CE coordinators

2013-Mar-27:	Cross-verifiers report results to CE participants

2013-xxx -x:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-xxx -x:	Summary report uploaded

# References

- E. François, A. Tabatabai, E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, Switzerland, 14–23 Jan. 2013