| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0058_r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0035 |
| --- | --- |

| Title: | On profile, tier and level information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Tomoyuki Yamamoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to 1) remove profile_ref_minus1, 2) add inference rules for sub-layer profile and level information and 3) byte alignment for profile_tier_level.

It is asserted that the proposal 1 to 3 have the following benefit.

- Reduce bit amount of profile_ref_minus1[ i ] (and easy to byte-aligned).

- Avoid confusing undefined value handling of profile_tier_level() especially when it is refered by profile_level_tier_idx[ i ].

- Easy to decode profile_tier_level( ) of layer set, which is important when negotiation. (e.g. the general profile or general level is not decodable but some layer set can be decodable).

Introduction

The profile_tier_level( ) for layer sets is decoded as follows:

- Decode a list of profile_tier_level( ) with vps_profile_present_flag, profile_ref_minus1.

- Select decoded profile_tier_level( ) by profile_level_tier_idx[i].

However, there are several issues, called Issue A to Issue C in this contribution, on the current design.

- profile_ref_minus1 constantly uses 6 bits. However, it is not so much useful because, in many cases, the information is inferred from the previous one. Note that encoder can arrange elements of a list of profile_tier_level( ).

- profile_tier_level( ), which is referred by profile_level_tier_idx[ i ], can be not present. It is very confusing that there are unknown values that can be referred.

- profile_tier_level( ) in vps_extension is not byte-aligned.

Proposal

Our proposals are summarised as follows;

- To solve issue A, remove profile_ref_minus1.

- To solve issue B, add the following inference rules:
1) When the level information is not present for the i-th sub-layer, it is inferred to be equal to the (i+1)-th sub-layer’s.
2) When vps_profile_present_flag[i] of i-th profile_tier_level( ) is equal to 1 and the profile information is not present for the i-th sub-layer, it is inferred to be equal to (i+1)-th sub-layer’s.

- To solve issue C, the following applies: 
1) Add byte-alignment bits before profile_tier_level( ) or extend bit-length of profile_ref_minus1[i] from 6 bits to 8 bits without proposal a).

2) Add a syntax vps_ptr_info_offset indicating byte-offset from starting of VPS NAL to the syntax(vps_num_layer_sets_minus1) related to profile_tier_level( ) or move syntaxes related to profile_tier_level( ) after scalability_mask_flag.

For more details, please see the Text change in section 4.

Conclusion

This contribution proposes to 1) remove profile_ref_minus1, 2) add inference rules for sub-layer profile and level information and 3) byte alignment for profile_tier_level.

It is asserted that the proposal 1 to 3 have the following benefit.

- Reduce bit amount of profile_ref_minus1[ i ] (and easy to byte-aligned).

- Avoid confusing undefined value handling of profile_tier_level() especially when it is refered by profile_level_tier_idx[ i ].

- Easy to decode profile_tier_level( ) of layer set, which is important when negotiation. (e.g. the general profile or general level is not decodable but some layer set can be decodable).

It is recommended to adopt proposals to SHVC and MV-HEVC.

Text changes

Solution regarding issue A

- Remove profile_ref_minus1.

Changes are highlighted in yellow, and both of removals and typos are stroke through in red.

F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| …snipped… |  |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| …snipped… |  |
| } |  |

F.7.4.3.1.1 Video parameter set extension semantics

vps_profile_present_flag[ i ] equal to 1 specifies that the profile and tier information for layer set i is present in the i-th profile_tier_level( ) syntax structure. vps_profile_present_flag[ i ] equal to 0 specifies that profile and tier information is not present in the i-th profile_tier_level( ) syntax structure and is inferred to be equal to the profile and tier information of the ( i-1 )-th profile_tier_level( ) syntax structure. When the index i is equal to 1, vps_profile_present_flag[i] shall be equal to 1.

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile_ref_minus1[ i ] + 1)-th layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than i.

F.7.4.4 Profile, tier and level semantics

The profile_tier_level( ) syntax structure provides profile, tier and level information used for a layer set. The profile_tier_level( ) syntax structure, specified by profile_level_tier_idx[ i ] for the i–th layer set with i larger than 0, provides profile, tier and level information used for a layer set specified by the index i. When the profile_tier_level( ) syntax structure is included in a vps_extension( ) syntax structure, the applicable layer set to which the profile_tier_level( ) syntax structure applies is specified by the corresponding lsIdx variable in the vps_extension( ) syntax structure. When the profile_tier_level( ) syntax structure is included in a VPS, but not in a vps_extension( ) syntax structure, the applicable layer set to which the profile_tier_level( ) syntax structure applies is the layer set specified by the index 0. When the profile_tier_level( ) syntax structure is included in an SPS, the layer set to which the profile_tier_level( ) syntax structure applies is the layer set specified by the index 0.

For interpretation of the following semantics, CVS refers to the CVS subset associated with the layer set to which the profile_tier_level( ) syntax structure applies.

When vps_profile_present_flag[ i ] is equal to 0, When the syntax elements general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_reserved_zero_44bits are not present for the applicable layer set in the i-th profile_tier_level( ) syntax structure, they are inferred to be equal to the corresponding values of the layer set prior profile_tier_level( ) syntax structure specified by the index ( profile_layer_set_ref_minus1[ lsIdx ] +1 )  (i-1), where the value of i shall be in the range of 1 to (vps_num_profile_tier_level_minus1+1), inclusive.

When vps_profile_present_flag[ i ] is equal to 0, the syntax elements sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_source_flag[ i ], sub_layer_interlaced_source_flag[ i ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ i ], sub_layer_reserved_zero_44bits[ i ] are not present for the applicable layer set in the i-th profile_tier_level( ) syntax structure, and they are present in or inferred for the layer set specified by the index ( profile_layer_set_ref_minus1[ lsIdx ] +1 ), they are inferred to be equal to the corresponding values of the layer set prior profile_tier_level( ) syntax structure specified by the index ( profile_layer_set_ref_minus1[ lsIdx ] +1 ) (i-1), where the value of i shall be in the range of 1 to (vps_num_profile_tier_level_minus1+1), inclusive.

Solution regarding issue B

In addition to the above changes of issue A, the following changes are applied;

- Add inference rule which indicates that, when the level information is not present for the i-th sub-layer, it is inferred to be equal to the (i+1)-th sub-layer’s or general level information.

- Add inference rule which indicates that, when the profile information is not present for the i-th sub-layer, it is inferred to be equal to the (i+1)-th sub-layer’s or general profile information.

Changes are highlighted in yellow and removals are stroke through in red.

F.7.4.4 Profile, tier and level semantics

// inference rule for sub-layer level information

When sub_layer_level_present_flag[ i ] is equal to 0, sub_layer_level_idc[ i ] is inferred by applying the follows:

for ( i = maxNumSubLayersMinus1; i >= 0; i-- ) {

if (!sub_layer_level_present_flag[ i ])

sub_layer_level_idc[ i ] = (i == maxNumSubLayersMinus1) ? general_level_idc: sub_layer_level_idc[ i+1 ]

}

}

// inference rule for sub-layer profile information

When vps_profile_present_flag[ i ] is equal to 1 and sub_layer_profile_present_flag[ j ] is equal to 0 and the index j is less than vps_max_sub_layers_minus1, sub_layer_profile_space[ j ], sub_layer_tier_flag[ j ], sub_layer_profile_idc[ j ], sub_layer_profile_compatibility_flag[ j ][ k ], sub_layer_progressive_source_flag[ j ], sub_layer_interlaced_source_flag[ j ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ j ], and sub_layer_reserved_zero_44bits[ j ] are infered to be equal to the corresponding value of the (j+1)-th sub-layer.

When vps_profile_present_flag[ i ] is equal to 1 and sub_layer_profile_present_flag[ j ] is equal to 0 and the index j is equal to vps_max_sub_layers_minus1, sub_layer_profile_space[ j ], sub_layer_tier_flag[ j ], sub_layer_profile_idc[ j ], sub_layer_profile_compatibility_flag[ j ][ k ], sub_layer_progressive_source_flag[ j ], sub_layer_interlaced_source_flag[ j ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ j ], and sub_layer_reserved_zero_44bits[ j ] are infered to be equal to the corresponding value of general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_reserved_zero_44bits, and general_level_idc, respectively

Solution regarding issue C

The following changes are applied:

- Replace the syntax elements (e.x splitting_flag to vps_vui_present_flag).

- Option A) add a syntax vps_ptr_info_offset indicating byte-offset from starting of VPS NAL to the syntax(vps_num_layer_sets_minus1) related to profile_tier_level( )

- Option B) move syntaxes related to profile_tier_level( ) after scalability_mask_flag.

The following changes are also applied;

- Option1) Add byte-alignment bits between those syntaxes, with signalling vps_profile_present_flag[i] and profile_tier_level( ) in vps_extension( ) separately

- Option2) Extend bit-length of profile_ref_minus1[i] from 6 bits to 8 bits

Changes are highlighted in cyan and yellow and removals are stroke through in red.

F.7.4.3.1.1 Video parameter set extension semantics

vps_ reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for vps_ reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Although the value of vps_ reserved_zero_6bits is required to be equal to 0 in this version of this Specification, decoders shall allow other values of vps_ reserved_zero_6bits to appear in the syntax.

vps_ptr_info_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the set of fixed-length coded information starting from vps_number_layer_sets_minus1, in the VPS NAL unit. Emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

| vps_extension( ) { // Option A | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| vps_nuh_layer_id_present_flag | u(1) |
| max_tid_ref_present_flag | u(1) |
| all_ref_layers_active_flag | u(1) |
| rep_format_idx_present_flag | u(1) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | Byte-aligned u(1) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| vps_reserved_6bits | u(6) |
| vps_vui_offset | u(16) |
| vps_ptr_info_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| … |  |
| // option 1) |  |
| while(!byte_aligned() ) |  |
| vps_aliginment_bit_equal_to_one | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| } |  |
| for( i = ((8-(vps_num_profile_tier_level_minus1 + 1)%8)%8); i >0 ;i-- ) { |  |
| vps_reseved_zero_bit[ i ] | u(1) |
| } | Byte-aligned |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| // option 2) |  |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) u(8) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

| vps_extension( ) { // Option B | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| vps_nuh_layer_id_present_flag | u(1) |
| max_tid_ref_present_flag | u(1) |
| all_ref_layers_active_flag | u(1) |
| rep_format_idx_present_flag | u(1) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| vps_reserved_6bits | u(6) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| vps_number_layer_sets_minus1 | Byte-aligned u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| // option 1) |  |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| } |  |
| for( i = ((8-(vps_num_profile_tier_level_minus1 + 1)%8)%8); i >0 ;i-- ) { |  |
| vps_reseved_zero_bit[ i ] | u(1) |
| } |  |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| // option 2) |  |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) u(8) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| … // move syntaxes related to profile_tier_level() to top part in vps_extension() |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

// option 2)

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile _ref_minus1[ i ] + 1)-th profile_tier_level( ) syntax structure layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than i.

Reference

- Y. Wang, et.al, “AHG9: On VPS and SPS in HEVC 3DV and scalable extensions”, JCTVC-M0268, Incheon, KR, 18–26 Apr. 2013.

- J. Chen, et.al, “SHVC Draft 3”, JCTVC-N1008, Vienna, AT, 25 July–2 Aug. 2013.

- G. Tech, et.al, “MV-HEVC Draft Text 5”, JCT3V-E1004, Vienna, AT, 27 July–2 Aug. 2013.

Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).