| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0181_r1 |
| --- | --- |

| Title: | Non-RCE3: Implicit derivation for adaptively turning filtering off in intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jewon Kang, Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8457 jewonk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes implicit derivation methods for adaptively turning off a set of filters used for intra prediction. These include the interpolation filter, intra smoothing filters and the horizontal and vertical gradient filters. In the proposed schemes, the decision is made based solely on the transform_skip_flag for a TU or in conjunction with the reference samples used for intra prediction. Simulation results reportedly show that the proposed scheme achieves coding gains in YUV and RGB screen content sequence classes and incurs no loss in natural videos and RExt sequences.

# Introduction

Figure 1 shows the reference samples used for intra prediction for a 4×4 block. Mode-dependent intra smoothing and bi-linear filtering may be used to filter the reference samples depending on the block size and intra prediction mode. In addition, gradient filtering may be used for horizontal and vertical intra modes and DC prediction filtering may be used for DC mode. Similarly bi-linear interpolation is used in angular intra prediction process.

Screen content and non-camera generated videos may contain synthetic contents with sharp edges along object boundaries and characters. For such content, different kinds of filtering described above may be undesirable. In [1], it was proposed to use nearest neighbor interpolation instead of bi-linear interpolation for angular prediction for certain angles.

We propose a method to adaptively disable some of the filtering used in the intra prediction process using an implicit criterion. The criterion is based on the reference samples and/or the transform_skip_flag for a TU. Experimental results are presented to show that the implicit method provides BD-rate gains for F, SC YUV, SC RGB classes without incurring losses for other classes.

Figure 1: Reference samples of a 4×4 block for HEVC intra prediction

# Proposed method

The proposed derivation of the disable filtering flag to switch off filters in the intra prediction process is provided in subsection 2.1. If the disable filtering flag is 0, the filtering as in the current HEVC Range extension specification is performed.

## Implicit derivation of a disabling filter flag

## Method 1

- If the transform_skip_flag is 0, the disable filtering flag is set to 0, and the derivation process is terminated. Otherwise (if the transform_skip_flag is 1), the following applies:

- Apply a [1, -2, 1] FIR filter to the reference samples from the bottom-left position to the top-right position (i.e., along the red path) in Figure 1.

- If a filtered value greater than a threshold is found, the disabling flag is set to 1, and the derivation process is terminated. The threshold is set to (1<<(bitdepth-2)+ 1<<(bitdepth-4))

- Otherwise the disable filtering flag is set to 0.

Method 2

- If the transform_skip_flag is 0, the disable filtering flag is set to 0, and the derivation process is terminated.

- If the transform_skip_flag is 0, the disable filtering flag is set to 0, and the derivation process is terminated.

- Otherwise, the disable filtering flag is set to 1.

The disable filtering flag controls MDIS, bi-linear interpolation, and gradient filtering applied to horizontal and vertical intra prediction modes. When bi-linear interpolation is disabled, nearest neighbor interpolation as described in [1] is used.

# Experimental results

Experimental results for the proposed methods are presented in this section with lossy and lossless configuration under AHG8 test sequences and conditions. The anchor is AHG8 anchor based on HM12.0RExt-4.1 software.

## Method 1: Using transform_skip_flag + FIR filtering for implicit derivation (4x4 block only)

Table 1: Lossy coding results for Method 1 applied to blocks of size 4×4. Anchor is HM12.0+RExt4.1.

## Method 2: Using only transform_skip_flag (4x4 blocks only)

Table 2: Lossy coding results for Method 2 applied to blocks of size 4×4. Anchor is HM12.0+RExt4.1.

## Lossless coding

In the results of this subsection, the disable filter flag for method 1 is derived using the same derivation procedure in Section 2.1, and applied to a 4x4 TU. Instead of the transform_skip_flag, the cu_transquant_bypass_flag is used. For lossless coding for all blocks, the cu_transquant_bypass_flag is 1. Thus, Table 3 shows the results when only the FIR filter is used as the decision criteria. We show results for the “All Intra” configuration. More details can be found in the accompanying excel files. Method 2 is not applied in the lossless case. This is because for lossless coding, the cu_transquant_bypass_flag is 1 for all the blocks. Thus it can not be used as a criterion for selection of nearest neighbor interpolation or bilinear interpolation.

Table 3: Lossless coding results for Method 1 applied to blocks of size 4x4. Anchor is HM12.0+RExt4.1.

# Conclusion

This contribution proposes implicit derivation methods for adaptively turning off a set of filters used for intra prediction. These include the interpolation filter, intra smoothing filters and the horizontal and vertical gradient filters. In the proposed schemes, the decision is made based solely on the transform_skip_flag for the TU or in conjunction with reference samples used for intra prediction. Simulation results show that the proposed scheme achieves a coding gain in YUV and RGB screen content sequence classes and incurs no loss for natural videos and RExt sequences.

# Working Draft text

# References

- H. Chen, A. Saxena, F. Fernandes, “Non-RCE2: Enhanced angular intra prediction for screen content coding,” JCTVC-N0183, Vienna, Austria, Aug. 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).