- 7.3.8.8 Palette syntax

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if( !palette_share_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } |  |
| if( currentPaletteSize != 0 ) |  |
| { |  |
| palette_escape_transpose_flag | ae(v) |
| } |  |
| if( palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_palette_abs | ae(v) |
| if( cu_qp_delta_palette_abs ) |  |
| cu_qp_delta_palette_sign_flag | ae(v) |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_palette_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_palette_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

palette_escape_transpose_flag equal to 0 specifies that palette_escape_val_present_flag = 0 and palette_transpose_flag = 0. If it equals to 1, it specifies that palette_escape_val_present_flag = 0 and palette_transpose_flag = 1. If it equals to 2, it specifies that palette_escape_val_present_flag = 1 and palette_transpose_flag = 0. If it equals to 3, it specifies that palette_escape_val_present_flag = 1 and palette_transpose_flag = 1.

| Table 9-38 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| palette_coding( ) | palette_share_flag | FL | cMax = 1 |
|  | palette_predictor_run | EG0 | - |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |
|  | palette_entry | FL | cMax = cIdx = = 0 ? ( (1<<BitDepthY) − 1 ) : ( (1<<BitDepthC) − 1 ) |
|  | palette_escape_transpose _flag | TR | cMax = 3 |
|  | cu_qp_delta_abs | 9.3.3.11 | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | cu_chroma_qp_offset_flag | FL | cMax = 1 |
|  | cu_chroma_qp_offset_idx | TR | cMax = chroma_qp_offset_list_len_minus1, cRiceParam = 0 |
|  | palette_run_type_flag | FL | cMax = 1 |
|  | palette_index_idc | TB | cMax = adjustedIndexMax |
|  | palette_run_msb_id_plus1 | TR | cMax = Floor( Log2((MaxPaletteRun ) ) + 1, cRiceParam = 0 |
|  | palette_run_refinement_bits | TB | cMax = ((1<< palette_run _msb_id_plus1) >= MaxPaletteRun)? MaxPaletteRun - (1 <<( palette_run_msb_id_plus1-1)) : (1<< (palette_run _msb_id_plus1 – 1)) – 1 |
|  | palette_escape_val | 9.3.3.12 | cIdx, qP |

| Table 9-43 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| pred_mode_flag | 0 | na | na | na | na | na |
| palette_mode_flag | 0 | na | na | na | na | na |
| palette_share_flag | 0 | na | na | na | na | na |
| palette_predictor_run | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_entry | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_escape_transpose _flag | bypass | bypass | bypass | na | na | na |
| cu_qp_delta_palette_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| … |  |  |  |  |  |  |

      - Binarization process for palette_escape_traverse_flag

Input to this process is a request for a binarization for the syntax element palette_escape_traverse_flag.

Output to this process is the binarization of palette_escape_traverse_flag.

The binarization for the syntax element palette_escape_traverse_flag is specified in Table 9-xx.

Table 9-42 – Binarization for inter_pred_idc

| Value of palette_escape_traverse_flag | Bin string |  |
| --- | --- | --- |
|  | currentPaletteSize = 1 | currentPaletteSize > 1 |
| 0 | 1 or 00 | 1 |
| 1 | 01 | 01 |
| 2 | n/a | 001 |
| 3 | n/a | 000 |