VIF Extension of HDRTools Package

Release Notes

- Visual Information Fidelity (VIF) Extension 0.1

VIF extension 0.1 has been implemented in HDRTools package which computes the VIF value of two input frames (namely original frame and the modified (or distorted) frame).

- What's New for VIF Extension 0.1

## New Features

    - A new define preprocessor macro called ‘VIF_SUPPORT’ is added to the HDRTools package. In order to enable the VIF feature of the code, this macro is set to one in the software.

    - A variable (VIFBitDepthY) is added to the configuration which needs to be set the same as the bit depth of the input signals. This variable is added for the function rgb_pq_yuv in DistortionMetricVIF.cpp in Common part. The VIFBitDepthY variable is also added to the ProjectParameters and Frame classes.

    - Two types of steerable pyramid filters [1, 3] are added including sp1filters and sp5filters (Please see DistortionMetricVIF.h). For each of them a define macro is introduced. The default version of software uses sp5filters for filtering and downsampling.

  - New Functions

    - New functions have been added for VIF computation (Please see DistortionMetricVIF.h and DistortionMetricVIF.cpp).

- Limitations

The current version of VIF extension of HDRTools supports only OpenEXR files of type BT.709 and BT. 2020.