| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 40th Meeting: by teleconference, 24 June – 1 July 2020 | Document: JCTVC-AN0024 |
| --- | --- |

| Title: | Some HEVC and AVC errata items |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports some errata items for HEVC and AVC.

# HEVC, for both the ITU-T and the ISO texts:

- Semantics of the deblocking disable flag in the PPS should be updated to better reflect the behavior.

- Corresponding to the following changes to VVC, an errata report for AVC and HEVC, to change the MinCr limit to be derived from the CPB size limit or add a note for cases where the CPB size imposes a tighter limit than the MinCr does:

MaxCPB = 80 000 for level 6, 120 000 for level 6.1, 180 000 for level 6.2, and change MinCrScaleFactor for the 4:4:4 profile to 0.75, and change MinCrBase to 8 for level 6.2.

- The following two bullet items on non-scalable-nested BP/PT/DUI SEI messages should be changed as shown below:

–	For a non-scalable-nested SEI message, when payloadType is equal to 0 (buffering period) or 130 (decoding unit information), the non-scalable-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdList containing all values of nuh_layer_id in all VCL units in the bitstream, and has only the base layer as the output layer.

–	For a non-scalable-nested SEI message, when payloadType is equal to 1 (picture timing), the frame field information carried in the syntax elements pic_struct, source_scan_type and duplicate_flag, when present, in the non-scalable-nested picture timing SEI message applies to the base layer only, while the picture timing information carried in other syntax elements, when present, in the non-scalable-nested picture timing SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdList containing all values of nuh_layer_id in all VCL units in the bitstream, and has only the base layer as the output layer.

# HEVC and AVC

All those mentioned for HEVC and 	AVC, in JVET-S0237-v9, and JVET-S0156, and later possibly mre in the JVET-S main meeting minutes.

Add "The variable DuHrdPreferredFlag is either specified by external means, or when not specified by external means, set equal to 0." to clause 8.1.2.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).