| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0022 |
| --- | --- |

| Title: | Proposed improvements to the Adaptive multiple Core transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | V. Lorcy (bcom) P. Philippe (Orange) | Tel: Email: | +33299124061 victorien.lorcy@b-com.com pierrick.philippe@orange.com |
| Source: | Orange, b<>com |  |  |

_____________________________

# Abstract

This contribution proposes a change in the Adaptive Multiple Core transform tool in the current JEM2 design. The changes are limited to the introduction of two additional core transforms and corresponding tables.

It is asserted that these changes bring -0.37% and -0.23% Y-BDrate improvement for AI and RA configurations respectively with a limited impact on the encoding/decoding complexity.

# Introduction

The JEM2 design currently uses 5 types of trigonometric transforms [1] and their inverse. The core transforms are:

- DCT-II, DCT-V and DCT-VIII

- DST-I and DST-VII

These transforms are arranged by pair (vertical and horizontal transform) and per prediction modes in the JEM software. Five pairs are pre-determined for each prediction mode.

The encoder selects among five pairs of pre-determined transforms the best performing one in the RDO sense; the best pair is signalled to the decoder.

The first pair is always the combination of a DCT-II both for the horizontal and vertical transforms. If the RDO of this arrangement is satisfactory for all the TUs in the CU under consideration, it is used by default. Otherwise a TU level indicator, conveyed on two bits, signals the best pair for each TU in the CU.

In this contribution, this general design is kept and two additional core transforms are proposed to improve the JEM2 compression efficiency. The number of core transform possibilities is increased but the number of pairs per prediction mode is kept unchanged. Consequently, the signalling mechanism and the encoder selection process are kept unchanged.

# Additional core transforms

Two transforms are proposed along with the legacy transforms: the DST-IV and the ID transform.

The ID is the identity transform and is implemented as a scaling of the input coefficients, therefore it implies no addition / multiplication. It is associated with any trigonometric core transform to handle residuals that exhibit banded patterns. This transform mostly used for 4x4 and 8x8 TUs.

HEVC residuals have a non-uniform energy distribution among residual pixels: the energy of the residual tends to increase from left to right and from top to bottom within a block. The DST-VII was introduced in HEVC to accommodate such profiles, it was also demonstrated [2] that this transform is optimal for signal having tri-diagonal autocorrelation functions. The DST-IV transform is similar with that respect and is able to decorrelate intra-predicted like signals. The transform is comparable to the DST-VII as displayed hereunder but exhibits a second border. This can be noticed by comparing the second and the fifth vector of the DST-IV basis to the DST-VII).

Figure 1. DST-VII basis left, DST-IV basis right.

The DST-IV core transform was already suggested during the HEVC development [3], and motivated by the existence of fast algorithms for its implementation. Moreover DST-IV fast algorithms, which rely on the DCT-IV, were extensively studied and rely mostly on DCT-II alike algorithms and can be expressed in a recursive fashion [4].

# Implementation and Evaluation

The proposed additional transforms were implemented in a straightforward fashion, consistent with the JEM2 software. The fast implementation of the proposed ID/DST-IV is not made part of this proposal.

The tables defining the pair of horizontal/vertical transforms (essentially g_aucTrSetVert and g_aucTrSetHorz) were updated to introduce the newly introduced core transforms that are applied only for the Intra prediction modes.

These modifications represent the two changes introduced to the JEM2.

The software was compiled using gcc (v 4.9.2-10) and run under a Debian 3.16.7 Unix distribution.

The coding results are presented hereunder using the JVET common test conditions defined in [5] in comparison with the JEM2.

From these numbers it can be concluded that:

- The newly introduced core transforms provide an average luma BD-Rate improvement for both test conditions, in the range of -0.37% in AI and -0.23% in RA.

- All the test items are improved.

- Class F (not accounted in the average calculation) exhibits larger improvements.

- The encoding and decoding times are comparable to the JEM2. Although a slight increase in coding times can sometimes be noticed. This probably comes from an increased usage of the non-DCT-II transforms that don’t have benefit from a fast implementation in the JEM2 software yet.

|  | All Intra over JEM2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.20% | -0.06% | 102% | 101% |
| Class A2 | -0.31% | 0.01% | 0.02% | 102% | 101% |
| Class B | -0.33% | -0.28% | -0.21% | 101% | 100% |
| Class C | -0.45% | -0.19% | -0.18% | 100% | 101% |
| Class D | -0.48% | -0.29% | -0.23% | 99% | 100% |
| Class E | -0.53% | -0.31% | -0.34% | 102% | 100% |
| Overall | -0.37% | -0.21% | -0.16% | 101% | 101% |
| Class F (optional) | -1.32% | -0.54% | -0.51% | 100% | 100% |
|  | Random Access over JEM2 (parallel) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.20% | -0.02% | 99% | 98% |
| Class A2 | -0.39% | -0.17% | -0.03% | 100% | 100% |
| Class B | -0.17% | 0.19% | 0.11% | 100% | 102% |
| Class C | -0.24% | 0.04% | -0.07% | 99% | 101% |
| Class D | -0.25% | -0.21% | -0.27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.23% | 0.02% | -0.05% | 100% | 100% |
| Class F (optional) | -1.09% | -0.34% | -0.61% | 99% | 87% |
|  | Low Delay B over JEM2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.14% | 0.12% | 98% | 99% |
| Class C | -0.11% | 0.02% | -0.24% | 97% | 98% |
| Class D | -0.02% | -0.90% | -0.55% | 99% | 100% |
| Class E | -0.14% | -0.23% | -0.65% | 97% | 98% |
| Overall (Ref) |  |  |  |  |  |
| Class F (optional) | -0.67% | 0.07% | -0.32% | 98% | 98% |
|  | Low Delay P over JEM2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | ## -0.04% |  | ## -0.04% |  | ## -0.27% |  | 98% |  | 98% |  |
| Class C | -0.08% | -0.14% | -0.15% | 97% | 98% |
| Class D | -0.11% | -0.29% | 0.48% | 98% | 97% |
| Class E | -0.05% | -0.27% | -0.60% | 97% | 97% |
| Overall (Ref) |  |  |  |  |  |
| Class F (optional) | -0.63% | 0.08% | -0.17% | 98% | 99% |

# Conclusions

This contribution introduces two additional core transforms in the AMT design: the DST-IV and the ID. Both transforms can benefit from know fast implementations. It is reported that the BDrate is improved using that change with no impact on the complexity.

We would like to thank Samsung for performing the crosscheck of this proposal.

# References

- JVET-B1001 Algorithm description of Joint Exploration Test Model 2, March 2016.

- J. Han et al, Towards jointly optimal spatial prediction and adaptive transform in video/image coding, IEEE ICASSP 2010.

- JCT-VC D0235, Enhancements to Intra Coding, S. Kanumuri, T.K. Tan, F. Bossen, Jan. 2011.

- V. Britanak et al. Discrete Cosine and Sine Transforms — General Properties, Fast Algorithms and Integer Approximations, 1st Edition, Academic Press, 2006

- JVET-B1010 JVET common test conditions and software reference configurations, March 2016

# Patent rights declaration

Orange Labs and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).