| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0047 |
| --- | --- |

| Title: | Generalized bi-prediction for inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 chunchi.chen@interdigital.com xiaoyu.xiu@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This proposal presents a generalized bi-prediction (GBi) mode that allows bi-prediction weights to be signalled at the CU level and used for the bi-predicted PUs in that CU. The candidate set of weights in the GBi mode includes a total of 7 weights, including 0.5 which corresponds to the conventional bi-prediction mode. An index pointing to the entry position of a weight value in the candidate weight set is signalled. At most one index per CU is signalled and the corresponding weight values are shared across all the PUs and all color components in that CU. When compared with the JEM-2.0 anchor, GBi performs consistently better in all test cases, with average Y BD-rate reduction of 1.00%/0.79%/0.79% for Random Access cases (Class B/C/D) and 0.93%/0.84%/1.09% for Low Delay B cases (Class C/D/E).

# Introduction

When illumination fluctuates from one picture to another, existing methods such as weighted prediction (WP) [1] and local illumination compensation (IC) [2] could be used to improve the performance of motion compensated prediction. Both WP and IC apply a weight and an offset to one motion-compensated prediction block associated with a reference picture. In the case of bi-prediction, the corresponding weights and offsets are applied to each of the two prediction blocks before the two prediction blocks are added up together. For WP, the weights and offset values are signalled at slice level. For IC, the weights and offsets are derived from causal-neighbouring template.

In bi-prediction mode, weights may be still needed even though there is no obvious illumination change between two motion-compensated prediction blocks. This is because the prediction quality of one motion-compensated prediction block may differ from the other due to factors such as quantization. This proposal introduces a generalized bi-prediction (GBi) mode to predict a block by combining two motion-compensated prediction blocks using block-level adaptive weights from a pre-defined set of candidate weights. As such, the prediction process of GBi can reuse the existing logics of weighted prediction, and no extra decoding burden is introduced. We describe the proposed prediction technique in the next section.

# Technical description

## Generalized bi-prediction

GBi is a weighted bi-prediction technique. It computes the prediction signal of a block as a weighted average of two motion-compensated prediction blocks using block-level adaptive weights, where the weight values need not be restricted to 0.5 which conventional bi-prediction uses. This prediction can be formulated as

| - P[x] = (1 - w) * P0[x + v0] + w * P1[x + v1], | (1) |
| --- | --- |

where P[x] denotes the prediction of a current-block sample x located at a picture position x, each Pi[x+vi], ∀i∈{0, 1}, is a motion-compensated prediction of x using a motion vector (MV) vi from a reference picture in reference list Li, 1-w and w represent weight values applied to each samples in P0[x+v0] and P1[x+v1], respectively. In this proposal, in addition to 0.5, six more weight values are specified. All the weight values are arranged from the lowest to the highest in the candidate set, W={-1/4, 1/4, 3/8, 1/2, 5/8, 3/4, 5/4}.

## Coding of weight values

An index is introduced to indicate the entry position where w is located in the set (that is W) of candidate weights. It is then binarized using one of the two binarization schemes specified in Table 1. The choice of the binarization scheme adapts for each slice, depending on the value of the slice-level flag, mvd_l1_zero_flag. When it is equal to 0, Scheme #1 is used; otherwise (when it is equal to 1), Scheme #2 is used. Each bin is then context-coded after binarization.

To reduce signalling overhead for the index, each coding unit (CU) is assigned with at most one w shared across all its bi-prediction prediction units (PUs). The index of w is signalled explicitly, if a CU contains at least one bi-prediction PU that needs signalling motion vector difference (MVD). Otherwise, no index needs to be signalled. Then, the following rules are applied for determining the weight value for each PU in a CU.

- For each bi-prediction PU that requires signalling MVD (i.e. normal inter prediction mode and affine prediction mode [3]), its weight value is set equal to the explicitly signalled w.

- For each bi-prediction PU that is coded with merge modes, advanced temporal motion vector prediction [4] or the affine merge mode [3], its weight value w is inferred directly from the weight value used for the associated merge candidate.

- For the remaining bi-prediction PUs, their weight values are set equal to the default weight value, that is, 0.5.

Due to its similarity to conventional bi-prediction (i.e. w=0.5), the GBi mode is extended in a straightforward manner to temporal prediction techniques that support bi-prediction in the JEM-2.0 software [5], such as merge modes, advance temporal motion vector prediction, affine prediction mode and affine merge mode, bi-directional optical flow [6], and overlapped block motion compensation [7].

- Index binarization schemes.

| Index | Weight value | Binarization Schemes |  |
| --- | --- | --- | --- |
|  |  | Scheme #1 (mvd_l1_zero_flag=0) | Scheme #2 (mvd_l1_zero_flag=1) |
| 0 | -1/4 | 000000 | 000000 |
| 1 | 1/4 | 00001 | 000001 |
| 2 | 3/8 | 001 | 0001 |
| 3 | 1/2 | 1 | 01 |
| 4 | 5/8 | 01 | 1 |
| 5 | 3/4 | 0001 | 001 |
| 6 | 5/4 | 000001 | 00001 |

## Combination with local illumination compensation

For each PU coded with IC and GBi, the prediction in Eq. (1) is re-formulated as

| - P[x] = (1 - w) * (w0 * P0[x + v0] + o0) + w * (w1 * P1[x + v1] + o1), | (2) |
| --- | --- |

where w0 and o0 are the derived weight and offset value for P0, and w1 and o1 are the derived weight and offset value for P1.

# Experimental results

The experiment for GBi were conducted using the HM-16.6+JEM-2.0 software [5] and the common test conditions [8] to collect the BD-rate numbers and the encoding and decoding time ratios. The performance results are summarized in Table 2. Additional results for performing the GBi on top of the HM-16.6 are also provided in Table 3. It is noted that this HM-16.6 software base is referred to as the JEM-2.0 software with disabling all the non-HM macros, and the parallel encoding and parallel decoding as described in JVET-B0036 are applied to Random Access cases for both anchor and test.

- BD-rate results and processing time ratios relative to JEM-2.0.

|  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -1.00% | -0.97% | -0.99% |  |  |  |
| Class C | -0.79% | -0.62% | -0.81% | -0.93% | -1.22% | -1.54% |
| Class D | -0.79% | -0.64% | -0.66% | -0.84% | -1.15% | -1.39% |
| Class E |  |  |  | -1.09% | -1.04% | -0.78% |
| Avg. |  |  |  |  |  |  |
| Enc. Time [%] |  |  |  |  |  |  |
| Dec. Time [%] |  |  |  |  |  |  |

- BD-rate results and processing time ratios relative to HM-16.6.

|  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.77% | -1.25% | -1.26% |  |  |  |
| Class A2 | -1.39% | -1.93% | -1.82% |  |  |  |
| Class B | -1.25% | -1.34% | -1.33% | -1.87% | -1.73% | -1.48% |
| Class C | -0.93% | -1.02% | -1.11% | -2.30% | -1.69% | -2.13% |
| Class D | -0.68% | -0.71% | -0.71% | -1.21% | -0.88% | -0.66% |
| Class E |  |  |  | -2.15% | -1.40% | -1.25% |
| Avg. | -1.01% | -1.26% | -1.25% | -1.87% | -1.45% | -1.40% |
| Enc. Time [%] | 346% |  |  | 397% |  |  |
| Dec. Time [%] | 104% |  |  | 111% |  |  |

# Conclusion

In this contribution, a generalized bi-prediction mode is proposed. Coding performance improvement relative to the JEM-2.0 software is reported. The prediction process of GBi can be realized by reusing the existing processing unit of the weighted prediction, so nearly no extra decoding burden is introduced. In view of these, we would recommend the committee to consider integrating the proposed technique into the next release of JEM software.

# Patent rights declaration

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

High Efficiency Video Coding (HEVC), Rec. ITU-T H.265 and ISO/IEC 23008-2, Jan. 2013.

M. Karczewicz, J. Chen, W.-J. Chien, X. Li, A. Said, L. Zhang and X. Zhao, “Study of coding efficiency improvements beyond HEVC,” ISO/IEC MPEG doc. m37102, Oct. 2015.

S. Lin, H. Chen, H. Zhang, S. Maxim, H. Yang, and J. Zhou, “Affine transform prediction for next generation video coding,” ITU-T SG16/Q6 Doc. COM16–C1016, Oct. 2015.

J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang and X. Zhao, “Coding tools investigation for next generation video coding,” ITU-T SG16/Q6 Doc. COM16–C806, Feb. 2015.

JEM reference software JEM-2.0, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware, Mar. 2016.

Kiho Choi, E. Alshina, A. Alshin and C. Kim, “Information on coding efficiency improvements over HEVC for 4K content,” ISO/IEC MPEG Doc. m37043, Oct. 2015.

M. Karczewicz, J. Chen, W.-J. Chien, X. Li, A. Said, L. Zhang and X. Zhao, “Study of coding efficiency improvements beyond HEVC,” ISO/IEC MPEG Doc. m37102, Oct. 2015.

K. Suehring and X. Li, “JVET common test conditions and software reference configurations,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, Feb. 2016.