| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0052 |
| --- | --- |

| Title: | The cross-check for BIO improvement to reduce the encoder and decoder complexities in JVET-C0031 |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | E. Alshina | Email: | elena_a.alshina@samsung.com |
| Source: |  | Samsung Electronics, Ltd. |  |  |

____________________________

Abstract

This contribution verifies the performance of BIO simplifications proposed in JVCT-C0031 by LG. The goal of verified contribution is finding better performance complexity trade-off for BIO algorithm which is the part of JEM2.0. Performance perfectly matches to the data reported by proponent. Complexity reduction is analysed.

1. Introduction

Bi-directional optical flow is modification for bi-prediction which allows refinement of motion on sample level w/o any signaling. It was adapted to the HEVC-KTA at Geneva VCEG meeting [1], and later was modified by [2] for better adjustment to other tools. Eventually BIO has become the part of JEM [3].

Verified contribution proposes conditional disabling BIO in RA test configuration if following “consistent motion” condition is valid:

- two predictions come from different time directions;

- distances to reference frames are the same;

- block’s 2 MVs are exactly opposite to each other.

Additionally block size restriction could be applied.

We have tested 4 variants in total:

- Test1: BIO is disabled for “consistent motion” in sub-PUs 44 and smaller

- Test2: BIO is disabled for “consistent motion” in sub-PUs 88 and smaller

- Test3: BIO is disabled for “consistent motion” in sub-PUs 1616 and smaller

- Test4: BIO is disabled for all PUs with “consistent motion”.

Performance degradation and encoding / decoding run time reduction for all 4 tests are summarized in Table 1.

Table 1. Tests performance summary.

|  | BD-rate Y | BD-rate U | BD-rate V | Enc. Time | Dec. Time |
| --- | --- | --- | --- | --- | --- |
| Test1 (44) | 0.1% | 0.0% | 0.0% | 0,96 | 0,85 |
| Test2 (88) | 0.1% | 0.0% | 0.0% | 0,96 | 0,84 |
| Test3 (1616) | 0.1% | 0.1% | 0.0% | 0,94 | 0,83 |
| Test4 all sizes | 0.3% | 0.1% | 0.1% | 0,94 | 0,74 |

Which blocks use BIO?

In JEM2.0 BIO is applied for all bi-predicted PUs (except CUs with affine MC and illumination compensation). Figure 1 below shows “BIO enable map”. Blocks with BIO are shadowed. For BQSquare, QP=37, POC=8 we can see that ~85% of whole frame uses BIO. The same “BIO enabling map” is shown for the Test3 (BIO is disable for “consistent motion” in PUs no larger than 16x16). In this case about 65% of blocks are predicted using BIO.

| Fig. 1. BIO application map in JEM2.0 (left) and in JVET-C0031 (right); block which use BIO are shadowed. |  |
| --- | --- |

Test results

We ran all tests listed in Table 1 under JVET common test condition [4]. Verified proposal changes performance in RA configuration only. Strange effect we have observed is encoder run-time increment for high resolution classes B, A1, A2 despite some BIO related calculations are disabled in small PUs. This indicates that BIO prediction frequently results is zero residual and early termination happens during encoder’s search.

| Reference: | BIO is off in sub-PUs with “consistent motion” |  |  |  |  |  | BIO is off in sub-PUs with “consistent motion” |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JEM2.0 |  |  | 44 |  |  |  |  |  | 88 |  |  |
|  | Random Access Main 10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 | 0,0% | 0,2% | 0,1% |  |  |  |  |  |  |  |  |
| Class A2 | 0,0% | -0,1% | -0,1% |  |  |  |  |  |  |  |  |
| Class B | 0,1% | 0,1% | 0,1% | 106% | 99% |  | 0,1% | 0,0% | 0,0% | 98% | 89% |
| Class C | 0,1% | 0,0% | 0,0% | 89% | 84% |  | 0,1% | 0,0% | -0,1% | 87% | 83% |
| Class D | 0,2% | 0,1% | -0,1% | 81% | 77% |  | 0,2% | 0,2% | 0,0% | 80% | 76% |
| Overall JVET | 0,1% | 0,0% | 0,0% |  |  |  |  |  |  |  |  |
| Class F (optional) | 0,0% | 0,0% | 0,0% | 93% | 105% |  | -0,1% | -0,1% | 0,0% | 90% | 95% |

| Reference: | BIO is off in sub-PUs with “consistent motion” |  |  |  |  |  | BIO is off in sub-PUs with “consistent motion” |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JEM2.0 | 1616 |  |  |  |  |  | all sizes |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0,1% | 0,0% | 0,1% | 103% | 100% |  | 0,2% | 0,1% | 0,1% | 96% | 83% |
| Class C | 0,2% | 0,0% | 0,0% | 88% | 85% |  | 0,3% | 0,1% | 0,1% | 83% | 76% |
| Class D | 0,4% | 0,0% | -0,1% | 80% | 76% |  | 0,6% | 0,4% | 0,2% | 75% | 68% |
| Overall JVET |  |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) | -0,1% | 0,0% | 0,1% | 87% | 90% |  | -0,2% | -0,1% | -0,1% | 86% | 82% |

Conclusion

Among verified tests the first one (BIO disabled for sub-PUs with consistent motion and sizes no larger than 44) provides minimal performance degradation.

References

- E.Alshina, A.Alshin, J.-H. Min, K. Choi, A. Saxena, M. Budagavi, “Known tools performance investigation for next generation video coding,” VCEG-AZ05, Warsaw, June

- X. Li, J. Chen, W.-J. Chien, M. Karczewicz, “Harmonization and improvement for BIO”, ITU-T SG16 Doc. COM16–C1045, Oct. 2015.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm description of Joint Exploration Test Model 2”, JVET-B1001, San-Diego, Feb. 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-B1010, 2nd Meeting, Feb. 2016.

Patent rights declaration(s)

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).