| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JVET-C0069 |
| --- | --- |

| Title: | Direction-dependent scan order with JEM tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp ichigaya.a-go@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

In this contribution, performance analysis of direction-dependent sub-TU scan order (DDSO) on top of JEM is addressed. In the 2nd JVET meeting, JVET-B0028 proposed an improvement of intra prediction by DDSO on top of HM16.7. To clarify an interaction of improvement with intra-related tools integrated on JEM software, DDSO is implemented on top of JEM.

Introduction

In the 2nd JVET meeting in San Diego, direction-dependent sub-TU scan order (DDSO) is proposed [1]. DDSO focuses on inaccurate directional intra prediction caused by reference substitution process. In case that intra prediction direction is from top-right to bottom-left or from bottom-left to top-right, which is corresponding to from 2 to 17 or from 51 to 66 of JEM, a part of predicted samples of sub-TUs are inaccurately predicted because of neighboring sample substitution process. To avoid the inaccurate intra prediction, we proposed direction-dependent sub-TU scan order.

Direction-dependent sub-TU scan order

To simplify the explanation of the proposed DDSO, angular modes of intra prediction are classified into three categories as shown in Figure 1.

Figure 1. Categorization of directional intra prediction

When intra CU is divided into quadtree TUs and intra prediction direction is set to Category 1, half of the reference samples used for prediction of the upper-right TU are not available since the reference samples located in the lower-left TU are not decoded at this point. In that case, unavailable reference samples are substituted by immediately neighboring available reference sample. Figure 2 illustrates an example of inaccurate intra prediction caused by reference substitution process, where, gray pixels are unavailable reference samples and the value of the immediately neighboring available reference sample 'α' is copied to those gray pixels. As a result, all the red bordered pixels inside predicted signals have same value.

Figure 2. Inaccurate intra prediction

To avoid this inaccurate intra prediction, direction-dependent intra sub-TU scan order is proposed. If intra prediction direction is set to Category 1 or 3, sub-TU z-scan order is vertically or horizontally reversed. Figure 3 and 4 illustrate the proposed modification of z-scan order for intra prediction Category 1 and 3, respectively. The vertical reverse of z-scan order enables upper two sub-TUs to utilize immediate lower decoded samples as reference samples, therefore, prediction accuracy of upper sub-TUs can be improved, whereas that of lower sub-TUs remains unchanged. These accuracy improvements go for the horizontal reverse of z-scan order for Category 3. Additionally, when the CU is hierarchically divided into more than one level quadtrees and if immediate lower/right neighboring sub-TU inside other level of quadtree is available, lower reference samples can be also utilized for the prediction as shown in Figure 5.

Figure 3. Modified z-scan order for intra prediction Category 1

Figure 4. Modified z-scan order for intra prediction Category 3

Figure 5. Two level hierarchical TU division

Residual flipping for reversed reference position

As mentioned in section 2, immediately lower reference samples can be used for prediction for upper-left and upper-right TUs when the scan order is vertically reversed. If the reference sample position is changed, the residual statistics may also be changed since prediction accuracy has strong correlation with a distance between reference samples and predicted pixels. Table 1 shows a relationship between residual statistics, prediction mode and reference position. Figures in the table indicate residual statistics, where darker region nearby reference samples and lighter region far from reference samples are prone to be a small and large residual, respectively.

Table 1. Reference pattern and Residual statistics

| Prediction mode | 2 | 2 | 66 | 66 |
| --- | --- | --- | --- | --- |
| Reference position | Normal | Vert. reverse | Normal | Hori. reverse |
| Residual statistics |  |  |  |  |

In HEVC, DCT-II and DST-VII are adopted as transformation, and DST-VII is applied for small intra TU. The idea of the use of DST-VII which has asymmetric basis is coming from the fact that residual nearby reference samples is prone to be a small value.

In this contribution, adaptive residual flipping according to the reference position is proposed. If the scan order of sub-TUs is determined to be vertically/horizontally reversed and if the reference sample position can be also vertically/horizontally reversed, the residuals are vertically/horizontally flipped before transform. At the decoder side, the reconstructed residuals are also vertically/horizontally flipped after inverse transform.

Interaction with JEM intra-related tools

In JEM, several tools related to intra prediction are integrated such as 67 intra prediction modes, four-tap interpolation filter, boundary prediction filter, position dependent intra prediction combination (PDPC), and adaptive reference sample smoothing. Because of the scan order and reference position change proposed in this contribution, something to be concerned related to PDPC and boundary filter is observed. The improvements of the rest of those tools may be additive with DDSO according to the experimental results shown in the following section.

Position Dependent Prediction Combination (PDPC)

In PDPC [2], prediction is generated by combination of unfiltered and filtered reference samples. When reference sample position is reversed, the computation of the new prediction needs to be changed. Figure 6 illustrates examples of intra prediction in 4×4 blocks by PDPC with vertically reversed reference position.

Figure 6. PDPC harmonized with DDSO

Corresponding to reference position change, the computation of the new prediction is modified as follows:

,

where c1(v), c2(v), c1(h), and c2(h) are stored prediction parameters, d=1 for blocks with size smaller than or equal to 16×16, and d=2 for larger blocks, and b[x,y] is a normalization factor derived as follows:

Although the set of the prediction parameters is originally defined per intra prediction mode, proposed harmonization modifies the definition of the set per intra prediction and reference position. When the reference position is vertically reversed and let intra prediction mode be D, the set of the prediction parameters for (2*Hor-D) mode is used for PDPC, where Hor denotes horizontal intra prediction mode i.e. 18. In the same fashion, PDPC with horizontal reversed reference position is also modified. In this case, the set of the prediction parameter for (2*Ver-D) mode is used, where Ver denotes vertical intra prediction mode i.e. 50.

Intra Boundary Filter

When DDSO is enable, the left or top reference samples used for intra boundary filter process may not be available since those reference samples are not decoded in case that scan order is vertically or horizontally reversed. Therefore, intra boundary filter is disable when DDSO is enable in this contribution.

Experimental Results

The proposed DDSO is integrated on top of JEM1.0 and simulations were conducted following configuration:

- HM16.6+DDSO (vs HM16.6)

- HM16.6+DDSO+Residual Flipping (vs HM16.6)

- JEM1.0- Intra Boundary Filter (BF)+DDSO+Residual Flipping+PDPC harmoniztion (vs JEM1.0-BF)

Table 2. BD-rate of HM16.6+DDSO vs HM16.6

| Class | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | V | U | V |
| ClassA1 | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassA2 | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassA | -0.23% | -0.47% | -0.38% | -0.10% | -0.35% | -0.23% |
| ClassB | -0.23% | -0.44% | -0.45% | -0.11% | -0.14% | -0.25% |
| ClassC | -0.14% | -0.23% | -0.20% | -0.04% | 0.02% | -0.10% |
| ClassD | -0.08% | -0.12% | -0.17% | -0.04% | -0.07% | -0.21% |
| ClassE | -0.29% | -0.46% | -0.55% | -0.17% | -0.42% | -0.44% |
| ClassF | -0.10% | -0.15% | -0.10% | -0.08% | -0.27% | 0.00% |
| Average | -0.18% | -0.31% | -0.30% | -0.09% | -0.19% | -0.20% |
| Enc Time | 103% |  |  | 100% |  |  |
| Dec Time | 104% |  |  | 104% |  |  |

Table 3. BD-rate of HM16.6+DDSO+Residual Flipping vs HM16.6

| Class | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | V | U | V |
| ClassA1 | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassA2 | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassA | -0.27% | -0.55% | -0.35% | -0.13% | -0.35% | -0.37% |
| ClassB | -0.25% | -0.46% | -0.47% | -0.12% | -0.15% | -0.21% |
| ClassC | -0.17% | -0.25% | -0.21% | -0.06% | -0.13% | -0.14% |
| ClassD | -0.11% | -0.14% | -0.15% | -0.05% | 0.19% | -0.32% |
| ClassE | -0.35% | -0.49% | -0.64% | -0.24% | -0.50% | -0.48% |
| ClassF | -0.12% | -0.23% | -0.19% | -0.06% | -0.17% | -0.21% |
| Average | -0.21% | -0.35% | -0.33% | -0.11% | -0.17% | -0.28% |
| Enc Time | 103% |  |  | 100% |  |  |
| Dec Time | 104% |  |  | 103% |  |  |

Table 4. BD-rate of JEM1.0-BF+DDSO+Residual Flipping+PDPC harm. vs JEM1.0-BF

| Class | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | V | U | V |
| ClassA1 | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassA2 | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassA | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassB | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassC | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassD | -0.10% | -0.17% | -0.36% | TBP | TBP | TBP |
| ClassE | TBP | TBP | TBP | TBP | TBP | TBP |
| ClassF | TBP | TBP | TBP | TBP | TBP | TBP |
| Average | TBP | TBP | TBP | TBP | TBP | TBP |
| Enc Time | TBP |  |  | TBP |  |  |
| Dec Time | TBP |  |  | TBP |  |  |

Recommendation

In this contribution, we propose direction-dependent sub-TU scan order (DDSO) with adaptive residual flipping. DDSO is implemented on top of JEM1.0 with PDPC harmonization. We recommend to integrate this tool on prospective JEM software for study and evaluation of the modification of sub-TU z-scan order.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).