| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0084 |
| --- | --- |

| Title: | Cross-check of JVET-C0040 (Adaptive Clipping) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | P. Philippe V. Lorcy T. Biatek | Email: | pierrick.philippe@orange.com |
| Source: |  | Orange, b<>com, TDF |  |  |

____________________________

# Abstract

This contribution reports the evaluation of JVET-C0040 (Adaptive Clipping in JEM2.0).

The performance of the proposal in terms of runtime and BDrate is assessed and the code is analysed to highlight the proposed changes.

# Introduction

Contribution JVET-C0040 proposes an improved clipping scheme is proposed. The clipping operation as proposed occurs on a component basis. The clipping bounds are conveyed at the slice level. Six elements are represented at that level: ymin, ymax, cmin, cmax, c1min and c1max.

# Software analysis

The proposal consists in the modification of 27 files consisting in roughly 1300 modified lines. The changes are clearly identified using a macro called TCH_CONTRIB_CLIP, the code is pretty clear and understandable.

The lines containing clipping functions represent the vast majority of the changes. For example TComInterpolationFilter.cpp includes 229 lines changed, this represents 71 blocs of modifications that consist only in adaptation of the function interfaces to handle the clipping.

The clipping thresholds are computed, on a slice-by-slice basis, for each component, this is performed by the determination of the residual dynamic (min/max). A set of functions called by computeTchClipParam (see TComSlice.cpp) is added for that purpose.

Additional macros are defined to propose refinement of the basic principles:

- TCH_CONTRIB_CLIP proposes a residual smoothing.

- TCH_CONTRIB_CLIP_ON_SELECTION provides a method to automatically activate/deactivate the clipping for the slice depending on some predetermined thresholds.

# Test results

We ran simulations under JVET common test condition. The three macros presented above are switched on in the simulation.

Test results are summarized in the Table below for AI, RA and LDB modes. The results match those presented in JVET-C0040.

|  | All Intra over JEM2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.14% | -0.16% | 107% | 112% |
| Class A2 | -0.05% | -0.05% | -0.11% | 107% | 110% |
| Class B | -0.20% | -0.11% | -0.02% | 106% | 107% |
| Class C | -0.15% | 0.04% | -0.02% | 105% | 108% |
| Class D | -0.34% | -0.05% | -0.21% | 105% | 109% |
| Class E | -0.02% | -0.05% | 0.15% | 108% | 110% |
| Overall | -0.17% | -0.06% | -0.07% | 106% | 109% |
| Class F (optional) | -0.86% | 0.09% | 0.00% | 112% | 108% |
|  | Random Access over JEM2 (parallel) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.02% | -0.10% | 105% | 105% |
| Class A2 | -0.26% | -0.28% | -0.25% | 104% | 104% |
| Class B | -0.35% | 0.16% | 0.02% | 103% | 102% |
| Class C | -0.17% | 0.00% | 0.08% | 103% | 102% |
| Class D | -0.75% | -0.59% | -0.48% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.35% | -0.13% | -0.14% | 104% | 103% |
| Class F (optional) | -0.59% | 0.26% | -0.03% | 108% | 104% |
|  | Low Delay B over JEM2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | -0.05% | 0.21% | 104% | 103% |
| Class C | -0.15% | -0.03% | -0.26% | 104% | 104% |
| Class D | -0.51% | -0.62% | -0.08% | 103% | 101% |
| Class E | 0.16% | 0.77% | 0.11% | 104% | 103% |
| Overall (Ref) | -0.23% | -0.03% | 0.00% | 104% | 103% |
| Class F (optional) | -0.56% | 0.25% | -0.82% | 108% | 106% |

A mismatch occurs for the decoding times, but we believe that the numbers reported are subject to uncertainty. Based on the code analysis, it is believed that the decoding times should be very similar to the reference (JEM2).

The encoding time is slightly increased; this is mostly due to the residual analysis performed to determine the clipping thresholds. The increase is on average less than 6%.

On a per sequence basis:

- For AI the gains are in the range of -2.21% - +0.25% (a negative value means improvement)

- For RA the gains are in the range of -2.84% - +0.14%

- For LDB the gains are in the range of -2.05% to 0.27%

# Conclusion

In this contribution, the proposal described in JVET-C0040 is analyzed and evaluated according to the common test conditions. The average performance is -0.17%, -0.35% and -0.23% respectively for AI, RA and LDB configurations. This improvement comes at the price of a minor increase in encoding runtimes

# Patent rights declaration(s)

Orange, b<>com, TDF may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: list of the modified source files

Expressed in number of lines affected.

source/App/TAppEncoder/TAppEncCfg.cpp              |  28

source/App/TAppEncoder/TAppEncCfg.h                |   3

source/App/TAppEncoder/TAppEncTop.cpp              |   3

source/Lib/TLibCommon/CommonDef.h                  |  87

source/Lib/TLibCommon/TComAdaptiveLoopFilter.cpp   |  15

source/Lib/TLibCommon/TComInterpolationFilter.cpp  | 229

source/Lib/TLibCommon/TComInterpolationFilter.h    |  27

source/Lib/TLibCommon/TComLoopFilter.cpp           |  33

source/Lib/TLibCommon/TComLoopFilter.h             |   6

source/Lib/TLibCommon/TComPrediction.cpp           |  69

source/Lib/TLibCommon/TComPrediction.h             |   8

source/Lib/TLibCommon/TComRom.cpp                  |  18

source/Lib/TLibCommon/TComSampleAdaptiveOffset.cpp |  47

source/Lib/TLibCommon/TComSampleAdaptiveOffset.h   |   6

source/Lib/TLibCommon/TComSlice.cpp                |  85

source/Lib/TLibCommon/TComSlice.h                  |   7

source/Lib/TLibCommon/TComYuv.cpp                  |  86

source/Lib/TLibCommon/TComYuv.h                    |   8

source/Lib/TLibCommon/TypeDef.h                    |   7

source/Lib/TLibDecoder/TDecCAVLC.cpp               |  84

source/Lib/TLibDecoder/TDecCu.cpp                  |  20

source/Lib/TLibDecoder/TDecGop.cpp                 |   7

source/Lib/TLibDecoder/TDecTop.cpp                 |   1

source/Lib/TLibEncoder/TEncCavlc.cpp               |  89

source/Lib/TLibEncoder/TEncCfg.h                   |   7

source/Lib/TLibEncoder/TEncSearch.cpp              | 136

source/Lib/TLibEncoder/TEncSlice.cpp               | 280