| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0026 |
| --- | --- |

| Title: | AHG8: New GoPro Test Sequences for Virtual Reality Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adeel Abbas, Balu Adsumilli | Tel: Email: | aabbas@gopro.com |
| Source: | GoPro |  |  |

_____________________________

# Abstract

This contribution provides a new set of GoPro Virtual Reality (VR) video sequences. The new sequences are of much higher quality than the ones that were previously shared, with the hope that they would enable a more thorough testing for VR compression tools.

# Introduction

In the last meeting, GoPro shared some VR test sequences with the goal that JVET would consider investigating compression tools for VR content [1]. Based on the overwhelming interest and valuable feedback, we felt that it was necessary to share some more content. The new content addresses limitations of the old content in following ways:

- Higher resolution. Our new content is provided at 8192x4096 master resolution.

- No encoding is done after stitching. This way, the new content has lesser compression artifacts

- Lesser stitching artifacts, due to I) better stitching algorithms and II) the fact that our new camera rigs can do frame level synchronization (Genlock-ed)

# Sequences

The 8K source files are being provided in equirectangular projection (ERP) format. The specifications of the files are tabulated below:

| Sequence | Resolution@FPS | Bit Depth | #Frames | Stereo |
| --- | --- | --- | --- | --- |
| train_le train_re | 8192x4096@60 | 8 | 600 | Yes |
| skateboard_trick_le skateboard_trick_re | 8192x4096@60 | 8 | 600 | Yes |
| jam_session_le jam_session_ re | 8192x4096@30 | 8 | 300 | Yes |
| basketball | 8192x4096@30 | 10 | 300 | No |
| chairlift_ride | 8192x4096@30 | 10 | 300 | No |
| skateboard_in_lot | 8192x4096@30 | 10 | 300 | No |

The ERP-8K files have been converted to ERP-4K format using PCT360 software [2]. PCT360 does downsampling and upsampling along with format conversion in one step, by sampling points on a sphere. This provides higher quality resampling than using conventional 2D resampling method.

Following is an overview of the new 4K sequence files:

| Sequence | Resolution@FPS | Bit Depth | #Frames | Stereo |
| --- | --- | --- | --- | --- |
| train_le train_re | 4096x2048@60 | 10 | 600 | Yes |
| skateboard_trick_le skateboard_trick_re | 4096x2048@60 | 10 | 600 | Yes |
| jam_session_le jam_session_re | 4096x2048@30 | 10 | 300 | Yes |
| basketball | 4096x2048@30 | 10 | 300 | No |
| chairlift_ride | 4096x2048@30 | 10 | 300 | No |
| skateboard_in_lot | 4096x2048@30 | 10 | 300 | No |

# Sequence Thumbnails

train_le

skateboard_trick_le

jam_session_le

basketball

chairlift_ride

skateboard_in_lot

# References

- A. Abbas, “GoPro test sequences for Virtual Reality video coding”, 54th Meeting: May 2016, Geneva (JVET-C0021)

- Y. He, B. Vishwanath and Y. Ye, “InterDigital’s projection format conversion tool”, 4th Meeting: Chengdu, CN, 15–21 October 2016

# Sequence Md5Sums

| train_le_4096x2048_60Hz_10bit_P420.yuv | 5dbf64bdb596c4e325962e840c393575 |
| --- | --- |
| train_le_8192x4096_60Hz_8bit_P420.yuv | 90287c1b8cb03c71973e1e10949f75ad |
| train_re_4096x2048_60Hz_10bit_P420.yuv | 3a20240c69c94ba3c659ac4a8947c535 |
| train_re_8192x4096_60Hz_8bit_P420.yuv | 5ae42e209e71eac028142bbb58deed91 |
| skateboard_trick_le_4096x2048_60Hz_10bit_P420.yuv | 3a4117475f4825b4db194c21bd5e89e7 |
| skateboard_trick_le_8192x4096_60Hz_8bit_P420.yuv | 24029fe5bf03eca813952e2a6c700ba3 |
| skateboard_trick_re_4096x2048_60Hz_10bit_P420.yuv | 57c0d4eb4d8dda7f7a3fbb7be2a93d0f |
| skateboard_trick_re_8192x4096_60Hz_8bit_P420.yuv | 68659c0142fd05f329c1b782530e620b |
| jam_session_le_4096x2048_30Hz_10bit_P420.yuv | 57683049be8e306afa91ef006e3e7436 |
| jam_session_le_8192x4096_30Hz_8bit_P420.yuv | 664bc78b8086a53a6c855122aad72a6b |
| jam_session_re_4096x2048_30Hz_10bit_P420.yuv | 32930208862e669680c08ef9f4dbfe62 |
| jam_session_re_8192x4096_30Hz_8bit_P420.yuv | cec55032e9594f0942c646353ba755d7 |
| basketball_4096x2048_30Hz_10bit_P420.yuv | fef31663f1ca56c16a5d941d0abca1d4 |
| basketball_8192x4096_30Hz_10bit_P420.yuv | 6b00c50efbf9cdc2cdd4bf4501ba8212 |
| chairlift_ride_4096x2048_30Hz_10bit_P420.yuv | beb72d938c2cedd52824e81b88707f76 |
| chairlift_ride_8192x4096_30Hz_10bit_P420.yuv | 66a2b36158e726a9c9c825e73d39ee23 |
| skateboard_in_lot_4096x2048_30Hz_10bit_P420.yuv | 460696a0049785cf98fa67db76fc6dfd |
| skateboard_in_lot_8192x4096_30Hz_10bit_P420.yuv | e8eae04c43e959060f641fec4892fced |

# Copyright Information

The sequences provided in this contribution are owned by GoPro.

The following uses are allowed for the proposed sequences:

- Can be published in technical papers, played at technology research and development events.

- Can be used by Standards committees. (e.g., SMPTE, ITU, MPEG, VQEG).

The following uses are NOT allowed for the proposed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.