| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0033-syntax |
| --- | --- |

| Title: | EE7 Adaptive Clipping syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Bordes, F. Galpin, F. Le Léannec, E. Francois 975 Avenue des Champs Blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33 2 99 27 31 98 { philippe.bordes, franck.galpin, fabrice.leleannec, edouard.francois} @technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a syntax for signalling and coding adaptive clipping parameters described in JVET-D0033, in line with the recommendations of the chairs.

# Introduction

EE7 goal was to investigate in-loop adaptive clipping, with signaling of the clipping bounds in slice header. Several tests have been conducted [4] showing that gains up to 0.4% in RA w/o Enc. run time increment can be achieved [5].

Chair’s notes:

The min and max values are signalled per component at slice level. From the EE results, there is some small benefit when clipping is performed within the loop (0.1% difference).

One disadvantage of the slice header approach is the impact on delay or necessity of multi-pass encoding.

The EE7 proposal is asserted to be beneficial for obtaining gain achievable by clipping, without concluding that this would be a viable solution to be used in a final standard.

It has been decided to adopt D0033, simple version with explicit signalling of clipping values for the three components in the slice header, no prediction, no variable length coding. Additional signalling of the bit depth of the clipping values in SPS.

# Syntax

Signalling of the bit depth of the clipping values in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| adaptive_clipping_enabled_flag | u(1) |
| if ( adaptive_clipping_enabled_flag ) { |  |
| bit_depth_shift_clip | ue(v) |
| } |  |
| … |  |
| } |  |

adaptive_clipping_enabled_flag equal to 1 indicates that the adaptive clipping may be enabled in the pictures. adaptive_clipping_enabled_flag equal to 0 indicates that the adaptive clipping is disabled in the pictures.

bit_depth_shift_clip specifies the bit depth shift of the clipping bounds values. If not present, it is inferred to be equal to 0. The value of bit_depth_shift_clip shall be in the range 0 to 6.

Explicit signalling of clipping values in the slice header (no prediction, no variable length coding)

| slice_segment_header() { | Descriptor |
| --- | --- |
| … |  |
| if (first_slice_segment_in_pic_flag) { |  |
| if ( adaptive_clipping_enabled_flag ) { |  |
| adaptive_clipping_slice_flag | u(1) |
| if ( adaptive_clipping_slice_flag ) { |  |
| clip_min_quant_adaptive[0] | ue(v) |
| clip_max_quant_adaptive[0] | ue(v) |
| adaptive_clipping_slice_chroma_flag | u(1) |
| if (adaptive_clipping_slice_chroma_flag ) { |  |
| for (cIdx=1 ; cIdx<3 ; cIdx++ ) { |  |
| clip_min_quant_adaptive[cIdx] | ue(v) |
| clip_max_quant_adaptive[cIdx] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

adaptive_clipping_slice_flag equal to 1 indicates that the adaptive clipping is enabled in the picture and the clipping bounds syntax elements are present. adaptive_clipping_slice_flag equal to 0 indicates that the adaptive clipping is disabled in the picture and the clipping bounds syntax elements are not present. If not present, adaptive_clipping_slice_flag is inferred to be 0.

adaptive_clipping_slice_chroma_flag equal to 1 indicates that the adaptive clipping is enabled in the picture for chroma samples and the clipping bounds syntax elements for chroma are present. adaptive_clipping_slice_chroma_flag equal to 0 indicates that the adaptive clipping for chroma is disabled in the picture and the clipping bounds syntax elements for chroma are not present. If not present, adaptive_clipping_slice_chroma_flag is inferred to be 0.

The values of clip_min_quant_adaptive[0] and clip_max_quant_adaptive[0] shall be in the range of 0 to (1<<(BitDepthY-bit_depth_shift_clip)-1). If not present, clip_min_quant_adaptive[0] and clip_max_quant_adaptive[0] are inferred to be equal to 0 and ((1<<BitDepthY)-1) respectively. The values of clip_min_quant_adaptive[cIdx] and clip_max_quant_adaptive[cIdx] for cIdx=1,2, shall be in the range of 0 to (1<<(BitDepthC-bit_depth_shift_clip)-1). If not present, clip_min_quant_adaptive[cIdx] and clip_max_quant_adaptive[cIdx] for cIdx=1,2, are inferred to be equal to 0 and ((1<<BitDepthC)-1) respectively.

If adaptive clipping is enabled in the picture, the clipping bounds values are derived as follows (cIdx=0,1,2):

clip_min_adaptive[cIdx] = clip_min_quant_adaptive[cIdx] << bit_depth_shift_clip

clip_max_adaptive[cIdx] = clip_max_quant_adaptive[cIdx] << bit_depth_shift_clip

# References

- JEM-3.0, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-3.0/

- F. Galpin, P. Bordes, F. Le Léannec, “Adaptive Clipping in JEM2.0,” 3rd Meeting: Geneva, CH, 26 May – 1 June 2016, JVET-C0040.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 3rd Meeting

- F. Galpin, P. Bordes, F. Le Léannec, E. Francois, “Adaptive Clipping in JEM3.0,” 4th Meeting: Chengdu, CN, 15–21 October 2016, JVET-D0033.

- E. Alshina, J. Boyce, Xiaoyu Xiu, Yao Jen Chang, Vadim Seregin, S.-H. Kim, “Exploration Experiments on Coding Tools Report,” 4th Meeting: Chengdu, CN, 15–21 October 2016, JVET-D0010.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).