| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0042 |
| --- | --- |

| Title: | AHG6: On BIO memory bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Alshin, E. Alshina Seoul R&D Campus F-10 A Zone, Seongchon-gil 34, Seocho-gu, Seoul, Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution analyzes memory access needed for BIO compared to HEVC motion compensation, suggests small modification which equalizes memory access for BIO and HEVC motion compensation from view point of the memory bandwidth. Suggested simplification of BIO doesn’t lead to the performance degradation but reduces Enc. run time by 13% and Dec. run time by 18% in RA configuration.

# Introduction

HEVC standard uses 8-taps interpolation filter for Luma and 4-taps filter for Chroma motion compensation. Bi-prediction is restricted for blocks smaller than 88. MC for small blocks 44 is not allowed in HEVC. With those restrictions memory bandwidth of HEVC is ~15% higher than H.264/AVC, and this was considered to be acceptable at the time HEVC was developed.

JEM includes multiple tools which increase memory bandwidth compared to HEVC. This contributions studies one of JEM3.0 tools namely bi-directional optical flow (BIO). It was found that in a form BIO implemented in JEM3.0 small block extension (2 lines from each side of block) is needed for blocks compensated with BIO, which results in some memory band-width increment (despite BIO uses shorter interpolation filters). The reason for this was found. Modification of BIO which doesn’t require block extension has been tested.

# Memory access for BIO

Bi-directional optical flow (BIO) is the tool which refines small motion inside the block on top of Block-base motion compensation (Fig. 1).

Figure 1: bi-directional optical flow.

Conventional B-slice prediction is semi-sum of prediction from different references.

Pred=I0+I12 					(1)

However formula can be enhance using model of optical flow [3]-[7]. This enhancement is additional term which represents displacement of each sample in a block according to the motion vector vx,vy. In most frequent case τ0=τ1=τ . It can be written as :

Pred=I0+I12-τ2vxIx0-Ix1+vyIy0-Iy12.            (2)

Here I0,I1 are Luminance from Ref0 and Ref1 and Ix0,Iy0,Ix1,Iy1 are horizontal and vertical gradients of signal in Ref0 and Ref1. Motion vector refinement is unknown in advance however it can be efficiently estimated using assumption of local steady motion (motion vector is constant inside small window side). In this case motion vector is solution of minimum for quadratic functional which estimates difference between two predictions from different references. The minimum of functional if its’ derivatives are equal to zero. This leads to the associated linear problem. Solving it we get an explicit formula for motion vector refinement vx,vy is following

vxi,jτ=σ1i,j+r>m?Clip3-l,l,-σ3i,jσ1i,j+r:0,

vyi,jτ=σ5i,j+r>m?Clip3-l,l,-σ6i,j-vxi,jσ22σ5i,j+r:0;                      (3)

where

σ1i,j=i',j'∈ΩIx1i',j'+Ix0i',j'2;

σ3i,j=i',j'∈ΩIx1i',j'+τ0Ix0i',j'I1i',j'-I0i',j'

σ2i,j=i',j'∈ΩIx1i',j'+Ix0i',j'Iy1i',j'+Iy0i',j'           (4)

σ5i,j=i',j'∈ΩIy1i',j'+Iy0i',j'2;

σ6i,j=i',j'∈ΩIy1i',j'+Iy0i',j'I1i',j'-I0i',j'.

Additional clipping was introduced to restrict value of motion vectors. As it is well known that optical flow model is valid only for small motion.

Mathematical meaning of this formula is following. Instead luminance values in collocated positions (as conventional bi-prediction combines) BIO (2) combines values in positions A and B (Fig. 1) which are D displaced relatively to the collocated position. The difference between luminance values in positions A and B together with their surrounding Ω is minimal. If A and B together with their surroundings are similar to each other then they likely belong to the same object. So by design BIO essentially combines samples along the motion trajectory.

In order to perform calculations in (4) both texture value and gradients are needed in all samples in a block and in their surroundings Ω. In JEM3.0 Ω is (2M+1)(2M+1), M=2 square window centered in currently predicted point.

So, in order to perform BIO calculations (4) both prediction for texture and gradients are calculated for the block which is 2 lines extended on each side (Fig. 2). Additionally block extension for interpolation is needed. This block extension depends on interpolation filter length T.

Since because Luma interpolation filter length is 8, for luminance W×H block motion compensation HEVC requires accessing the memoryW+7×H+7 in reference frame.

BIO uses shorter interpolation filter of length T=6, but due to the averaging in the surrounding Ω in (4) total memory access is W+T-1+2M×H+T-1+2M=W+9×H+9.

Figure 2: BIO memory access: JEM3.0. (left), tested (right).

In order to equalize memory access between regular motion compensation and BIO in this contribution we have redesigned (4) on a way only texture and gradients inside predicted block are used in averaging:

σ1i,j=j',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Ix1i',j'+Ix0i',j'2 wi',j'i,j

σ3i,j=i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Ix1i',j'+Ix0i',j'I1i',j'-I0i',j'wi',j'i,j

σ2i,j=i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Ix1i',j'+Ix0i',j'Iy1i',j'+Iy0i',j'  wi',j'i,j         (5)

σ5i,j=i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Iy1i',j'+Iy0i',j'2wi',j'i,j;

σ6i,j=i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1Iy1i',j'+Iy0i',j'I1i',j'-I0i',j'wi',j'i,j.

This additional weights wi',j' allows compensating the lack of neighboring pixels if current pixel near boundary. Weights wi',j'  are all zeros outside of predicted block, so instead i',j'∈Ω in (4) we have  i',j'∈Ω;0≤i'≤W-1;0≤j'≤H-1 in (5). It means amount of summand in (5) is fewer than in (4) and both texture and gradients values outside of predicted block are not needed.

Thus memory access for BIO becomes the same as for regular motion compensation (as shown on right side of the Fig. 2). Additionally since both perdition and gradients calculation in 2 lines surrounding of the predicted block is not required, so tested modification reduces computational complexity a lot.

# Test results

Modification (5) was implemented in JEM3.0 and tested in RA test scenario using common test conditions [2]. Test results are shown in Table 1. As we can see this modification doesn’t change performance much, doesn’t introduce any loss.

Table 1. Summary of test results in RA. (tested: no extra memory access in BIO, anchor: JEM3.0)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.19% | -0.07% | 84% | 85% |
| Class A2 | -0.09% | -0.22% | -0.17% | 83% | 82% |
| Class B | 0.01% | 0.03% | -0.04% | 90% | 81% |
| Class C | 0.01% | 0.06% | -0.03% | 92% | 81% |
| Class D | -0.01% | -0.06% | 0.15% | 86% | 80% |
| Class E |  |  |  |  |  |
| Overall JVET | -0.02% | -0.1% | 0.0% | 87% | 82% |
| Class F (optional) | -0.05% | 0.0% | 0.0% | 94% | 93% |

The reason for run time reduction is skipping of interpolation process (both texture prediction and gradients’ calculation) in 2 extra lines on each side of the compensated block.

# Conclusions

BIO has been simplified on a way 2 lines block extension on each side is not required any more. This modification 1) equalizes memory access for BIO and regular bi-prediction 2) reduces computational complexity of BIO. Experimental results in RA show no drop from this simplification while Enc. run time decreases 13% and Dec. run time decreases 18%.

# References

- JVET AHG report: JEM algorithm description editing (AHG2) .

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, JVET-B1010, February, 2016.

- A. Alshin, E. Alshina, “Bi-directional optical flow,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C204, Guangzhou, China, 10-15 October, 2010.

- Shunichi Sekiguchi, Yusuke Itani (Mitsubishi), “Report of CE1 :Decoder-Side Motion Vector Derivation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D098, Daegu, Jan. 2011

- A. Alshin, E. Alshina, T. Lee “Bi-directional optical flow for improving motion compensation,” Picture Coding Symposium (PCS), 2010.

- A.Alshin, E.Alshina, M.Budagavi, K. Choi, J. Min, M. Mishourovsky, Y. Piao, A.  Saxena “Coding efficiency improvements beyond HEVC with known tools”, SPIE Optics + Photonics 2015, San Diego.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).