| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0072 |
| --- | --- |

| Title: | AHG8: Area Weighted Spherical PSNR for 360 video quality evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bharath Vishwanath ,Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

At the 3rd JVET meeting, AHG8 on 360 video test conditions was established. One of AHG8’s mandates is to “discuss definition of common test conditions, test sequence formats, and evaluation criteria.” In terms of quality evaluation for 360 video content, it is generally agreed that the conventional PSNR metric is not a suitable quality metric. Spherical PSNR (SPSNR) and WS-PSNR (uniform Weighted SPNSR) have been previously proposed as more suitable quality metrics for 360 video quality evaluation. In this document, we propose Area Weighted Spherical PSNR (AW-SPSNR). Based on similar concept as WS-PSNR in MPEG document m38551, AW-SPSNR reportedly overcomes some of the shortcomings of SPSNR, and is therefore asserted to be a more suitable metric for 360 video quality evaluation. AW-SPSNR is implemented in the PCT360 tool described in JVET-D0021, and is implemented for a variety of projection formats, including equirectangular (ERP), equal-area (EAP), cubemap (CMP), and octahedron (OHP).

# Introduction

Spherical video may be presented in different projection formats on a 2D plane, for example, using equirectangular projection (ERP)[1], cubemap projection (CMP) [2], equal-area projection (EAP) [3], and octahedron projection (OHP) [4]. Implementations of these projection formats are available in the PCT360 software tool described in JVET-D0021 [5]. Depending on the specific projection format used to represent spherical video as 2D planar video, different samples on the projected 2D plane could correspond to different sampling densities on the sphere. For example, in ERP, the sampling density is infinite at the top and bottom of the 2D plane, which correspond to the North and South poles, respectively, on the sphere. For CMP, the sampling density is higher at the boundaries of the cube faces than at the centers of the cube faces. This scenario is different from a normal 2D video, where the sampling density is uniform across the 2D plane regardless of the sample location. Thus, for spherical videos, the conventional PSNR cannot provide a meaningful quality measurement, as it weights the distortion at each sample location uniformly. In order to measure the quality of spherical video without being affected by the problem of uneven sampling densities on the 2D plane, M. Yu et al. proposed the Spherical PSNR (SPSNR) metric for 360 video quality evaluation [6]. As shown in Figure 1, given a sampling point on the sphere, labeled as “s”, and SPSNR finds the corresponding sampling location in the projected 2D plane, and measure the distortion between the reference signal and test signal at that corresponding 2D plane sampling location. SPSNR calculates the distortion at a total of 655362 sample locations uniformly sampled on the sphere, and averages the distortion across the entire set of sample locations.

Figure 1. Spherical PSNR

In [6], Yu. et al. further considered the fact that  different samples on the sphere could have different viewing probabilities depending on their locations. For example, the samples close to the equator are generally more likely to be viewed as compared to the samples at the poles. Correspondingly, the authors also proposed Weighted Spherical PSNR (W-SPNSR) and Latitude Spherical PSNR (L-SPNSR) to take into account the viewing probability. Besides SPSNR, PCT360 also implements L-SPSNR.

Because SPSNR only evaluates 360 videos on a subset of points on the geometry, it neglects many samples for evaluation. In the current implementation, only 655362 samples are used. For a projected 2D video in 4K resolution, for example, 3840x1920, SPSNR evaluates distortion using less than 10% of all available samples (655362/3840x1920 = 8.9%). Such low ratio may not be suitable for all content. For example, the number of points at which we evaluate SPSNR should depend on the maximum frequency information contained in that content. However, we also do not want the number of points used in SPSNR calculation to become a function of the content to be evaluated, as this complicates the computation of quality metrics.

Another consideration in computing SPSNR is whether to apply interpolation. As shown in Figure 1, calculation of SPSNR starts from a point on to the sphere to a corresponding point in the projected 2D plane. If the point is not on the integer positions of the 2D sampling grid, then either sample value from the nearest neighbor is used, or interpolation needs to be applied to get the value at the mapped location. Neither is ideal: using nearest neighbor could be a crude approximation, whereas applying interpolation could introduce additional interpolation error in the metric evaluation, and introduce the impact due to specific choice of interpolation filters.

# Area Weighted SPSNR (AW-SPSNR)

In order to overcome the above shortcomings, we propose AW-SPSNR which considers the entire set of samples available in the projected 2D plane, and weighs the errors introduced at each sample location according to the solid angle covered by the corresponding sample on the unit sphere.

Dist=m=0M-1n=0N-1(wm,n*Dm,n)m=0M-1n=0N-1wm,n

Where Dist is the overall distortion over the entire 2D plane, and wm,n and Dm,nare the weight and distortion at the position (m,n), respectively.

In the following, we introduce how to calculate the weights wm,n used in AW-SPSNR for ERP, CMP, EAP, and OHP.

## AW-SPSNR for ERP

Let (m,n) be the coordinate location of the pixel on ERP. Its corresponding latitude and longitude position (ϕ, θ)can be obtained using the following formula:

| ϕ = (ue/W-0.5)·2π | ϕ = (m/W-0.5)·2π. θ = (0.5-n/H)·π. |  |
| --- | --- | --- |
| θ = (0.5-ve/H)·π |  |  |

Where W and H are the width and height of ERP picture, respectively. For this sample at (ϕ, θ), we can calculate that the area covered by the sample on the unit sphere given delta ϕ and delta θ as

| solid_angle(ϕ,θ)= cos(θ)·dθ·dϕ. |  |
| --- | --- |

Since dθ and dϕ is the same for all samples in ERP, since the latitude and longitude are evenly sampled, the corresponding weights wm,n is evaluated as:

| wm,n= cos(θ). |  |
| --- | --- |

## AW-SPSNR for CMP

Cubemap has six symmetric faces. The weights derived for one face can be used for all the six faces. The calculation is done for the face ABCD in Figure 2.Let x and y be the normalized coordinates of a point (m, n) on the face ABCD (x and y range from -1 to +1). The relation between (x,y) and (ϕ, θ) is as follows:

x = 2m/W - 1

y = 2n/H - 1

ϕ = tan-1(x)

θ =tan-1(y×costan-1x)

Since ϕ is a function of x alone, its derivative can be computed as:

dϕ =cos2(ϕ)dx

θ is a function of both x and y. We thus compute the partial derivatives ∂θ/∂x and ∂θ/∂y and then compute the total derivative. The partial derivatives are:

∂θ∂x= -sin(θ)cos(θ)sin(ϕ)cos(ϕ) ∂θ∂y=cos2(θ)cos(ϕ)

The total derivative can then be computed as:

dθ= (∂θ∂x)dx + (∂θ∂y)dy

However, we need the length of dθ. We compute the Euclidian norm assuming ∆x and ∆y to be equal as:

|dθ| = ∂θ∂x2 + ∂θ∂y212dx

The area or equivalently the weight of the sample is:

solid_angleϕ,θ=

cosθ×cos2(ϕ)sinθcosθsinϕcosϕ2+cos2θcosϕ212dxdy

In each cubemap face, x and y are evenly sampled. Therefore dx and dy are the same for each point. The weight for the point (m, n) is evaluated as:

wm,n=cosθ×cos2(ϕ)sinθcosθsinϕcosϕ2+cos2θcosϕ212

Figure 2. Sphere geometry representation with CMP

## AW-SPSNR for EAP

Let (m, n) be the pixel coordinates on the EAP frame. The geometry mapping between the pixel coordinates and the position on unit sphere is as follows:

ϕ = (m/W-0.5)·2π

θ = sin-1(20.5-nH)

The derivatives of ϕ and θ are computed as:

dϕ =(1/W)∙ 2π∙dm

dθ =-(2/(H∙cosθ)∙dn

Correspondingly the solid angle or equivalently the area spanned by the sample at (m, n) is:

solid_angle(m, n) = (dm∙dn ∙π )/(W∙H)

We observe that m and n are equally sampled. Thus the area is same for all the samples. Therefore, for EAP, AW-SPSNR is essentially the same as PSNR.

## AW-SPSNR for OHP

The weights used in AW-SPSNR for a face in OHP is essentially the same as those used for a face in CMP, except that in OHP, each face is a triangle, and the samples on each OHP face correspond to a subset of samples on a CMP square face. Therefore, a subset of weights that corresponds to the subset of samples on a CMP face is used in AW-SPSNR calculation for OHP face.

# WS-PSNR in m38551

In the Geneva meeting, Zhejiang University proposed a metric called uniform Weighted Spherical PSNR (WS-PSNR) for 360 video quality evaluation in MPEG document m38551 [6]. Proposed independently, WS-PSNR and AW-SPSNR are in fact based on the same concept. Table 1 summarizes how each metric is implemented for each projection format.

Table 1. Comparison of WS-PSNR and AW-SPSNR

| Projection format | WS-PSNR | AW-SPSNR | Notes |
| --- | --- | --- | --- |
| ERP | 360tools*, standalone | PCT360 | Same weights are used. Results essentially the same among different implementations |
| CMP | Standalone | PCT360 | Weights are derived differently, but values are very similar. |
| EAP | NA | PCT360 |  |
| OHP | NA | PCT360 |  |

* 360tools is from Samsung, available at https://github.com/Samsung/360tools

Figure 3 and Figure 4 show the weights assigned to each sample on a square CMP face with dimension 720x720, for AW-SPSNR and WS-PSNR. As shown, they generally look very similar, with the center of the face being assigned higher weights(that is, each sample toward the center of the face corresponds to a bigger area on the sphere), the boundaries of the face being assigned lower weights (that is, each sample on the face boundary corresponds to a smaller area on the sphere), and the four corners of the face being assigned the lowest weights (that is, the four corner samples correspond to the smallest area on the sphere). Figure 5 shows the weight difference between WS-PSNR and AW-SPSNR in numerical values for the same 720x720 CMP face. The difference is very small, with the biggest difference being on the order of 10-8.

Figure 3. Area weights for a 720x720 cube face in AW-SPSNR

Figure 4. Area weight for a 720x720 cube face in WS-PSNR [6]

Figure 5.Difference of two weight maps between AW-SPSNR and WS-PSNR

# Experimental results

As our main motivation for proposing AW-SPSNR is to keep with SPSNR’s main spirit (i.e., quality evaluation based on spherical coordinates) but try to overcome its shortcomings (i.e., sparse sampling and potential need for interpolation), we evaluate the AW-SPSNR metric by comparing it with SPSNR. We use the PCT360 tool [5] to generate results for both metrics. Our test workflow is depicted in Figure 6. This test workflow follows the suggested test methodology in [8]. The only difference is that, since the focus here is to evaluate how the metrics behave for each projection format, the metrics are calculated between original and reconstructed signal in different coding geometries, including ERP, CMP, EAP, and OHP.

In our simulation, we use the set of six new 8K ERP test sequences released by GoPro [9] and two 8K ERP sequences from ETRI. We perform conversion, coding, and back conversion on one GOP, that is, 17 frames, of each test sequence using the Random Access configuration. For coding, we use HM16.12.

Figure 6. Workflow used to collect quality metric (SPSNR and AW-SPSNR) results

Table 2, Table 3, Table 4, and Table 5 show the SPSNR and AW-SPSNR results for ERP, CMP, EAP, and OHP, respectively. Only geometry conversion is applied; coding is not applied. As shown, for ERP, CMP, and EAP projection formats, the AW-SPSNR values track very closely the SPSNR values for all sequences (usually 0.01-0.02 dB difference). For OHP, the AW-SPSNR values and the SPSNR values differ more, by up to ~0.5 dB. Based on this observation, and that fact that AW-SPSNR overcomes both shortcomings of SPSNR discussed earlier (that is, no sparse sampling problem, and no need to consider interpolation), it is proposed to use AW-SPSNR as a 360 video quality metric for future exploration of 360 video coding. The fact that WS-PSNR was proposed independently based on the same concept also serves as evidence that using the relative spherical area to weight the distortion at each sample location on the 2D plane should be considered a sensible approach.

Table 2. SPSNR and AW-SPSNR metrics, coding geometry set to CMP

|  |  |  | SPSNR |  |  | AW-SPSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP Slice | kbps | Y | U | V | Y | U | V |
| Basketball | 22 | 17295.4 | 42.50 | 53.30 | 52.53 | 42.52 | 53.34 | 52.57 |
|  | 27 | 8556.3 | 39.63 | 51.21 | 51.01 | 39.64 | 51.25 | 51.05 |
|  | 32 | 4149.5 | 36.75 | 49.08 | 49.38 | 36.77 | 49.13 | 49.42 |
|  | 37 | 1711.6 | 34.09 | 47.72 | 48.38 | 34.12 | 47.76 | 48.41 |
| Chairlift | 22 | 27776.1 | 42.40 | 48.69 | 49.75 | 42.42 | 48.70 | 49.76 |
|  | 27 | 13168.4 | 39.41 | 46.44 | 48.22 | 39.44 | 46.45 | 48.22 |
|  | 32 | 6219.9 | 36.61 | 44.70 | 46.92 | 36.64 | 44.71 | 46.92 |
|  | 37 | 2948.9 | 34.18 | 43.53 | 45.91 | 34.22 | 43.54 | 45.91 |
| NewLot | 22 | 32506.2 | 42.65 | 49.48 | 50.37 | 42.68 | 49.51 | 50.39 |
|  | 27 | 13809.0 | 39.42 | 47.31 | 48.59 | 39.44 | 47.35 | 48.62 |
|  | 32 | 6067.2 | 36.75 | 45.34 | 46.89 | 36.78 | 45.38 | 46.92 |
|  | 37 | 2903.0 | 34.55 | 44.03 | 45.72 | 34.58 | 44.09 | 45.75 |
| Jam | 22 | 4753.5 | 47.42 | 51.56 | 51.73 | 47.42 | 51.54 | 51.72 |
|  | 27 | 2487.9 | 45.01 | 49.34 | 49.83 | 45.02 | 49.33 | 49.83 |
|  | 32 | 1367.5 | 42.38 | 47.20 | 47.91 | 42.39 | 47.19 | 47.91 |
|  | 37 | 772.3 | 39.66 | 45.73 | 46.51 | 39.68 | 45.73 | 46.52 |
| Train | 22 | 17169.0 | 45.29 | 51.51 | 52.10 | 45.30 | 51.52 | 52.11 |
|  | 27 | 9653.2 | 42.36 | 49.17 | 49.77 | 42.39 | 49.19 | 49.78 |
|  | 32 | 5303.3 | 39.39 | 47.23 | 47.79 | 39.40 | 47.26 | 47.80 |
|  | 37 | 2749.6 | 36.51 | 45.77 | 46.41 | 36.53 | 45.81 | 46.42 |
| Trick | 22 | 46985.9 | 44.04 | 50.88 | 51.79 | 44.01 | 50.86 | 51.79 |
|  | 27 | 21155.2 | 40.99 | 48.58 | 49.70 | 40.97 | 48.56 | 49.70 |
|  | 32 | 9682.2 | 38.14 | 46.57 | 47.76 | 38.12 | 46.54 | 47.75 |
|  | 37 | 4612.5 | 35.68 | 45.14 | 46.60 | 35.65 | 45.13 | 46.60 |
| Dancing | 22 | 7491.2 | 45.11 | 50.92 | 49.49 | 45.11 | 50.92 | 49.48 |
|  | 27 | 3824.4 | 43.29 | 49.33 | 48.16 | 43.28 | 49.33 | 48.14 |
|  | 32 | 2013.4 | 40.98 | 47.77 | 46.86 | 40.99 | 47.78 | 46.86 |
|  | 37 | 1059.1 | 38.56 | 46.62 | 45.85 | 38.55 | 46.61 | 45.84 |
| Driving | 22 | 4852.9 | 46.10 | 50.58 | 50.23 | 46.15 | 50.61 | 50.24 |
|  | 27 | 2285.0 | 43.99 | 49.31 | 49.12 | 44.06 | 49.34 | 49.13 |
|  | 32 | 1236.3 | 41.63 | 48.07 | 47.98 | 41.70 | 48.10 | 47.99 |
|  | 37 | 678.9 | 39.08 | 46.80 | 46.81 | 39.13 | 46.83 | 46.83 |

Table 3. SPSNR and AW-SPSNR metrics, coding geometry set to ERP

|  |  |  | SPSNR |  |  | AW-SPSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP Slice | kbps | Y | U | V | Y | U | V |
| Basketball | 22 | 18247.1 | 42.50 | 53.14 | 52.44 | 42.49 | 53.17 | 52.47 |
|  | 27 | 8792.8 | 39.54 | 50.97 | 50.88 | 39.53 | 51.00 | 50.90 |
|  | 32 | 4266.6 | 36.72 | 48.91 | 49.26 | 36.72 | 48.93 | 49.27 |
|  | 37 | 1863.8 | 34.20 | 47.42 | 48.15 | 34.21 | 47.45 | 48.16 |
| Chairlift | 22 | 27532.6 | 42.16 | 48.46 | 49.62 | 42.16 | 48.46 | 49.62 |
|  | 27 | 12876.7 | 39.08 | 46.24 | 48.05 | 39.09 | 46.23 | 48.04 |
|  | 32 | 5914.1 | 36.24 | 44.44 | 46.71 | 36.25 | 44.44 | 46.71 |
|  | 37 | 2728.9 | 33.83 | 43.34 | 45.77 | 33.84 | 43.34 | 45.76 |
| NewLot | 22 | 32901.0 | 42.60 | 49.28 | 50.23 | 42.60 | 49.29 | 50.24 |
|  | 27 | 13556.8 | 39.29 | 47.08 | 48.37 | 39.29 | 47.09 | 48.38 |
|  | 32 | 5582.3 | 36.54 | 45.04 | 46.70 | 36.54 | 45.06 | 46.71 |
|  | 37 | 2521.3 | 34.31 | 43.80 | 45.48 | 34.31 | 43.82 | 45.50 |
| Jam | 22 | 4418.2 | 47.11 | 51.18 | 51.32 | 47.10 | 51.16 | 51.32 |
|  | 27 | 2331.2 | 44.69 | 48.93 | 49.40 | 44.68 | 48.91 | 49.39 |
|  | 32 | 1281.2 | 42.01 | 46.85 | 47.52 | 42.00 | 46.83 | 47.52 |
|  | 37 | 723.7 | 39.30 | 45.33 | 46.10 | 39.27 | 45.29 | 46.09 |
| Train | 22 | 14633.6 | 45.12 | 51.14 | 51.71 | 45.16 | 51.17 | 51.71 |
|  | 27 | 8332.9 | 42.12 | 48.88 | 49.42 | 42.13 | 48.89 | 49.42 |
|  | 32 | 4539.0 | 39.00 | 46.90 | 47.50 | 39.01 | 46.91 | 47.51 |
|  | 37 | 2274.2 | 36.02 | 45.52 | 46.23 | 36.04 | 45.53 | 46.23 |
| Trick | 22 | 37543.0 | 43.76 | 50.49 | 51.44 | 43.73 | 50.47 | 51.43 |
|  | 27 | 17009.4 | 40.59 | 48.20 | 49.28 | 40.56 | 48.17 | 49.27 |
|  | 32 | 7744.5 | 37.69 | 46.13 | 47.37 | 37.66 | 46.09 | 47.34 |
|  | 37 | 3619.5 | 35.22 | 44.77 | 46.06 | 35.19 | 44.74 | 46.05 |
| Dancing | 22 | 6574.8 | 45.17 | 50.81 | 49.44 | 45.18 | 50.79 | 49.44 |
|  | 27 | 3488.6 | 43.24 | 49.15 | 48.05 | 43.22 | 49.14 | 48.05 |
|  | 32 | 1824.9 | 40.77 | 47.54 | 46.71 | 40.75 | 47.55 | 46.70 |
|  | 37 | 951.8 | 38.23 | 46.34 | 45.70 | 38.22 | 46.34 | 45.69 |
| Driving | 22 | 4265.0 | 45.98 | 50.42 | 50.15 | 46.00 | 50.43 | 50.14 |
|  | 27 | 2040.3 | 43.76 | 49.15 | 48.97 | 43.77 | 49.16 | 48.97 |
|  | 32 | 1091.4 | 41.30 | 47.79 | 47.84 | 41.32 | 47.79 | 47.82 |
|  | 37 | 592.4 | 38.67 | 46.96 | 46.71 | 38.69 | 46.96 | 46.72 |

Table 4. SPSNR and AW-SPSNR metrics, coding geometry set to EAP

|  |  |  | SPSNR |  |  | AW-SPSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP Slice | kbps | Y | U | V | Y | U | V |
| Basketball | 22 | 18020.8 | 42.86 | 53.59 | 52.70 | 42.87 | 53.60 | 52.71 |
|  | 27 | 8895.0 | 39.90 | 51.44 | 51.15 | 39.90 | 51.45 | 51.16 |
|  | 32 | 4303.6 | 37.04 | 49.39 | 49.43 | 37.04 | 49.40 | 49.43 |
|  | 37 | 1849.1 | 34.47 | 47.90 | 48.41 | 34.48 | 47.91 | 48.42 |
| Chairlift | 22 | 29754.8 | 42.56 | 48.70 | 49.74 | 42.56 | 48.70 | 49.74 |
|  | 27 | 14024.3 | 39.54 | 46.43 | 48.19 | 39.55 | 46.42 | 48.19 |
|  | 32 | 6560.0 | 36.72 | 44.65 | 46.83 | 36.73 | 44.64 | 46.82 |
|  | 37 | 3078.4 | 34.29 | 43.51 | 45.91 | 34.30 | 43.51 | 45.90 |
| NewLot | 22 | 34517.6 | 43.04 | 49.82 | 50.58 | 43.04 | 49.83 | 50.59 |
|  | 27 | 14644.2 | 39.85 | 47.69 | 48.81 | 39.85 | 47.70 | 48.82 |
|  | 32 | 6422.0 | 37.18 | 45.73 | 47.18 | 37.19 | 45.75 | 47.19 |
|  | 37 | 3094.5 | 34.99 | 44.44 | 46.00 | 34.99 | 44.45 | 46.01 |
| Jam | 22 | 5192.7 | 47.58 | 51.72 | 51.80 | 47.57 | 51.70 | 51.80 |
|  | 27 | 2767.2 | 45.30 | 49.53 | 49.95 | 45.30 | 49.52 | 49.95 |
|  | 32 | 1535.0 | 42.76 | 47.48 | 48.09 | 42.74 | 47.46 | 48.08 |
|  | 37 | 875.6 | 40.07 | 45.97 | 46.74 | 40.05 | 45.95 | 46.73 |
| Train | 22 | 17146.5 | 45.53 | 51.64 | 52.18 | 45.55 | 51.66 | 52.18 |
|  | 27 | 9682.4 | 42.66 | 49.43 | 49.98 | 42.68 | 49.44 | 49.98 |
|  | 32 | 5322.3 | 39.66 | 47.51 | 48.03 | 39.68 | 47.52 | 48.03 |
|  | 37 | 2736.4 | 36.73 | 46.20 | 46.69 | 36.75 | 46.21 | 46.69 |
| Trick | 22 | 45610.4 | 44.22 | 51.03 | 51.90 | 44.20 | 51.00 | 51.88 |
|  | 27 | 20730.9 | 41.19 | 48.79 | 49.81 | 41.16 | 48.77 | 49.79 |
|  | 32 | 9514.2 | 38.35 | 46.80 | 48.01 | 38.31 | 46.77 | 48.00 |
|  | 37 | 4542.0 | 35.88 | 45.44 | 46.73 | 35.85 | 45.41 | 46.71 |
| Dancing | 22 | 7873.3 | 45.35 | 51.08 | 49.64 | 45.34 | 51.05 | 49.63 |
|  | 27 | 4118.7 | 43.58 | 49.49 | 48.30 | 43.57 | 49.48 | 48.30 |
|  | 32 | 2182.1 | 41.30 | 47.89 | 47.00 | 41.30 | 47.88 | 47.00 |
|  | 37 | 1164.6 | 38.88 | 46.82 | 45.98 | 38.86 | 46.82 | 45.97 |
| Driving | 22 | 5309.4 | 46.45 | 50.77 | 50.36 | 46.46 | 50.78 | 50.36 |
|  | 27 | 2473.2 | 44.42 | 49.55 | 49.30 | 44.45 | 49.56 | 49.30 |
|  | 32 | 1323.3 | 42.14 | 48.27 | 48.23 | 42.17 | 48.27 | 48.22 |
|  | 37 | 735.8 | 39.62 | 47.29 | 47.27 | 39.64 | 47.30 | 47.26 |

Table 5. SPSNR and AW-SPSNR metrics, coding geometry set to OHP

|  |  |  | SPSNR |  |  | AW-SPSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP Slice | kbps | Y | U | V | Y | U | V |
| Basketball | 22 | 18413.9 | 42.60 | 53.41 | 52.55 | 42.71 | 53.24 | 52.35 |
|  | 27 | 9217.4 | 39.71 | 51.25 | 51.04 | 39.81 | 51.05 | 50.86 |
|  | 32 | 4515.1 | 36.83 | 49.25 | 49.42 | 36.93 | 49.02 | 49.25 |
|  | 37 | 1984.5 | 34.31 | 47.70 | 48.36 | 34.33 | 47.51 | 48.15 |
| Chairlift | 22 | 29088.9 | 42.34 | 48.47 | 49.63 | 42.68 | 48.73 | 49.77 |
|  | 27 | 13772.6 | 39.27 | 46.20 | 48.02 | 39.69 | 46.46 | 48.20 |
|  | 32 | 6540.4 | 36.42 | 44.42 | 46.65 | 36.90 | 44.71 | 46.89 |
|  | 37 | 3141.8 | 34.02 | 43.26 | 45.69 | 34.50 | 43.55 | 45.95 |
| NewLot | 22 | 39277.5 | 42.95 | 49.74 | 50.50 | 42.77 | 49.39 | 50.21 |
|  | 27 | 17333.0 | 39.90 | 47.60 | 48.74 | 39.61 | 47.16 | 48.36 |
|  | 32 | 7874.0 | 37.29 | 45.65 | 47.02 | 36.91 | 45.21 | 46.64 |
|  | 37 | 3882.5 | 35.08 | 44.33 | 45.92 | 34.66 | 43.91 | 45.55 |
| Jam | 22 | 5585.3 | 47.38 | 51.62 | 51.71 | 47.80 | 51.78 | 51.91 |
|  | 27 | 3094.7 | 45.08 | 49.40 | 49.77 | 45.47 | 49.58 | 49.95 |
|  | 32 | 1789.8 | 42.54 | 47.37 | 47.97 | 42.88 | 47.54 | 48.15 |
|  | 37 | 1018.2 | 39.84 | 45.87 | 46.54 | 40.07 | 46.03 | 46.74 |
| Train | 22 | 18305.7 | 45.21 | 51.41 | 51.83 | 45.55 | 51.44 | 51.89 |
|  | 27 | 10101.7 | 42.33 | 49.06 | 49.53 | 42.67 | 48.85 | 49.56 |
|  | 32 | 5471.2 | 39.34 | 47.12 | 47.66 | 39.71 | 46.91 | 47.73 |
|  | 37 | 2830.4 | 36.41 | 45.81 | 46.41 | 36.82 | 45.53 | 46.45 |
| Trick | 22 | 47794.6 | 43.95 | 50.81 | 51.66 | 44.16 | 50.86 | 51.73 |
|  | 27 | 21469.3 | 40.93 | 48.47 | 49.52 | 41.14 | 48.37 | 49.45 |
|  | 32 | 9862.3 | 38.14 | 46.46 | 47.64 | 38.36 | 46.39 | 47.56 |
|  | 37 | 4738.4 | 35.72 | 45.11 | 46.32 | 35.92 | 45.10 | 46.28 |
| Dancing | 22 | 8750.5 | 45.14 | 50.92 | 49.50 | 45.29 | 50.95 | 49.48 |
|  | 27 | 4629.5 | 43.39 | 49.27 | 48.13 | 43.50 | 49.27 | 48.06 |
|  | 32 | 2538.9 | 41.19 | 47.70 | 46.79 | 41.25 | 47.69 | 46.68 |
|  | 37 | 1418.6 | 38.76 | 46.61 | 45.78 | 38.78 | 46.60 | 45.64 |
| Driving | 22 | 6716.4 | 46.40 | 50.69 | 50.32 | 46.19 | 50.62 | 50.27 |
|  | 27 | 3324.4 | 44.46 | 49.40 | 49.18 | 44.20 | 49.20 | 49.08 |
|  | 32 | 1938.6 | 42.18 | 48.25 | 48.07 | 41.90 | 47.91 | 47.92 |
|  | 37 | 1181.8 | 39.63 | 47.10 | 47.07 | 39.23 | 46.75 | 46.88 |

# References

- Equirectangular projection, https://en.wikipedia.org/wiki/Equirectangular_projection

- Cubemap, https://en.wikipedia.org/wiki/Cube_mapping

- Lambert cylindrical equal-area projection, https://en.wikipedia.org/wiki/Lambert_cylindrical_equal-area_projection

- Octahedron, https://en.wikipedia.org/wiki/Octahedron

- Y. He, B. Vishwanath, X. Xiu, Y. Ye, “AHG8: InterDigital's projection format conversion tool”, JVET-D0021, Oct. 2016, Chengdu, China.

- Y. Sun, A. Lu, L. Yu, “[FTV-AHG] WS-PSNR for 360 video quality evaluation”, MPEG document no m38551, June 2016, Geneva, Switzerland.

- M. Yu, H. Lakshman, B. Girod, “A Framework to Evaluate Omnidirectional Video Coding Schemes”, IEEE International Symposium on Mixed and Augmented Reality, 2015.

- B. Vishwanath,Y. He, X. Xiu, Y. Ye, “AHG8: Suggested test methodology for evaluating projection formats for 360 video coding”, JVET-D0074, Oct. 2016, Chengdu, China.

- A. Abbas, B. Adsumilli, “New GoPro Test Sequences for Virtual Reality Video Coding”, JVET-D0026, Oct. 2016, Chengdu, China.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).