| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0097 |
| --- | --- |

| Title: | EE8: Decoder-side intra mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Exploration Experiment (EE) 8, which integrates decoder-side intra mode derivation (DIMD) on top of HM16.6-JEM-3.0. The DIMD approach derives the intra prediction mode and reduces the overhead of intra mode signaling. Further, because no mode signaling is needed in DIMD, intra prediction can be performed at higher granularity than in JEM. One flag is signaled for each quad-tree plus binary tree (QTBT) leaf node to indicate the usage of DIMD. It is reported that compared to HM16.6-JEM-3.0 anchor, the proposed method achieves on average 0.78% and 0.48% BD-rate savings for AI and RA configurations.

# Introduction

In HM16.6-JEM-3.0 [1] intra modes are extended to 67 from 35 modes in HEVC [2], and they are derived at encoder and explicitly signaled to decoder. A significant amount of overhead is spent on intra mode coding. To reduce the overhead of intra mode signaling, a decoder-side intra mode derivation (DIMD) approach is proposed in [3]. Instead of signaling intra mode explicitly, the DIMD derives the information at both encoder and decoder from the neighboring reconstructed samples of one block. It is included in Exploration Experiments (EEs) [4] as EE8 for a thorough testing and rigorous evaluation. This document reports the simulation results of the DIMD on top of the HM-16.6-JEM-3.0.

# Technical description

## DIMD-based intra coding

Figure 1 illustrates the process of deriving intra mode by the DIMD. In Figure 1, the target denotes the current block (of block size N) whose intra mode is to be estimated. The template (indicated by the patterned region in Figure 1) specifies a set of already reconstructed samples, which are used to derive the intra mode. The template size is denoted as the number of samples within the template that extends to the above and the left of the target block, i.e., L. By default, a template size of 2 (i.e., L=2) is used for blocks for which either width or height is not larger than 8 and a template size of 4 (i.e., L=4) is used for the other block sizes. The reference of template (indicated by the dotted region in Figure 1) refers to a set of neighboring samples from above and left of the template. For each intra prediction mode, the DIMD calculates the absolute difference (SAD) between the reconstructed template samples and its prediction samples obtained from the reference samples of the template. The intra prediction mode that yields the minimum SAD is selected as the final intra prediction mode of the target block.

Figure 1 Target samples, template samples and the reference samples of template used in the DIMD.

The DIMD is used as one additional intra mode which is adaptively selected by comparing the DIMD intra mode with the optimal normal intra mode (i.e., being explicitly signaled). For P and B slices, one DIMD flag is signaled for each quad-tree plus binary tree (QTBT) leaf node. If the flag is one, then the block is predicted using the intra mode derived by DIMD; otherwise, the DIMD is not applied and the block is predicted using the intra mode explicitly signaled in the bit-stream. When the DIMD is enabled, chroma components always reuse the same intra mode as that derived for luma component, i.e., DM mode. For I slice, the signaling of the DIMD flag is also synchronized with QTBT leaf nodes except that the maximum binary tree depth (BTDepth) of signaling the DIMD flag is restricted to be one. Additionally, when the DIMD is enabled, the derived intra mode is only applied to luma component while the intra mode of chroma components are always explicitly signaled.

As no mode signaling is needed in DIMD, the number of angular directions for DIMD-coded blocks is increased to 129, and the DC and planar modes still remain the same. To accommodate the increased granularity of angular intra modes, the precision of intra interpolation filtering for DIMD-coded CUs increases from 1/32-pel to 1/64-pel. Additionally, in order to use the derived intra mode of a DIMD coded block as MPM candidate for neighboring intra blocks, those 129 directions of the DIMD-coded CUs are converted to “normal” intra modes (i.e., 65 angular intra directions) before they are used as MPM.

## DIMD search algorithm

In order to reduce encoding/decoding complexity, one fast intra mode search algorithm is used for DIMD. Firstly, the intra modes of five spatial neighbors, which are acquired in the same way of merge mode, are checked. If all the spatial candidates are coded by DC or Planar, then only DC and Planar will be considered as the search candidates to find the optimal DIMD intra mode. Otherwise, one initial estimation process is performed to provide a starting point for DIMD search. Specifically, an initial candidate list is created by selecting N fixed modes from the allowed intra modes. Then, the SAD is calculated for all the candidate intra modes and the one that minimizes the SAD is selected as the starting intra mode. To achieve a good complexity/performance trade-off, the initial candidate list consists of DC, planar and every 4-th mode of the 33 angular intra directions as defined in HEVC [2]. If the starting intra mode is either DC or planar, it is used as the optimal DIMD mode. Otherwise (i.e., the starting intra mode is one angular mode), one refinement process is then applied where the optimal intra mode is identified through one iterative search process.

# Simulation results

The proposed method has been implemented on top of HM16.6-JEM-3.0 which can be download from https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-3.0-EE8-DIMD/. All the simulations are conducted by following the JVET common test conditions (CTCs). The encoding/decoding running time are not reliable due to inconsistent cores.

To validate the performance of the DIMD, seven sets of tests are conducted on top of HM16.6-JEM-3.0 as described in [4]. For all the tests, the results of HM-16.6-JEM-3.0 are used as anchor.

- Test #1: DIMD with template size L = 1 for all the block sizes

- Test #2: DIMD with template size L = 2 for all the block sizes

- Test #3: DIMD with template size depending on the block size

- Test #4: Test #3 but disable DIMD for bocks which contain less than 64 samples

- Test #5: Test #3 but use the same number of intra modes as HM-16.6-JEM-3.0.

- Test #6: HM-16.6-JEM-3.0 with similar encoding complexity as Test #3. This is done by increasing the number of intra modes that are used for the RD calculation in the HM16.6-JEM-3.0 by three times.

- Test #7: Test #3 plus Test #6

Table I. DIMD with template size L = 1 for all the block sizes

Table II. DIMD with template size L = 2 for all the block sizes

Table III. DIMD with template size depending on the block size

Table IV. Disabled DIMD for bocks which contain less than 64 samples

Table V. DIMD with the same number of intra modes as HM-16.6-JEM-3.0

Table VI. HM-16.6-JEM-3.0 encoder with increased number of intra modes for RD calculation

Table VII. DIMD plus HM-16.6-JEM-3.0 encoder with increased number of intra modes for RD calculation

# References

- J. Chen, E. Alshina, G.-J. Sullivan, J.-R. Ohm, J, Boyce, “Algorithm description of joint exploration test mode 3 (JEM3),” JVET-C1001, Geneva, Switzerland, June, 2016.

- High Efficiency Video Coding, Rec. ITU-T H.265 and ISO/IEC 23008-2, Jan. 2013.

- X. Xiu, Y. He, Y. Ye, “Decoder-side intra mode derivation,” JVET-C0061, Geneva, Switzerland, June, 2016.

- E. Alshina, J. Boyce, Y.-J. Chang, S.-H. Kim, V. Seregin, X. Xiu, “Description of exploration experiments on coding tools” JVET-C1011, Geneva, Switzerland, June, 2016.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, San Diego, USA, Feb. 2016.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).