| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0142 |
| --- | --- |

| Title: | AHG8: An efficient compact layout for octahedron format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hung-Chih Lin, Chia-Ying Li, Jian-Liang Lin, Shen-Kai Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Hung-Chih Lin +886-3-5670766 ext. 24349 {hungchih.lin, erica.li, jl.lin, shenkai.chang, cc.ju} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes an efficient compact layout for octahedron format with the minimum discontinuous edges. The compression efficiency between the ROHP layout and the proposed layout is examined by using HM16.12. The experimental results reportedly demonstrate that, as compared to the ROHP layout, the BD-rate reduction provided by the proposed layout achieves 5.54% in AI, 4.73% in RA, 5.59% in LD-B and, 5.67% in LD-P, respectively.

Introduction

The 360VR conversion tool [1] proposed by Samsung provides three types of regular polyhedron formats. One of these formats is the octahedron format, which is composed of eight equilateral triangle faces. In order to avoid blank area, this conversion tool [1] supports a compact layout, ROHP. As shown in Fig. 1, the ROHP layout has eight discontinuous edges. Moreover, the 360VR video content represented by this layout may not be well arranged.

Fig. 1 ROHP layout

As analyzed in [2], a proper layout arrangement that maximally achieve continuity between faces could have superior compression efficiency. Based on this principle, we rearrangement these eight faces to minimize the discontinuous edges as illustrated in Fig. 2. In addition, Fig. 3 depicts an example of the proposed layout.

| (a) |  |
| --- | --- |
| (b) | (c) |

Fig. 2 (a) Rearrangement step 1 (b) Rearrangement step 2 (c) Final proposed layout

Fig. 3 An example of proposed layout

Test Conditions

There are ten 360VR video test sequences provided to JVET by GoPro [3] and Nokia [4]. These test sequences are represented in equirectangular format. We use the Samsung conversion software [1] to convert the test sequences from 8-bit equirectangular format to 10-bit ROHP format. Moreover, we implement our proposed layout by modifying the Samsung conversion tool software [1]. The area of these two octahedron formats are the same. The detailed information is listed in Table I. Moreover, the reference software is HM16.12 and the test conditions are the Main-10 reference configurations specified in [5].

Table I. List of 360VR test sequence and resolution

| Source | YUV Sequence | Resolution |  |  |
| --- | --- | --- | --- | --- |
|  |  | ERP | Octahedron |  |
|  |  |  | ROHP layout | Proposed layout |
| GoPro | abyss_great_shark_vr | 3840x1920 | 5984x1296 | 2992x2592 |
|  | bicyclist_vr | 3840x1920 | 5984x1296 | 2992x2592 |
|  | glacier_vr | 3840x1920 | 5984x1296 | 2992x2592 |
|  | paramotor_training_vr | 3840x1920 | 5984x1296 | 2992x2592 |
|  | skateboarding_vr | 4096x2048 | 6352x1376 | 3176x2752 |
|  | timelapse_basejump_vr | 3840x1920 | 5984x1296 | 2992x2592 |
|  | timelapse_building_vr | 3840x1920 | 5984x1296 | 2992x2592 |
| Nokia | BearAttack-equirect-left | 3840x1920 | 5984x1296 | 2992x2592 |
|  | GT_Sheriff-equirect-left | 4320x2160 | 6688x1448 | 3344x2896 |
|  | LRRH-equirect-left | 3840x1920 | 5984x1296 | 2992x2592 |

Experimental Results

The experimental results of BD-rate comparison computed in ERP domain are listed in Table II. The BD-rate reduction provided by the proposed layout achieves 5.54% in AI, 4.73% in RA, 5.59% in LD-B and, 5.67% in LD-P, respectively. In octahedron format, a video encoder usually uses finer partitions, which consumes more bits, to minimize coding error around the discontinuous edges. The improvement in compression efficiency is thus expected because the proposed layout has the minimum discontinuous edges and is more suitable for video compression.

Table II. Simulation results (in ERP with PSNR metric)

|  | All Intra Main10 |  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| GoPro | -6.50% | -15.50% | -13.52% | GoPro | -6.14% | -18.71% | -15.07% |
| Nokia | -3.29% | -6.67% | -6.03% | Nokia | -4.29% | -10.27% | -10.65% |
| Overall | -5.54% | -12.85% | -11.27% | Overall | -5.59% | -16.18% | -13.74% |
|  | Random Access Main10 |  |  |  | Low Delay P Main10 |  |  |
|  | Y | U | V |  | Y | U | V |
| GoPro | -5.44% | -16.78% | -15.30% | GoPro | -6.25% | -18.42% | -14.12% |
| Nokia | -3.07% | -6.54% | -6.25% | Nokia | -4.31% | -10.53% | -10.85% |
| Overall | -4.73% | -13.71% | -12.59% | Overall | -5.67% | -16.06% | -13.14% |
| Anchor: ROHP layout; Tested: Proposed layout |  |  |  |  |  |  |  |

Conclusion

The proposed compact layout for octahedron format has the minimum discontinuous edges. The experimental data also report that the coding performance of the proposed layout is noticeably superior to that of the ROHP layout.

References

- 360tools. Projection and quality evaluation tools for VR video compression exploration experiments (https://github.com/Samsung/360tools)

- H.-C. Lin, J.-L. Lin, S.-K. Chang, and C.-C. Ju, “AHG8: Compact cube layout with tile partition,” Document of Joint Video Exploration Team, JVET-D0104, October, 2016

- A. Abbas, “GoPro test sequences for Virtual Reality video coding,” Document of Joint Video Exploration Team, JVET-C0021, May, 2016

- J. Ridge, M. M. Hannuksela, E. B. Aksu, J. Lainema, and A. Aminlou, “Nokia test sequences for virtual reality video coding,” Document of Joint Video Exploration Team, JVET-C0064, May, 2016

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations,” Document of Joint Video Exploration Team, JVET-B1010, February, 2016

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).