| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0186 |
| --- | --- |

| Title: | PSNR unification for sequential and parallel results for JVET CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mullakhmetov Ruslan, Sychev Maxim Moscow, Russia | Tel: Email: | +7-925-008-1124 Mullakhmetov.Ruslan@huawei.com Sychev.Maxim@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

Current version of Reference SW allow sequential and parallel simulation, but has some deviations from corresponding sequential simulation due to rounding in logs, most of real values are shown with just 4 digit after point. This contribution provides additional full precision representation of PSNR, bit-rate in logs and adopts aggregation scripts from JVET-B0036 to use this metrics, what removes any rounding steps in parallel simulation results calculation. At the previous JVET meeting the contribution JVET-C0037 was proposed to obtain the same PSNR results. During AhG discussions it was suggested to review and discuss PSNR unification at this meeting to get the clear decision.

# Proposed solution

## Previous solution

New information was added to JEM logs under summary sections containing PSNR, MSE, bitrate in full hexadecimal representation of IEEE 754 double floating point format, this is how JEM internally store this metrics, so no rounding take place. Example of this precise metrics see below.

SUMMARY --------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

49    a     100.3837   32.3969   38.8336   37.9851   33.6056

I Slices--------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

2    i    1267.2000   34.9092   39.0088   38.9925   35.8911

P Slices--------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

0    p      -1.#IND   -1.#IND   -1.#IND   -1.#IND   -1.#IND

B Slices--------------------------------------------------------

Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR

47    b      50.7319   32.2900   38.8262   37.9422   33.5307

Precise All --------------------------------------------------------

Total Frames |          Bitrate           Y-PSNR           U-PSNR           V-PSNR          YUV-MSE MAXVAL

49    x 4059188e1b2ad740 404032ce018ba25b 40436ab4f07358f9 4042fe17e70da657 407c58f9e1c2ae75   1020

Precise Excluding First --------------------------------------------------------

Total Frames |          Bitrate           Y-PSNR           U-PSNR           V-PSNR          YUV-MSE MAXVAL

48    x 40528c4444444444 40402cb4a8432814 40436ad719958b38 4042fbf1896f6edc 407c924037237238   1020

There are two types of metrics, one for all frames and one for all but first, to take into account segment intersection on IDR-frames with exception on first segment (see JVET-B0036). Aggregation scripts are modified to use this precise values and calculate overall metrics with full precision. The only possible deviation with sequential results can come from non-associative nature of IEEE 754 arithmetic, i.e. (a + b) + c != a + (b + c), which is negligible.

It should be noted that methods of calculation per-component and total YUV sequence PSNR are different in JEM 3.0 and below. For per-component PSNR each frame PSNRs are calculated and averaged over the entire sequence,

PSNRC= 1Ni=1N10logMAX2MSEiC,

where N is number of frames.

YUV-PSNR is calculated as PSNR of entire sequence as single indivisible signal, see below. Therefore it is impossible to calculate YUV-PSNR of whole sequence from YUV-PSNR’s of segments, to overcome this problem YUV-MSE is output to log, which is averaged easily. Final PSNR is calculated in averaging scripts.

PSNRyuv= 10logMAX21Ni=1NMSEiyuv.

Parallel and sequential simulation results are enclosed as csv tables, both original and stripped from unnecessary data to use any text comparison tool. YUV-metrics differ because of original using YUV-PSNR as source of aggregation, which is not working.

Aggregation scripts and JEM patch are provided with JVET-C0037.

## Advised solution

During AhG discussions alternative solution was discussed to exclude any influence on existing parsing scripts. It was suggested to output precise MSE values to separate log files.

Aggregation scripts and SW patch will be provided.

# Conclusion

This proposal provides the method for aggregating parallel simulation results exact with sequential accurate to floating-point error.

The method 1.2 is proposed to be used in JVET-CTC for JEM 4.0 and above (HM could be supported as well).

# Reference

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 10 (HM10) Encoder Description”, JCTVC-L1002, JCT-VC Meeting, Geneva, Jan 2013

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-B1010, 2nd Meeting: San Diego, USA, 20–26 February 2016.

- X. Ma, H. Chen, H. Yang “Simplification of the common test condition for fast simulation”, JVET-B0036, San Diego, USA, 20–26 February 2016.

- R. Mullakhmetov, M. Sychev, “Sequential/Parallel bistreams unification for JVET CTC”, JVET-C0037, Geneva, May 2016.

# Patent rights declaration(s)

Huawei Technologies have no current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).