| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Chengdu, CN, 15–21 October 2016 | Document: JVET-D0195 |
| --- | --- |

| Title: | AHG8: Viewport Recommendations for CTC Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adeel Abbas | Tel: Email: | aabbas@gopro.com |
| Source: | GoPro |  |  |

_____________________________

# Abstract

This contribution proposes viewports for the content that AHG8 has selected for common test conditions (CTC).

# Introduction

Yaw

In this contribution, we are specifying viewport in terms of Yaw and Pitch parameters (which map to center points of viewports). Pitch specifies y-location, also called zenith of the pixel and ranges from (-π/2 to +π/2). Yaw specifies x-location of the pixel, also referred to as azimuth and ranges from (-π to +π). (0,0) point corresponds to the center pixel of ERP image. It was agreed that viewports will be rendered in 75°x75° FOV using Rectilinear projection.

Pitch

Fig. 1: Introduction to convention of Yaw and Pitch

In the above diagram, Yaw and Pitch are specified in radian coordinate system. Conversion to degree coordinate system, or to absolute pixel values is trivial.

# Viewport Sizes

It has been agreed by AHG8 that viewports for all the CTC will be rendered at 75°x75° FOV. After some calculations, following viewport sizes are recommended. The viewport sizes are kept to be a multiple of 8.

| Class | Sequence name | Frame count | Resolution@FPS | Viewport Size |
| --- | --- | --- | --- | --- |
| 8K | Train | 600 | 8192x4096@60 | 1816x1816 |
| 8K | SkateboardingTrick | 600 | 8192x4096@60 | 1816x1816 |
| 8K | SkateboardInLot | 300 | 8192x4096@30 | 1816x1816 |
| 8K | ChairLift | 300 | 8192x4096@30 | 1816x1816 |
| 8K | KiteFlite | 300 | 8192x4096@30 | 1816x1816 |
| 8K | Harbor | 300 | 8192x4096@30 | 1816x1816 |
| 4K | PoleVault | 300 | 3960x1980@30 | 856x856 |
| 4K | AerialCity | 300 | 3840x1920@30 | 856x856 |
| 4K | DrivingInCity | 300 | 3840x1920@30 | 856x856 |
| 4K | DrivingInCountry | 300 | 3840x1920@30 | 856x856 |

Table 1: Sequencess, Resolutions and Viewport Resolutions

# Viewport Locations

Following viewport locations are recommended in radians and degrees.

|  | Viewport 1 (Yaw, Pitch) |  | Viewport 2 (Yaw, Pitch) |  |
| --- | --- | --- | --- | --- |
| Sequence | Radians | Degrees | Radians | Degrees |
| Train | 1.27, 0.16 | 72.7, 9.1 | 2.84, 0.29 | 162.7, 16.6 |
| SkateboardingTrick | -1.11, 0.34 | -63.6, 19.5 | 1.92, 0.13 | 110, 7.4 |
| SkateboardingInLot | 0.00, 0.00 | 0.00, 0.00 | -1.32, 0.48 | -75.6, 27.5 |
| Chairlift | 0.00, 0.00 | 0.00, 0.00 | -2.73, 1.02 | -156.4, 58.4 |
| KiteFlite | -1.20, 0.67 | -68.7, 38.4 | 1.02, 0.45 | 58.4, 25.8 |
| Harbor | -0.66, 0.72 | -37.8, 41.2 | -1.66, -0.47 | -95, -26.9 |
| PoleVault | 0.00, 0.00 | 0.00, 0.00 | 1.34, 0.67 | 76.7, 38.4 |
| AerialCity | 0.00, 0.00 | 0.00, 0.00 | 2.13, 0.68 | 122.0, 39.0 |
| DrivingInCity | 0.25, 0.50 | 14.3, 28.6 | 1.57, 0.00 | 90.0, 0.00 |
| DrivingInCountry | 0.00, 0.00 | 0.00, 0.00 | 2.38, 0.55 | 136.3, 31.5 |

Table 2: FOV angles for each sequence

Some example viewports for the train sequence are shown in Fig. 2 and Fig. 3.

Fig. 2: Viewport 1 in train seqeuence

Fig. 3: Viewport 2 in train seqeuence

# Playing Viewport Locations in a Player

# A VR player was proposed in [1] that allows playing a 360 file by selecting viewport and rendering into rectilinear projection. When the user presses “H” key, viewport statistics are shown as indicated below:

Fig. 4: Looking at viewport statistics inside the GoPro VR Player

On the bottom right part of the image, statistics like FOV angle Yaw and Pitch (in radians) are shown and updated in real time as the user changes viewport. Experts are encouraged to move viewports to recommended areas and take a look at recommended viewports.

# Extracting Viewports in PCT360

It was mentioned in [2] that PCT360 software can output any desired viewport.

In order to run software, the following order of parameters is used:

HFOV, VFOV, YAW, PITCH

Since AHG has already selected viewport size to be 75x75, the first two parameters would be 75. The Yaw and Pitch is specified in degrees. Furthermore, because of difference in interpretation, the software takes a value of pitch that’s negative of what GoPro VR Player takes (hence value of pitch in Table 2 is multiplied by -1. For example the two viewports for the train sequence are as follow:

Viewport 1: 75, 75, 72.7,  -9.1

Viewport 2: 75 75 162.7, -16.6

# Patent rights declaration(s)

GoPro may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

## A. Abbas, “GoPro VR Player: A Tool for VR Content Playback”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0177, 4th Meeting, Oct. 2016.

## B. Vishwanath, Y. He, X. Xiu, Y. Ye, “AHG8: Suggested test methodology for evaluating projection formats for 360 video coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0074, 4th Meeting, Oct. 2016.