| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0031 r1 |
| --- | --- |

| Title: | EE2-JVET-D0069 Bilateral filter Test1, Test2 and Test3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Kenneth Andersson Jack Enhorn Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46702672192 jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports results of EE2-JVET-D0069 Bilateral filter for Test1 (directly after inverse transform), Test2 (before de-blocking) and Test3 (as a post-filter). It is reported that the highest gains are obtained with Test1 with BD rate results of -0.42% / -0.42% / 0.36 / 0.20% for AI/RA/LDB/LDP with encoding time increase of 7% / 2% / 2% / 3% and decoding time increase of 5% / 0%/ 2%/ 1%. For screen content (class F) the BD rate results are -1.84% /-1.34%/ -1.50% / -1.81%. It is claimed that some visual improvements have been seen mostly in form of reduction of high frequency noise around text with most effect for Test 1.

# Introduction

At the JVET meeting in Chengdu the use of bilateral filter after inverse transform was proposed in JVET-D0069 [1].

A bilateral filter works by basing the filter weights not only on the distance to neighboring samples but also on their values. A sample located at (i, j), will be filtered using its neighboring sample (k, l). The weight ωi,j,k,l is the weight assigned for sample (k, l) to filter the sample (i, j), and it is defined as

ωi,j,k,l=e- i-k2+j-l22σd2 - Ii,j-Ik,l22σr2.                (Eq.1)

Here, I(i, j ) and I(k, l) are the original reconstructed intensity value of samples (i, j) and (k,l) respectively.

σd is the spatial parameter, and σr is the range parameter. The properties (or strength) of the bilateral filter are controlled by these two parameters. Samples located closer to the sample to be filtered, and samples having smaller intensity difference to the sample to be filtered, will have larger weight than samples further away and with larger intensity difference. In the original contribution JVET-D0069 it is proposed to set σd based on the transform unit size (Eq. 2), and σr to be set based on the QP used for the current block (Eq. 3).

σd=0.92-minTU block width, TU block height40                 (Eq.2)

σr=maxQP-172, 0.01                                                               (Eq.3)

It should be noted that in contribution JVET-E0032 [2] it is proposed to change this for inter blocks, however that is out of the scope of this exploration experiment.

# Test description

The software was made available December 2 and can be downloaded from:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE2-D0069

## Test 1 (directly after inverse transform)

In this test the bilateral filter is applied directly after the transform and is cross-checked by Qualcomm.

## Test 2 (before deblocking)

In this test the bilateral filter is applied before deblocking for blocks that have non-zero transform coefficients and uses same filter settings as Test 1 and is cross-checked by Qualcomm.

## Test 3 (as post filter)

In this test the bilateral filter is applied as a postfilter for blocks that have non-zero transform coefficients. This test uses same filter settings as Test 1 and is cross-checked by Samsung.

# Objective results

## Test 1(directly after inverse transform)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,17% | 0,17% | 0,09% | 107% | 106% |
| Class A2 | -0,37% | 0,02% | 0,09% | 106% | 105% |
| Class B | -0,17% | 0,18% | 0,20% | 111% | 109% |
| Class C | -0,81% | 0,09% | 0,05% | 107% | 104% |
| Class D | -0,56% | 0,10% | 0,05% | 106% | 100% |
| Class E | -0,56% | 0,03% | 0,10% | 107% | 106% |
| Overall | -0,42% | 0,11% | 0,10% | 107% | 105% |
| Class F (optional) | -1,84% | -0,28% | -0,27% | 104% | 105% |

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,36% | -0,23% | -0,06% | 104% | 98% |
| Class A2 | -0,60% | -0,68% | -0,59% | 102% | 102% |
| Class B | -0,45% | -0,22% | -0,58% | 103% | 101% |
| Class C | -0,31% | -0,28% | -0,45% | 101% | 96% |
| Class D | -0,38% | -0,21% | 0,09% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0,42% | -0,32% | -0,33% | 102% | 100% |
| Class F (optional) | -1,34% | -0,32% | -0,06% | 105% | 98% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,41% | 0,48% | 0,29% | 106% | 104% |
| Class C | 0,20% | 0,44% | 0,59% | 102% | 103% |
| Class D | -0,50% | 0,50% | 0,61% | 101% | 100% |
| Class E | 1,63% | 0,81% | 0,11% | 100% | 102% |
| Overall (Ref) | 0,36% | 0,54% | 0,41% | 102% | 102% |
| Class F (optional) | -1,50% | -0,40% | 0,10% | 102% | 101% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,13% | -0,04% | 0,01% | 105% | 103% |
| Class C | 0,00% | 0,50% | 0,40% | 105% | 103% |
| Class D | -0,54% | 0,46% | 0,41% | 99% | 97% |
| Class E | 1,57% | 0,19% | 0,25% | 100% | 100% |
| Overall (Ref) | 0,20% | 0,26% | 0,25% | 103% | 101% |
| Class F (optional) | -1,81% | -0,18% | -0,51% | 105% | 105% |

## Test 2(before deblocking)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,63% | -0,06% | -0,01% | 104% | 112% |
| Class A2 | 0,73% | -0,03% | -0,04% | 101% | 109% |
| Class B | 0,66% | -0,01% | -0,03% | 103% | 109% |
| Class C | 0,21% | -0,01% | -0,04% | 103% | 109% |
| Class D | 0,43% | 0,01% | 0,00% | 100% | 101% |
| Class E | 0,16% | 0,00% | 0,00% | 101% | 107% |
| Overall | 0,49% | -0,02% | -0,02% | 102% | 108% |
| Class F (optional) | -0,66% | -0,03% | -0,07% | 101% | 107% |

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,32% | -0.24% | -0,19% | 101% | 102% |
| Class A2 | 0,58% | -0,07% | -0,20% | 98% | 101% |
| Class B | 0,48% | -0,30% | -0,32% | 101% | 101% |
| Class C | 0,60% | -0,17% | -0,12% | 99% | 98% |
| Class D | 0,35% | -0,03% | -0,10% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0,47% | -0,17% | -0,19% | 100% | 100% |
| Class F (optional) | -0,55% | -0,14% | -0,09% | 102% | 100% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1,67% | 0,31% | 0,46% | 103% | 103% |
| Class C | 1,52% | 0,37% | 0,02% | 100% | 101% |
| Class D | 0,37% | 0,17% | 0,39% | 100% | 100% |
| Class E | 6,00% | 1,67% | 0,49% | 97% | 98% |
| Overall (Ref) | 2,12% | 0,54% | 0,34% | 100% | 101% |
| Class F (optional) | -0,06% | -0,37% | 0,02% | 100% | 100% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1,30% | -0,05% | -0,03% | 100% | 101% |
| Class C | 1,30% | 0,34% | -0,01% | 101% | 102% |
| Class D | 0,40% | 0,52% | 0,16% | 100% | 100% |
| Class E | 4,79% | 0,66% | 0,79% | 103% | 105% |
| Overall (Ref) | 1,73% | 0,32% | 0,18% | 101% | 102% |
| Class F (optional) | -0,32% | 0,02% | -0,51% | 103% | 104% |

## Test 3(as post filter)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1,38% | 0,00% | 0,00% | - | 112% |
| Class A2 | 1,35% | 0,00% | 0,00% | - | 115% |
| Class B | 1,46% | 0,00% | 0,00% | - | 114% |
| Class C | 0,94% | 0,00% | 0,00% | - | 112% |
| Class D | 1,10% | 0,00% | 0,00% | - | 102% |
| Class E | 0,99% | 0,00% | 0,00% | - | 108% |
| Overall | 1,22% | 0,00% | 0,00% | - | 110% |
| Class F (optional) | -0,82% | 0,00% | 0,00% | - | 107% |

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,59% | 0,00% | 0,00% | - | 103% |
| Class A2 | 0,41% | 0,00% | 0,00% | - | 102% |
| Class B | 0,55% | 0,00% | 0,00% | - | 107% |
| Class C | 0,34% | 0,00% | 0,00% | - | 102% |
| Class D | 0,20% | 0,00% | 0,00% | - | 105% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0,42% | 0,00% | 0,00% | - | 104% |
| Class F (optional) | -0,14% | 0,00% | 0,00% |  | 107% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,70% | 0,00% | 0,00% | - | 107% |
| Class C | 0,34% | 0,00% | 0,00% | - | 102% |
| Class D | 0,22% | 0,00% | 0,00% | - | 96% |
| Class E | 0,19% | 0,00% | 0,00% |  | 106% |
| Overall (Ref) | 0,39% | 0,00% | 0,00% | - | 103% |
| Class F (optional) | -0,20% | 0,00% | 0,00% |  | 110% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,72% | 0,00% | 0,00% | - | 107% |
| Class C | 0,38% | 0,00% | 0,00% | - | 105% |
| Class D | 0,24% | 0,00% | 0,00% | - | 104% |
| Class E | 0,19% | 0,00% | 0,00% |  | 109% |
| Overall (Ref) | 0,42% | 0,00% | 0,00% | - | 106% |
| Class F (optional) | -0,21% | 0,00% | 0,00% |  | 109% |

# Visual inspection

The bilateral filter can reduce high-frequency noise from quantization of transform coefficients. Below an example of the subjective effect of this for Test 1, Test 2 and Test 3 for the first frame of BQSquare all intra coded at QP37.

## Test 1

Since Test 1 is part of RDO decisions that approach can give most effect. Note that the ringing to the right of the object is reduced in the right-most image.

Left: Source, Middle: JEM.4.0, Right: Test 1

## Test 2

The same area as in Test1:

Left: Source, Middle: JEM4.0, Right: Test 2 with bugfix

## Test 3

The same area as in Test 1:

Left: Source, Middle: JEM-4.0, Right: Test 3

# Conclusions

The original proposed version as in Test 1 of having the bilateral filter after the inverse transform works best among the three tests. In summary, it gives BD rate results of -0.42%/-0.42%/0.36%/0.20%, while Test 2 (after deblocking) gives 0.49%/0.47%/2.12%/1.73%, which is worse in every case.  These are the new results of Test 2 after the bug fix, all results are now included. Test 3 (as a post filter) gives 1.22%/0.42%/0.39%/0.42%, which is also worse in every case.

Note that while Test 1 shows reduced performance for LDB and LDP, a new proposal where the strength of the bilateral filter is reduced for inter blocks [2] rectifies this.

# References

[1] J. Ström, P. Wennersten, Y. Wang, K. Andersson, J. Samuelsson,  ”Bilateral Filter After Inverse Transform”, JVET-D0069, Chengdu, China, 15–21 October 2016

[2] J. Ström, P. Wennersten, K. Andersson, J. Enhorn, ” Bilateral filter strength based on prediction mode”, JVET-E0032, Geneva, Switzerland, 12-20 January 2017

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).