| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0098 |
| --- | --- |

| Title: | EE5 #4: Cross-check of JVET-E0077 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Qiang Yao Kei Kawamura Sei Naito | Email: | qi-yao@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research Inc.) |  |  |

_____________________________

- Abstract

This document is a cross-check report of EE5: Enhanced Cross-component Linear Model Intra-prediction, with Supplementary Results (JVET-E0077) on test #4, which integrates a multi-model LM (MMLM) approach, a multi-filter LM (MFLM) method, and a combined LM-Angular prediction (LAP) mode. It is reported that the method shows -0.44%(Y)/-3.93% (Cb)/-4.10% (Cr), -0.24%(Y)/-3.37% (Cb)/-3.22% (Cr), and -0.00%(Y)/-0.71% (Cb)/-0.75% (Cr), BD-rate gain with 102%/103%, 101%/100%, and 100%/100% encoding/decoding time on average in All Intra, Random Access and Low delay B (LDB) configuration, respectively. The simulation results match with those provided by proponents.

Introduction

In this assigned cross check, the combination of test 1, test 2 and test 3 is simulated. It is reported that the method shows -0.44%(Y)/-3.93% (Cb)/-4.10% (Cr), -0.24%(Y)/-3.37% (Cb)/-3.22% (Cr), and -0.00%(Y)/-0.71% (Cb)/-0.75% (Cr), BD-rate gain on average in All Intra, Random Access and Low delay B (LDB) configuration, respectively. In addition, the encoding/decoding time is observed at 102%/103%, 101%/100%, and 100%/100% on average in All Intra, Random Access and Low delay B (LDB) configuration, respectively. The simulation results match with those provided by proponents.

SW implementation is clear, and the analysis of SW implementation is briefly described as below.

In the test #4, the total number of chroma mode is set to 11 when both of MMLM and MFLM are enabled. The number of multimode is set as two when MMLM is enabled.

One additional neighboring line in both Above and Left adjacent block is adopted if MMLM is enabled, and the motivation might come that including more samples provides better result. In order to keep only four neighboring lines accessible, when MFLM is disabled the downsampling filter of the second nearest neighboring line in the Left block is modified to 3131/8, while other downsampling filters remained same as JEM4.0.

Test assignment

The cross check corresponds to test 4 in the following EE test table.

List of tests to be performed

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 | Multiple linear models, samples are grouped in two sets | Qualcomm | Sharp |
| 2 | Multiple-filter LM | Qualcomm | ETRI |
| 3 | average of angular and LM mode | Qualcomm | Samsung |
| 4 | Test 1+ Test 2+ Test 3 | Qualcomm | KDDI |
| 5 | Multi direct modes for chroma coding | Qualcomm | KDDI |

Simulation results

The source code of the EE5 tests 4 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE5-D0110/, and tested under JVET CTC compared to the JEM4.0 as anchor. Provided software was compiled by GCC 4.8.5 on 64-bit Linux. The experiments were performed on the cluster PCs.

The simulation results are presented in the tables below and match with those provided by proponents.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.24% | -5.71% | -8.74% | 102% | 103% |
| Class A2 | -0.41% | -5.11% | -4.67% | 103% | 102% |
| Class B | -0.21% | -3.76% | -2.76% | 102% | 103% |
| Class C | -0.46% | -3.97% | -3.57% | 102% | 103% |
| Class D | -0.24% | -2.71% | -2.51% | 102% | 102% |
| Class E | -0.03% | -1.83% | -2.20% | 103% | 102% |
| Overall | -0.44% | -3.93% | -4.10% | 102% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -4.24% | -6.58% | 101% | 100% |
| Class A2 | -0.14% | -4.52% | -3.87% | 100% | 100% |
| Class B | -0.08% | -3.77% | -2.29% | 101% | 100% |
| Class C | -0.15% | -2.23% | -2.16% | 101% | 100% |
| Class D | -0.04% | -1.99% | -1.39% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | -0.24% | -3.37% | -3.22% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.97% | -0.58% | 100% | 99% |
| Class C | 0.08% | -0.43% | -0.37% | 100% | 100% |
| Class D | 0.00% | -0.28% | -1.00% | 101% | 101% |
| Class E | -0.15% | -1.24% | -1.22% | 100% | 100% |
| Overall (Ref) | 0.00% | -0.71% | -0.75% | 100% | 100% |

Conclusion

The simulation results of EE5 test 4 has been verified. The encoding / decoding runtime also matched closely to those reported by the proponent.

References

- Kai Zhang, Jianle Chen, Li Zhang, Marta Karczewicz, “Enhanced Cross-component Linear Model Intra-prediction”, JVET-D0110, 4th Meeting: Chengdu, CN, 15–21 October 2016

- Kai Zhang, Jianle Chen, Li Zhang, Marta Karczewicz, EE5: Enhanced Cross-component Linear Model Intra-prediction”, JVET-E0077, 5th Meeting: Geneva, CH, 12–20 January 2017