| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0110 |
| --- | --- |

| Title: | AHG4: Evaluation report of SDR test sequences (4K8-9 and 1080p1-5) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | You-Huei Ju, 1Ching-Chieh Lin, 2Chun-Lung Lin, Yao-Jen Chang and Po-Han Lin 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-5758 1JackLin@itri.com 2Chunlung@itri.com |
| Source: | ITRI international |  |  |

_____________________________

- Abstract

This document reports the encoding results of QP settings to match a specific target bitrate. Bitstreams of five 1080p and two 4K test sequences were encoded by HM-16.13 and JEM-4.0 reference software under the conditions specified in JVET-D1002. Objective and subjective evaluations are also presented in this document for assessment of test material.

Introduction

In Chengdu meeting, a work plan [1] was set up to evaluate picture qualities of 4K and 1080p test sequences encoded by HM-16.13 and JEM-4.0 at approximately the same bitrate. The test sequences we tested are five 1080p sequences: Metro, Food Market 2, Ritual Dance, Square Time lapse, Runners in JVET-C0028 [2] and JVET-C0041[3] and two new 4K sequences: iceAerial and iceRiver in JVET-D0060 [4].

Encoding conditions

The encoding conditions are set as follows:

- Encoder: JEM-4.0, HM-16.13 (Both mandatory)

- Encode only 4:2:0 yuv 10 bit

- Class A: 10 seconds will be encoded,

- Class B: 10 seconds will be encoded (Except Food Market 2)

- Class A(4K): RA, Class B(1080p) RA

- I period: 32 (30fps), 48 (50 fps), 64 (60 fps), 96 (100 fps) and 112 (120 fps)

- Level: 4.1(1080p), 6.2(4K)

- The following parameters are identical between HM and JEM

  - GOP size: 16

  - Lamda/QP handling

  - MV search range: 256 (For HM-16.13, we used the configures in HM-16.6-JEM-4.0/cfg/hm-16.13 instead of HM-16.13/cfg)

- For JEM-4.0, the parameter JVET_B0039_INC_NUM_QP_PRO is changed to 52

Target bitrates are defined in the following tables.

Table 1: Target bit rate for class A

|  | Target bit rate (Mbps) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frame rate (fps) | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 | Rate 6 |
| 100 | 1.5 | 2.3 | 3.6 | 6 | 11 | 18 |
| 60 | 1 | 1.5 | 2.4 | 4 | 7 | 12 |
| 50 | 0.8 | 1.2 | 2 | 3.3 | 6 | 10 |
| 30 | 0.6 | 1 | 1.6 | 2.7 | 5 | 8 |

According to the test plan [1], hard sequences (ToddlerFountain, ParkRunning, CampfireParty and Runners) require the use of rate 2 to rate 6 for proper picture qualities. As for the others, rate1 to rate 5 are used for testing. Since both iceAerial and iceRiver are new 4k test sequences, we encoded the two test sequences from rate1 to rate6 to study the encoding performance.

Table 2: Target bit rate for class B

|  | Target bit rate (Mbps) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frame rate (fps) | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| 60 | 0.6 | 0.9 | 1.5 | 2.6 | 4.3 |
| 50 | 0.5 | 0.8 | 1.2 | 2.0 | 3.5 |
| 30 | 0.4 | 0.6 | 1 | 1.7 | 2.9 |
| 24 | 0.3 | 0.5 | 0.8 | 1.3 | 2.2 |

The frame number of Food Market 2 tested in this contribution is 782. The down-conversion from 4K to 1080p was done by ffmpeg with the below command:

ffmpeg.exe -f rawvideo -pix_fmt yuv420p10le -s:v 4096x2160 -i Netflix_FoodMarket2_long_4096x2160_60fps_10bit_420.yuv -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd -vsync 0 -vf crop=iw-256:ih+0:128:0,scale=1920:1080,setsar=1/1 Netflix_FoodMarket2_long_1920x1080_60fps_10bit_420.yuv

The MD5 of the down-converted Food Market 2(782 frames) is: 900ade8539d0a352010ebcb6bc07caf

Test sequences evaluation

QP selection

From a series of integer QPs, we chose the QP value having a bitrate closest to but larger than the target bitrate. These QP points are our base QPs used to derive the float QPs. The selected base QPs are summarized below. Details of the coding results are attached in the accompanying excel files.

Table 3: Encoding results of 1080p test sequences by using HM-16.13 and JEM-4.0 with selected integer QP

| Metro |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 40 | 0.6567136 | 9.45% | 39 | 0.6496008 | 8.27% | -1.08% |
| R2 | 0.9 | 37 | 0.988548 | 9.84% | 36 | 0.9691592 | 7.68% | -1.96% |
| R3 | 1.5 | 34 | 1.5046648 | 0.31% | 32 | 1.6893128 | 12.62% | 12.27% |
| R4 | 2.6 | 29 | 2.9380112 | 13.00% | 28 | 2.8959128 | 11.38% | -1.43% |
| R5 | 4.3 | 26 | 4.4231184 | 2.86% | 25 | 4.4511984 | 3.52% | 0.63% |
| Food Market 2 |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| 60 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 41 | 0.6670287 | 11.17% | 40 | 0.6466337 | 7.77% | -3.06% |
| R2 | 0.9 | 38 | 1.0105528 | 12.28% | 37 | 0.9725119 | 8.06% | -3.76% |
| R3 | 1.5 | 35 | 1.5012884 | 0.09% | 33 | 1.6685543 | 11.24% | 11.14% |
| R4 | 2.6 | 30 | 2.9518969 | 13.53% | 29 | 2.8751196 | 10.58% | -2.60% |
| R5 | 4.3 | 27 | 4.4002631 | 2.33% | 26 | 4.3550682 | 1.28% | -1.03% |
| Ritual Dance |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| 60 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 45 | 0.6092392 | 1.54% | 42 | 0.6647432 | 10.79% | 9.11% |
| R2 | 0.9 | 40 | 1.024096 | 13.79% | 39 | 1.0002264 | 11.14% | -2.33% |
| R3 | 1.5 | 37 | 1.5182568 | 1.22% | 35 | 1.6691024 | 11.27% | 9.94% |
| R4 | 2.6 | 32 | 2.9077504 | 11.84% | 31 | 2.8443696 | 9.40% | -2.18% |
| R5 | 4.3 | 29 | 4.3018616 | 0.04% | 27 | 4.6658144 | 8.51% | 8.46% |
| Square Time lapse |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| 60 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R5 | 4.3 | 28 | 4.677008 | 8.77% | 27 | 4.5966376 | 6.90% | -1.72% |
| R1 | 0.6 | 42 | 0.627848 | 4.64% | 41 | 0.6219232 | 3.65% | -0.94% |
| R2 | 0.9 | 39 | 0.9713264 | 7.93% | 38 | 0.9489184 | 5.44% | -2.31% |
| R3 | 1.5 | 35 | 1.7081952 | 13.88% | 34 | 1.6883512 | 12.56% | -1.16% |
| R4 | 2.6 | 32 | 2.646236 | 1.78% | 30 | 2.9710888 | 14.27% | 12.28% |
| Runners |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| 30 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R5 | 2.9 | 33 | 2.9864456 | 2.98% | 32 | 3.0641352 | 5.66% | 2.60% |
| R1 | 0.4 | 49 | 0.4193096 | 4.83% | 47 | 0.4386336 | 9.66% | 4.61% |
| R2 | 0.6 | 45 | 0.6092392 | 1.54% | 44 | 0.6022384 | 0.37% | -1.15% |
| R3 | 1 | 40 | 1.162004 | 16.20% | 40 | 1.0317504 | 3.18% | -11.21% |
| R4 | 1.7 | 37 | 1.7356472 | 2.10% | 36 | 1.7777816 | 4.58% | 2.43% |

Table 4: Encoding results of 4K test sequences by using HM-16.13 and JEM-4.0 with selected integer QP

| iceAerial |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 50 | 0.6241272 | 4.02% | 46 | 0.647808 | 7.97% | 3.79% |
| R2 | 1 | 44 | 1.03116 | 3.12% | 41 | 1.0430552 | 4.31% | 1.15% |
| R3 | 1.6 | 39 | 1.6642096 | 4.01% | 36 | 1.733988 | 8.37% | 4.19% |
| R4 | 2.7 | 34 | 2.8182536 | 4.38% | 32 | 2.7983144 | 3.64% | -0.71% |
| R5 | 5 | 29 | 5.3427272 | 6.85% | 28 | 5.362144 | 7.24% | 0.36% |
| R6 | 8 | 26 | 9.6834464 | 21.04% | 26 | 8.6392584 | 7.99% | -10.78% |
| iceRiver |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| 30 fps | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | baseQP | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 39 | 0.6626528 | 10.44% | 38 | 0.6047352 | 0.79% | -8.74% |
| R2 | 1 | 35 | 1.097928 | 9.79% | 33 | 1.1261416 | 12.61% | 2.57% |
| R3 | 1.6 | 31 | 1.7952456 | 12.20% | 30 | 1.6026384 | 0.16% | -10.73% |
| R4 | 2.7 | 27 | 2.9236216 | 8.28% | 25 | 3.195888 | 18.37% | 9.31% |
| R5 | 5 | 23 | 5.4509576 | 9.02% | 22 | 6.137252 | 22.75% | 12.59% |
| R6 | 8 | 21 | 8.6641248 | 8.30% | 20 | 10.3794488 | 29.74% | 19.80% |

Target bitrate matching using a float QP

To get float QPs, we used the python code and excel file attached in the test plan [1] when encoding bitstreams by HM-16.13 and JEM-4.0 respectively. For HM-16.13, we used sequential encoding to encode bitstreams with float QPs. In order to encode JEM-4.0 bitstreams with float QP by parallel encoding, steps described in the test plan [1] are used.

In the excel template, there are two estimated QP values for each target bitrate. One is slightly less than the target bitrate, and the other is slightly larger than the target bitrate. According to the test plan, bitrate differences between JEM and HM should be within 5%. Therefore, we chose the bitrate closer to the result of HM-16.13. The bitrates and PSNR values are collected from the re-encoded sequences.

The encoding results of 1080p test sequences were cross checked by Sony, and the encoding results of 4K test sequences were cross checked by DJI and Qualcomm. The results are summarized below. More details can be found in the attached excel files.

Table 5: Encoding results of 1080p test sequences by using HM-16.13 and JEM-4.0 with float QP

| Metro (60p) |  | HM-16.13 (sequential enc) |  |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 40.68 | 0.5931088 | -1.15% | 3 | 39.21538 | 0.5968296 | -0.53% | 0.63% |
| R2 | 0.9 | 37.7867 | 0.8933168 | -0.74% | 3 | 36.47692 | 0.898488 | -0.17% | 0.58% |
| R3 | 1.5 | 34.0533 | 1.4936912 | -0.42% | 0 | 32.21538 | 1.4906008 | -0.63% | -0.21% |
| R4 | 2.6 | 29.92 | 2.6219544 | 0.84% | 0 | 28.21538 | 2.5802456 | -0.76% | -1.59% |
| R5 | 4.3 | 26.2533 | 4.2998624 | 0.00% | 6 | 25.21538 | 4.2978376 | -0.05% | -0.05% |
| Food Market 2 (60p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 41.7826 | 0.5969629 | -0.51% | 5 | 40.78261 | 0.599361 | -0.11% | 0.40% |
| R2 | 0.9 | 38.8158 | 0.8950975 | -0.54% | 5 | 37.7385 | 0.8992198 | -0.09% | 0.46% |
| R3 | 1.5 | 35.0384 | 1.4918357 | -0.54% | 2 | 33.47692 | 1.4932346 | -0.45% | 0.09% |
| R4 | 2.6 | 31 | 2.5973487 | -0.10% | 3 | 29.21538 | 2.5925702 | -0.29% | -0.18% |
| R5 | 4.3 | 27.2353 | 4.2852773 | -0.34% | 10 | 26.21538 | 4.2836329 | -0.38% | -0.04% |
| Ritual Dance (60p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 45.36 | 0.5962408 | -0.63% | 0 | 43 | 0.5983544 | -0.27% | 0.35% |
| R2 | 0.9 | 40.9667 | 0.8946896 | -0.59% | 2 | 39.47692 | 0.899476 | -0.06% | 0.53% |
| R3 | 1.5 | 37.1467 | 1.4921496 | -0.52% | 1 | 35.21538 | 1.4985216 | -0.10% | 0.43% |
| R4 | 2.6 | 32.865 | 2.6233872 | 0.90% | 3 | 31.21538 | 2.596 | -0.15% | -1.04% |
| R5 | 4.3 | 29.0533 | 4.2833192 | -0.39% | 3 | 27.51389 | 4.286272 | -0.32% | 0.07% |
| Square Time lapse (60p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 42.555 | 0.5977008 | -0.38% | 4 | 41.21538 | 0.5953088 | -0.78% | -0.40% |
| R2 | 0.9 | 39.6967 | 0.8928184 | -0.80% | 4 | 38.7385 | 0.9010952 | 0.12% | 0.93% |
| R3 | 1.5 | 36 | 1.4867128 | -0.89% | 1 | 34.47692 | 1.4942184 | -0.39% | 0.50% |
| R4 | 2.6 | 32.2967 | 2.5873888 | -0.49% | 0 | 30.7385 | 2.614348 | 0.55% | 1.04% |
| R5 | 4.3 | 28.7433 | 4.2798168 | -0.47% | 4 | 27.47692 | 4.2998744 | 0.00% | 0.47% |
| Runners (30p) |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
|  | target rate [Mbps] | baseQP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.4 | 49.7733 | 0.3950504 | -1.24% | 0 | 48 | 0.3988496 | -0.29% | 0.96% |
| R2 | 0.6 | 45.2533 | 0.5942176 | -0.96% | 8 | 44.48485 | 0.5967008 | -0.55% | 0.42% |
| R3 | 1 | 41 | 0.9946464 | -0.54% | 6 | 40.4848 | 0.9919552 | -0.80% | -0.27% |
| R4 | 1.7 | 37.2267 | 1.6931464 | -0.40% | 5 | 36.48485 | 1.6903248 | -0.57% | -0.17% |
| R5 | 2.9 | 33.28 | 2.8879504 | -0.42% | 5 | 32.4848 | 2.9074664 | 0.26% | 0.68% |

Table 6: Encoding results of 4K test sequences by using HM-16.13 and JEM-4.0 with float QP

| iceAerial |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30fps | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 50.78667 | 0.5939232 | -1.01% | 1 | 46.4848 | 0.5993208 | -0.11% | 0.91% |
| R2 | 1 | 44.41667 | 0.9950976 | -0.49% | 3 | 41.4848 | 0.9775592 | -2.24% | -1.76% |
| R3 | 1.6 | 39.46667 | 1.5906992 | -0.58% | 1 | 36.48485 | 1.5964744 | -0.22% | 0.36% |
| R4 | 2.7 | 34.42667 | 2.6951264 | -0.18% | 6 | 32.4848 | 2.6984336 | -0.06% | 0.12% |
| R5 | 5 | 29.46667 | 5.0004312 | 0.01% | 3 | 28.4848 | 4.7934352 | -4.13% | -4.14% |
| R6 | 8 | 26.86333 | 8.1844976 | 2.31% | 5 | 26.48485 | 7.88608 | -1.42% | -3.65% |
| iceRiver |  | HM-16.13 (sequential enc) |  |  | HM-16.6-JEM-4.0 (parallel enc) |  |  |  |  |
| 30fps | target rate [Mbps] | QP | bitrate [Mbps] | diff to target | RAS# | QPfrac/PAR | bitrate [Mbps] | diff to target | diff anchor vs test |
| R1 | 0.6 | 39.84 | 0.5967392 | -0.54% | 8 | 38.48485 | 0.5991368 | -0.14% | 0.40% |
| R2 | 1 | 35.81335 | 0.9953432 | -0.47% | 0 | 34 | 0.998492 | -0.15% | 0.32% |
| R3 | 1.6 | 32 | 1.5876104 | -0.77% | 8 | 30.48485 | 1.5888368 | -0.70% | 0.08% |
| R4 | 2.7 | 27.68 | 2.6935176 | -0.24% | 0 | 26 | 2.69126 | -0.32% | -0.08% |
| R5 | 5 | 23.52 | 4.9829456 | -0.34% | 1 | 22.48485 | 4.9683352 | -0.63% | -0.29% |
| R6 | 8 | 21.36 | 7.932324 | -0.85% | 0 | 21 | 7.9096288 | -1.13% | -0.29% |

The R-D curves of the encoding results by using float QPs are presented as follows. More detail of the PSNR values and bitrate can be found in the attached excel files “1080p_summary.xlsm” and “4K_summary.xlsm”.

Subjective test

In most cases, the qualities of rate 4 and rate 5 are acceptable and there are no noticeable artifacts, blurring or blocking effect. As a result, we focused on the qualities from rate 1 to rate 3 to compare the encoding performance between HM-16.13 and JEM-4.0.

Figure 1-1: #74 of Metro in HM-16.13 with QP40.68 (Rate 1)

Figure 1-2: #74 of Metro in JEM-4.0 with QP39.21538 (Rate 1)

Though both of them have serious blurring, the boundaries of objects in HM-16.13 are more unclear and blocking effect is observed.

Figure 2-1: #44 of Food Market 2 in HM-16.13 with QP41.78261 (Rate 1)

Figure 2-2: #44 of Food Market 2 in JEM-4.0 with QP40.47692 (Rate 1)

In the scene full of smoke, we can observe a serious blocks appeared in HM-16.13, but this problem was improved in JEM-4.0.

Figure 3-1: #76 Ritual Dance in HM-16.13 with QP37.14667 (Rate 3)

Figure 3-2: #76 Ritual Dance in JEM-4.0.0 with QP35.21538 (Rate 3)

In the original video, there is a bokeh effect but it is blurred with the woman’s face after compression performed both by HM-16.13 and JEM-4.0. Moreover, some unnatural blocks are observed in the video encoded by HM-16.13 at rate 3.

Figure 4-1: #195 of Square Time lapse in HM-16.13 with QP42.555 (Rate 1)

Figure 4-2: #195 of Square Time lapse in JEM-4.0 with QP41.21538 QP (Rate 1)

In HM-16.13, the edges of objectives are distorted and block effect is also observed. However, these issues are ameliorated in JEM-4.0.

Figure 5-1: #170 of Runners in HM-16.13 with QP41 (Rate 3)

Figure 5-2: #170 of Runners in JEM-4.0 with QP40.4848 (Rate 3)

The differences between HM-16.13 and JEM-4.0 are not apparent, more artifacts are observed around moving people in HM-16.13.

Figure 6-1: #296 of iceAerial in HM-16.13 with QP44.41667 (Rate 2)

Figure 6-2: #296 of iceAerial in JEM-4.0 with QP41.4848 (Rate 2)

For iceAerial sequence, most of the scenes are snow-covered landscape and it is not easy to observe differences between Rate 1 and Rate 6. Therefore, we paid attention to the flowing water. The block effect makes the details of the spray missing. In HM-16.13, it even becomes worse.

Figure 7-1: The left one (a) is #79 of iceRiver in HM-16.13 with QP21.36(Rate 6). The right one (b) is #79 of iceRiver in JEM-4.0 with QP20.4848 (Rate 6)

Figure 7-2: The left one (a) is #79 of iceRiver in HM-16.13 with QP39.84(Rate 1). The right one (b) is #79 of iceRiver in JEM-4.0 with QP38.48485 (Rate 1)

Compared with other test sequences, iceRiver is relatively easier to encode. For each rate, the results of HM-16.13 and JEM-4.0 look similar, and the distinctions between rate 1 and rate 6 are much smaller than other test sequences. In this case, it is suggested to use rate 1 and rate 6 for subjective evaluation so that the differences might be clear.

In summary, Ritual Dance and Runners may be hard sequences suggesting rate 3 and rate 5 for subjective evalutations because the qualities of rate 1 and rate 2 may not be good enough to evaluate. Others except iceRiver may be able to use rate 2 and rate 4 to compare. As for iceRiver, due to its easiness of encoding, rate 1 and rate 6 are suggested.

Conclusion

The contribution reports objective and subjective evaluation results of the five 1080p (1080p1-5) and two 4K (4k8-9) sequences. Bitstreams were generated by HM-16.13 and JEM-4.0 according to the test plan. It is suggested to include iceAerail, Metro and Food Market2 for further testing.

Reference

- “Work plan for assessment of test material,” JVET-D1002, November 2016

- “Suggested 1080P Test Sequences Downsampled from 4K Sequences,” JVET-C0028, May 2016

- “Proposed test sequences for 1080p class,” JVET-C0041, May 2016

- “Aerial photography sequences for video coding standard development”, JVET-D0060, October 2016

- ftp://ftp.ient.rwth-aachen.de/

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.13/

- https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-4.0/

Patent rights declaration(s)

ITRI international does not have any current or pending patent rights relating to the technology described in this contribution.