| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0123 |
| --- | --- |

| Title: | EE6: Cross-check and Supplemental result of JVET-E0055 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Zhao and Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Email: | jzhao@sharlabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This contribution reports the SHARP cross check of “EE6: Performance evaluation of adaptive scaling of JVET-D0118 for extended color volume material

Due to possible encoder configuration and/or sequence differences, we note that the results (both anchor and test) do not match the results presented in JVET-E0055. However, as the purpose of the EE is mainly to test the HDR/WCG CTC, results are provided as both test and anchor here use the same configuration and test sequences, and so the relative performance may still be useful to the group.

# Introduction

Software of EE6-D0118 was released by the proponent into SVN EE branch. During cross checking, we observed that it is the same as the software released in EE-D0124 with macro SHARP_DSCALE_PRED_ONLY enabled. It uses the luma value of a prediction block to determine scale factor and scale all the coefficients.

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE6-D0118

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-4.0-EE6-D0124

# Simulation results

Tests were done following HDR/WCG common test condition defined in JVET-D1020 [3]. Two set of test sequences are used. Table 1 sequences defined in JVET-D1020 are JVET sequences converted to PQ2100 representation. Table 2 sequences defined in JVET-D1020 are HDR sequences. Test QP 17, 22, 27, 32, 37 where the lower four Qp points ( 17 to 32) are denoted as Qp set1, and the higher four Qp points (22 to 37) are denoted as Qp set2 when being used to calculate BD rates. We ran AI and RA test configurations.

AI and RA are tested. Note that encoding and decoding time are not reliable because of uniformity of our cluster.

Table 1 Performance of EE6-D0118 vs . EE6-anchor (JVET-D1020 Table 1 sequences)

Table 2 Performance of EE6-D0118 vs . EE6-anchor (JVET-D1020 Table 1 sequences)

# Table 3 Performance of EE6-D0118 vs . EE6-D0124 (JVET-D1020 Table 1 sequences)

# Table 4 Performance of EE6-D0118 vs . EE6-D0124 (JVET-D1020 Table 2 sequences)

# Comments on the Mis-match

During the cross check, we determined that one source of the mis-match was the use of different values for the MaxCuDQPDepth parameter.  Specifically, JVET-E0055 used a value of MaxCuDQPDepth=0 while JVET-E0081 used a value of MaxCuDQPDepth=1.  However, we partially re-ran the cross check using both values and observed that we were not able to confirm the results of JVET-E0055 with either configuration.

A working hypothesis is that the sequences did not match (included the HDR data in Table 2) and suggest comparison of MD5 sums as next steps.

# Summary

Overall summary of performance of EE6-D0118 vs EE6-anchor and EE6-D124 are shown in the table below:

- JVET-D1020 Table 1 sequences (SDR PQ2100 sequences)

| Avg QP set1 & 2 |  | wPSNR Y | wPSNR U | wPSNR V | tPSNR Y | DE100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vs. EE6-anchor | AI | -1.76% | -1.17% | -0.89% | 1.12% | -0.88% | -1.51% | 1.11% | 0.72% | 1.01% |
|  | RA | -2.62% | -2.08% | -1.55% | -0.49% | -1.77% | -2.39% | -0.33% | 0.74% | 1.09% |
|  | LD |  |  |  |  |  |  |  |  |  |
| vs. EE6-D0124 | AI | -0.22% | 0.43% | 0.11% | 0.12% | 0.13% | -0.34% | 0.08% | 0.34% | 0.08% |
|  | RA | -0.21% | 0.53% | 0.30% | 0.08% | -0.06% | -0.30% | 0.07% | 0.50% | 0.23% |

For JVET-D1020 Table 1 sequences,

- Comparing to EE6-anchor, for AI: EE6-D0118 shows gain of 1.76%, 0.88% and 1.51% in terms of wPSNRY, deltaE100 and PSNRL100, shows loss of 1.12% and 1.11% in terms of tPSNRY and PSNRY;  for RA:  EE6-D0118 shows gain of 2.62%, 1.77% and 2.39% in terms of wPSNRY, deltaE100 and PSNRL100, shows gain of 0.49% and 0.33% in terms of tPSNRY and PSNRY;

- Comparing to EE6-D0124, for AI: EE6-D0118 shows gain of 0.22% and 0.34% in terms of wPSNRY and PSNRL100, shows loss of 0.12%, 0.13% and 0.08% in terms of tPSNRY, detlaE100 and PSNRY;   for RA:  EE6-D0118 shows gain of 0.21%, 0.06% and 0.3%% in terms of wPSNRY, deltaE100 and PSNRL100, shows loss of 0.08% and 0.07% in terms of tPSNRY and PSNRY

Please note that JVET-E0055 only provided the QP set2 (Qp 22-37) result. For reference, the results from the contribution :

| QP set2 |  | wPSNR Y | wPSNR U | wPSNR V | tPSNR Y | DE100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vs. EE6-anchor | AI | -2.00% | -1.18% | -1.05% | 1.45% | -0.28% | -1.19% | 1.69% | 1.34% | 1.53% |
|  | RA | -2.34% | -2.63% | -2.59% | 0.20% | -1.43% | -1.94% | 0.30% | 0.81% | 0.98% |
|  | LD |  |  |  |  |  |  |  |  |  |

For comparison, the Qp set2 result from the cross check was:

| QP set2 |  | wPSNR Y | wPSNR U | wPSNR V | tPSNR Y | DE100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vs. EE6-anchor | AI | -1.65% | -1.04% | -1.09% | 0.94% | -0.94% | -1.42% | 0.94% | 0.51% | 0.47% |
|  | RA | -2.30% | -1.59% | -1.52% | -0.67% | -1.55% | -2.14% | -0.47% | 0.62% | 0.53% |
|  | LD |  |  |  |  |  |  |  |  |  |

We would suggest that the WPSNR result of two results are relatively close, while the rest of the metrics are not.

- JVET-D1020 Table 2 sequences (HDR sequences):

| Avg Qp set1& 2 |  | wPSNR Y | wPSNR U | wPSNR V | tPSNR Y | DE100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vs. EE-6 anchor | AI | -1.10% | -1.26% | -2.18% | 0.31% | -0.90% | -1.66% | 0.35% | 0.88% | 0.41% |
|  | RA | -1.16% | -2.73% | -4.00% | 0.30% | -0.93% | -1.53% | 0.33% | 0.55% | -0.03% |
| vs. EE6-D0124 | AI | -0.30% | 0.14% | 0.12% | 0.24% | -0.09% | -0.89% | 0.18% | 0.15% | 0.15% |
|  | RA | -0.24% | -0.16% | -0.10% | 0.17% | 0.07% | -0.79% | 0.20% | 0.05% | -0.05% |

For JVET-D1020 Table 2 sequences,

- Comparing to EE6-anchor, for AI: EE6-D0118 shows gain of 1.10%, 0.9% and 1.66% in terms of wPSNRY, deltaE100 and PSNRL100, shows loss of  0.31% and 0.35% in terms of tPSNRY and PSNRY;  for RA:  EE6-D0118 shows gain of 1.16%, 0.93% and 1.53% in terms of wPSNRY, deltaE100 and PSNRL100, shows gain of 0.3% and 0.33% in terms of tPSNRY and PSNRY;

- Comparing to EE6-D0124, for AI: EE6-D0118 shows gain of 0.3%, 0.09% and 0.89% in terms of wPSNRY,  deltaE100 and PSNRL100, shows loss of 0.12%, 0.13% and 0.08% in terms of tPSNRY, detlaE100 and PSNRY;   for RA:  EE6-D0118 shows gain of 0.24% and 0.79%% in terms of wPSNRY, and PSNRL100, , shows loss of 0.17%, 0.07% and 0.20% in terms of tPSNRY, deltaE100 and PSNRY.

Please note that JVET-E0055 only provided the QP set2 (Qp 22-37) result. For reference, the results from the contribution :

| Qp set2 |  | wPSNR Y | wPSNR U | wPSNR V | tPSNR Y | DE100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vs. EE-6 anchor | AI | -1.56% | -2.03% | -2.89% | 1.00% | -0.46% | -2.06% | 1.01% | 2.52% | 2.87% |
|  | RA | -1.31% | -3.10% | -5.24% | 1.12% | -0.53% | -1.64% | 1.11% | 2.59% | 1.78% |

For comparison, the Qp set2 result from the cross check was:

| Qp set2 |  | wPSNR Y | wPSNR U | wPSNR V | tPSNR Y | DE100 | PSNRL100 | PSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vs. EE-6 anchor | AI | -1.16% | -1.22% | -2.28% | 0.15% | -0.99% | -1.60% | 0.21% | 1.04% | 0.34% |
|  | RA | -1.06% | -2.18% | -3.38% | 0.23% | -0.76% | -1.32% | 0.24% | 0.90% | 0.24% |

We would suggest that the WPSNR result of the two results are relatively close, while the rest are not.

# Reference

[1] D. Rusanovskyy, D. B. Sansli, A. K. Ramasubramonian, M. Karczewicz, “EE6: Performance evaluation of adaptive scaling of JVET-D0118 for extended color volume material”, JVET-E0055, Jan. 2017.

[2] J. Zhao, A. Segall and K. Misrak, “EE6: On Dequantization and Scaling for Extended Colour Volume Materials”, JVET-E0081, Jan. 2017

[3] A. Segall, S. Lasserre, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-D1020, Oct.2016

.