| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Geneva, CH, 12–20 January 2017 | Document: JVET-E0134 |
| --- | --- |

| Title: | AhG8: Viewport-based subjective evaluation of 360-degree video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye, Philippe Hanhart, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4841 Xiaoyu.xiu@interdigital.com Yan.Ye@interdigital.com Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This document provides results on projection format conversion only tests. Projection formats including ERP, EAP, CMP, OHP, ISP, and SSP as supported in 360Lib are tested. Two test settings are included. In the first test setting, the 360 CTC according to JVET-D1030 is followed when selecting the face resolutions for each projection format. In the second test setting, face resolutions are selected for each projection format such that the number of active samples in the converted projection format is approximately 100%.

# Introduction

Two test settings are used. In the first test setting, all conditions are kept the same as CTC as described in JVET-D1030, including the face resolutions (Table 3 in JVET-D1030), except that the encoding/decoding is bypassed. Five quality metrics are calculated between the original ERP (8K or 4K) sequences and the reconstructed (that is, reconstructed after projection format conversion and back conversion but without coding) ERP (8K or 4K) sequences. These five quality metrics include: SPSNR-NN, WS-PSNR, SPSNR-I, CPP PSNR, and PSNR (conventional PSNR). Detailed results of the first test setting can be found in the “CTC size” tab of the attached “projection conversion only.xls”. Additionally, to visualize the conversion-only quality difference between a given projection format and the ERP projection format, an additional “visualization of projection conversion only.xls” is attached. The visualization calculates the delta quality difference against ERP for each of the quality metrics. For any positive delta difference (better quality than ERP) that is above 0.5 dB, the corresponding cell is highlighted as green. For any negative delta difference (worse quality than ERP) that is below -0.5 dB, the corresponding cell is highlighted as red. All other cells are not highlighted.

Because the 360 CTC performs downsampling of the original sequences in addition to projection format conversion, in order to remove the effect due to downsampling, we additionally ran tests for a second test setting. In the second test setting, the face resolutions for each projection format are selected such that the number of active samples in the converted projection format is approximately 100% of the original ERP source. Table 1 lists the face width and height for each format and each input resolution. In the second test setting, because there is no meaningful conversion for ERP format, we did not include ERP in the second test setting. Detailed results of the second test setting can be found in the “100%” tab of the attached  “projection conversion only.xls”..

Table 1 face width and height for each projection format

| Format | Resolution | FaceWidth | FaceHeight | Percentage |
| --- | --- | --- | --- | --- |
| EAP | 8K | 8192 | 4096 | 1.00 |
|  | 4K | 3840 | 1920 | 1.00 |
| CMP | 8K | 2368 | 2368 | 1.00 |
|  | 4K | 1104 | 1104 | 0.99 |
| OHP | 8K | 3112 | 2696 | 1.00 |
|  | 4K | 1496 | 1296 | 1.05 |
| ISP | 8K | 1968 | 1704 | 1.00 |
|  | 4K | 928 | 792 | 1.00 |
| SSP | 8K | 2368 | 2368 | 1.00 |
|  | 4K | 1104 | 1104 | 0.99 |