| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 3 March – 7 April 2017 | Document: JVET-F0001 |
| --- | --- |

| Title: | JVET AHG report: Tool evaluation (AHG1) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Qualcomm Inc. Elena Alshina Samsung Electronics | Email: Email: | martak@qti.qualcomm.com elena_a.alshina@samsung.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 5h JVET meeting at Geneva, Switzerland (12–20  January 2017) and the 6th Meeting at Hobart, Australia (31 March – 07 April 2017).

# Mandates

At the 5th JVET meeting, AHG1 on Tool evaluation was established with the following mandates:

- Coordinate the exploration experiments.

- Investigate interaction of tools in JEM and exploration experiment branches.

- Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime.

- Study and summarize new technology proposals.

# Summary of Activities

There was only one e-mail related to AhG1 activity sent to the JVET reflector. But there are multiple e-mails related to Exploration Experiments activity in separate mail-list, which has ~50 subscribers from 25 companies.

# JEM Status

Algorithms included into JEM5.0 are described in [1] and listed below. Tools modified at the JVCT-E meeting are marked as bold. The biggest change is addition of the third decoder side motion derivation technique (DMVR). Additionally to the Luma BD-rate gain some performance improvement of JEM5.0 over JEM4.0 is observed for Chroma due to the improvement both Intra mode coding and cross-color prediction for Chroma.

JEM5.0 tools:

- Block structure

  - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

  - Quadtree plus binary tree (QTBT) block structure

- Intra prediction improvements

  - 65 intra prediction directions

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction  improved in JEM5.0

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction improvements

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)  improved in JEM5.0

  - 1/16 pel motion vector storage accuracy

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction

  - Pattern matched motion vector derivation improved in JEM5.0

  - Bi-directional optical flow (BIO)

  - Decoder-Side Motion Vector Refinement (DMVR)  new in JEM5.0

- Transform

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms Signal dependent transform (SDT)  disabled by default

- In-loop filter

  - Adaptive loop filter (ALF)  improved in JEM5.0

  - Content adaptive clipping

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

Performance progress for JEM (HM-KTA) in terms of BD-rate gain vs. encoder time increase in random access test configuration is demonstrated on Figure 1. Results are based on Software Development AHG reports. Noticeable encoder run-time reduction is observed for JEM5.0 compared to JEM4.0 (due to adopted SW optimization, JVET-E0023), but still encoder run time is more than factor 10 compared to HM.

JEM to HM encoding time ratio

KTA-1  KTA-2    JEM1     JEM2      JEM3     JEM4     JEM5

Fig. 1. The progress of JEM performance in RA test configuration.

Table 1: Coding performance compared to HEVC summary.

JEM5.0 (5th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | -20% | -28% | -28% | 63 | 2 |
| Random Access | -29% | -35% | -34% | 12 | 10 |
| Low Delay-B | -22% | -29% | -29% | 10 | 8 |
| Low Delay-B | -26% | -31% | -32% | 7 | 5 |

JEM3.0 (4rd meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | -19% | -26% | -25% | 62 | 2 |
| Random Access | -28% | -33% | -32% | 13 | 11 |
| Low Delay-B | -22% | -27% | -28% | 10 | 8 |
| Low Delay-B | -25% | -30% | -31% | 8 | 5 |

Significant gain is observed in both three color components. In random access test highest gain over HEVC is observed for CatRobot test sequence (38.7%), lowest gain JEM shows for ToddlerFountain video (14.9% only).

# CfE test conditions

JVET Common Test Conditions use integer QP settings QP=22, 27, 32, 37 for all videos in test set. CfE draft document [3] requires bit-matching. Table 2 shows BD-rate performance JEM5.0.1 for video test set from CfE. The left part contains performance for QP =22, 27, 32, 37 and the right part is BD-rate calculated for bit-rates matching target rate within 2%. Under CfE test conditions JEM5.0.1 achieves 33.7% BD-rate gain over HM with Encoder run time 11 and decoder run time 9.

Table 2. JEM5.0.1 vs HM under CfE test conditions

| Reference: HM |  | QP=22,27,32,37 |  |  |  |  | bit-matching |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V |
|  |  | Random Access CfE Main10 |  |  |  |  |  |  |  |
| A1 | Crosswalk1 | -33.8% | -31.8% | -35.7% | 1324% | 763% | -37.1% | -42.3% | -46.0% |
|  | FoodMarket3 | -31.4% | -32.3% | -36.4% | 1300% | 745% | -34.2% | -46.3% | -48.7% |
|  | Tango1 | -33.6% | -44.3% | -44.6% | 1532% | 673% | -35.7% | -54.6% | -48.6% |
|  | CatRobot | -38.9% | -48.1% | -40.5% | 1371% | 945% | -39.8% | -52.2% | -45.1% |
| A2 | DaylightRoad | -38.5% | -45.2% | -30.8% | 1521% | 503% | -40.2% | -53.3% | -37.9% |
|  | BuildingHall1 | -30.4% | -28.5% | -32.2% | 949% | 752% | -32.8% | -40.9% | -46.0% |
|  | ParkRunning2 | -27.1% | -17.7% | -18.9% | 1364% | 812% | -31.4% | -26.3% | -29.5% |
|  | CampfireParty | -34.1% | -36.4% | -57.9% | 1924% | 371% | -37.3% | -35.5% | -56.1% |
| 2K | BQTerrace | -29.0% | -44.4% | -52.0% | 1368% | 823% | -29.7% | -50.8% | -61.1% |
|  | RitualDance | -25.9% | -31.0% | -35.7% | 1490% | 710% | -26.9% | -37.8% | -41.1% |
|  | Timelapse | -22.7% | -42.8% | -47.1% | 1043% | 982% | -26.4% | -45.4% | -50.7% |
|  | BasketballDrive | -30.2% | -43.1% | -42.2% | 1660% | 661% | -31.5% | -46.7% | -43.2% |
|  | Cactus | -31.9% | -40.1% | -32.4% | 1563% | 892% | -35.6% | -48.9% | -44.5% |
|  | CfE | -31.3% | -37.4% | -38.9% | 1096% | 911% | -33.7% | -44.7% | -46.0% |

# Exploration Experiments activity

At the 2nd JVET meeting Exploration Experiments practice was established. In 5th JVET meeting 3 EEs were created. For each new coding tool under consideration special SW branch was created. After implementation of each tool announcement via JVET reflector was done. For all 3 EEs input contribution for this meeting were submitted. Summary of exploration experiments is provided in [2].

# New tools contributions to this Meeting

Reduction for the number of technical contribution is observed. In total 16 contributions proposing new coding tools for JEM, improvements of JEM design or including comments on the existing design were submitted in following categories:

- Structure (2)

- Intra (4),

- Inter (5),

- Transform (3),

- Entropy (0)

- In-loop filter (1),

All non-EE proposals can be considered as simplifications and unification of existing JEM tools.

| JVET-VC number | Title | Authors | Category |
| --- | --- | --- | --- |
| JVET-F0024 | EE1: Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS | K. Panusopone, S. Hong, Y. Yu, L. Wang (ARRIS) | Intra |
| JVET-F0028 | EE3: BIO w/o block extension | A.Alshin, E. Alshina (Samsung) | Inter |
| JVET-F0029 | On memory bandwidth of JEM5.0 tools | E.Alshina (Samsung) | Inter |
| JVET-F0031 | Redundant syntax signaling reduction for transform skip | H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE) | Transform |
| JVET-F0032 | EE3: Enhanced FRUC Template Matching Mode | Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon) | Inter |
| JVET-F0033 | EE1-Related : UWP with Constrained PDPC and Constrained ARSS | H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE) | Intra |
| JVET-F0034 | EE2-JVET-E0032 Bilateral filter Test 1, Test2 | J. StrÃ¶m, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. SjÃ¶berg (Ericsson) | Transform |
| JVET-F0043 | NSST memory reduction | S.-H. Kim, H. Jang, J. Lim (LGE) | Transform |
| JVET-F0044 | AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1 | Y. Yasugi, T. Ikai(Sharp) | Inter |
| JVET-F0046 | AHG5: Performance evaluation of Adaptive Search Range on JEM-5.0.1 | Yukinobu Yasugi, Tomohiro Ikai | Inter |
| JVET-F0051 | Unified Adaptive Loop Filter for Luma and Chroma | J. An, J. Zheng (HiSilicon) | Loop filter |
| JVET-F0054 | Non-EE1: Alternative setting for PDPC mode | M. Karczewicz, N. Hu, X. Zhao, V. Seregin (Qualcomm) | Intra |
| JVET-F0055 | Non-EE1: Explicit flag signalling for ARSS | V. Seregin, X. Zhao, M. Karczewicz (Qualcomm) | Intra |
| JVET-F0063 | AHG5: Enhanced fast algorithm of JVET-E0078 | P.-H. Lin, C.-L. Lin, Y.-J. Chang, C.-C. Lin, Y.-H. Ju (ITRI) | Structure |
| JVET-F0074 | Fast QTBT Encoding Setting | Ismail Marzuki, Yong-Jo Ahn, Seanae Park, Donggyu Sim | Structure |

# Recommendations

The AHG recommends:

- Conduct viewing for visual quality comparison of JEM and HEVC during the meeting.

- Consider encoder complexity as one of the criteria when evaluating the tools. Encourage further encoder and decoder complexity reduction.

- To review all the related contribution.

- To continue Exploration Experiments practice.

# References

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce (JEM editors) “Algorithm Description of Joint Exploration Test Model 5”, JVET-E1001, Geneva, January 2017.

- E. Alshina, L. Zhang “Exploration Experiments on Coding Tools Report”, JVET-F0010, Hobart, March, 2017

- M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki , Revised Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC, JVET-F0040 Hobart, March, 2017