| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 3 March – 7 April 2017 | Document: JVET-F0010 | Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0010 |
| --- | --- | --- | --- |

| Title: | Exploration Experiments on Coding Tools Report |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | Elena Alshina, Li Zhang | Email: | elena_a.alshina@samsung.com, lizhang@qti.QUALCOMM.com |
| Source: |  | EE coordinators |  |  |

____________________________

# Abstract

Three experiments on coding tools were agreed to carry out between JVET-E and JVET-D meetings in order to get better understanding of technologies considered for inclusion to the next version of JEM, analyse and verify their performance, complexity and interaction with existing JEM tools. This report summarises the status of each experiment.

# Introduction

JVET group coordinated experiments were conducted following principles were agreed in [1].

Key announcement related to EE activities were done using JVET reflector. Some discussion about EE tools, EE-SW branches discussion was carried on EE mail list which has 40 subscribers from 19 companies.

Exploration experiments were conducted according to the agreed during JVET-E time-line:

T1= 2 weeks after JVET-E meeting = Feb. 3: To revise EE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector.

T2 = JEM5.0 SW release + 2 weeks = Feb.17: Integration of all tools into separate EE branch of JEM is completed. This was slightly delayed due to switching to JEM5.0.1 (bug fix version of reference SW) instead of JEM5.0.

T3: JVET-F meeting start – 3 weeks = Mar. 10: Final version for each tool is available, start of official cross-check.

Contributions [2]-[5] were included into Exploration experiments. They were tested under JVET CTC compared to JEM5.0.1.

Table 1. Summary of Exploration Experiments.

| # | Tests and sub-tests | Document | Y-BD-rate (Enc/DecTime) | Cross-check |
| --- | --- | --- | --- | --- |
| 1 | EE1: Intra prediction | JVET-F0024 (Arris) |  | JVET-F0045 (Sharp) JVET-F0047 (ETRI) JVET-F0056 (Qualcomm) |
|  | - Unequal Weight Planar (UWP) |  | AI: -0.1% (ET 0.99, DT 1.00) |  |
|  | - UWP + Constrained PDPC4 (Dec) |  | AI: 0.0% (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained PDPC4 (Enc) |  | AI: 0.2% (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained PDPC3 (Dec) |  | AI: 0.0 % (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained PDPC3 (Enc) |  | AI: 0.2% (ET 0.91, DT 1.00) |  |
|  | - UWP + Constrained ARSS (Dec) |  | AI: 0.0% (ET 0.85, DT 1.00) |  |
|  | - UWP + Constrained ARSS (Enc) |  | AI: 0.2% (ET 0.85, DT 1.00) |  |
| 2 | EE2: Non-linear in-loop filters - Reduced LUT size | JVET-F0034 (Ericsson) | AI: -0.4% (ET 1.06, DT 1.03) RA: -0.5% (ET 1.02, DT 0.99) LD: -0.5% (ET 1.02, DT 1.00) LDP: -0.6% (ET 1.04, DT 1.01) | JVET-F0048 (ETRI) JVET-F0057 (Qualcomm) JVET-F0073 (Intel) |
| 3 | EE3: Decoder Side Motion Vector Derivation |  |  |  |
|  | - Enhanced Template Matching in FRUC (with aspect 1 and 2 ) - Enhanced Template Matching in FRUC (with aspect 2 only) | JVET-F0032 (Hi-Silicon) | RA: -0.4% (ET 0.99, DT 1.02) LD: -0.1% (ET 1.00, DT 1.02) RA: -0.3% (ET 0.99, DT 1.01) LD: -0.1% (ET 1.00, DT 1.01) | JVET-F0048 (ETRI) JVET-F0071 (Hulu) |
|  | - BIO w/o block extension | JVET-F0028 (Samsung) | RA: 0.1% (ET 0.95, DT 0.88) LD: 0.1% (ET 0.98, DT 0.95) | JVET-F0059 (Qualcomm) |

# List of Experiments

## EE1: Intra prediction

JVET-F0024 EE1: Unequal Weight Planar Prediction, Constrained PDPC, and Constrained ARSS [K. Panusopone, S. Hong, Y. Yu, L. Wang (ARRIS)]

Original contribution proposes replacement of planar mode in JEM by unequal weight planar prediction (UW-Planar). Compared to HEVC Planar prediction 2 multiplications per sample are needed. 2 multiplications per sample in Bottom Row and Right Column calculation also added. Unequal weight parameters are MaxblockSize*2+1 10 bits unsigned constants.

Additionally restriction on Position Dependent Intra Prediction combination mode (constrained PDPC) and Adaptive reference sample smoothing (constrained ARSS) are studied. Specifically, UW-Planar employs bottom-right position adjustment and unequal weight assignment for final predictor calculation.

Constrained PDPC allows PDPC mode only for intra CU that employs one of the following 4 angular modes: modes 2, 18, 34, and 50 (test 2 and 3) of one of the following 3 angular modes: 2, 18 and 50 (Test 4 and 5). Constrained PDPC was tested with syntax change (such that PDPC flag is only sent in the case of the mentioned four modes) and as encoder only modification.

Constrained Adaptive reference sample smoothing allows ARSS under certain condition:

diff > m_aucIntraFilter[toChannelType(compID)][sizeIndex]

- ( (compID==COMPONENT_Y && enableRSAF && sizeIndex==1) ? 2 : 0)

Constrained ARSS was tested with syntax change (such that ARSS flag is hidden in parity of coefficients magnitudes sum) and as encoder only modification.

Questions recommended to be answered during EE tests.

[Q]: Investigate options of encoder-only restriction of PDPC?

[A]: For both variant of constrained PDPC (3 or 4 modes use PDPC) syntax modification shows 0.2% better performance compared to encoder only modification.

[Q]: Check the tradeoff with different numbers of restricted modes.

[A]: Tests 2-3 examine PDPC constrained for 4 modes, in tests 4-5 PDPC was allowed for 3 angular modes.

Test 2 (UW-Planar+CPDPC4) shows a coding gain of 0.01% with encoder time of 91% as compared to the anchor.

Test 4 2 (UW-Planar+CPDPC3)s hows a coding loss of 0.01% with encoder time of 91% as compared to the encoder.

In addition, without UW planar, it is also observed that if PDPC is restricted for 4 modes, the average performance drops by 0.38% (with encoder time 92%) for AI as compared to the anchor

[Q] Also check tradeoff with ARSS, and different numbers of restricted modes

[A] Tests 6-7 examine constrained ARSS.

If ARSS is restricted then avg. performance drop  in AI test is 0.11% (86% Enc.time)

Summary: Use of unequal weight Planar, instead of current Planar, improves JEM performance by 0.15% (in AI test), with little increase in computations and memory. .. Constrained PDPC and ARSS with syntax modification is 0.2% better than encoder only change (w/o syntax). Combination of unequal weight Planar constraining PDPD for only 4 angular modes with syntax modification leads to 9% Encoder run time reduction and 0.01% BD-rate gain in AI test as compared to the JEM5.0. Combination of unequal weight Planar and constraining ARSS with syntax modification leads to 14% Encoder run time reduction and 0.04% BD-rate gain in AI test as compared to the anchor..

## EE2: Non-linear in-loop filters

JVET-F0034 EE2-JVET-E0032 Bilateral filter Test 1, Test2 [J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson)]

The bilateral filter operation is suggested to be performed on decoded sample values directly after the inverse transform. The proposed bilateral filter is a five-tap filter in the shape of a plus sign. The strength of the filter is based on coded mode (intra/inter), TU size and QP. The size of the table is 56148 bytes (16 bits precision of filter coefficients). The filter is automatically disabled when there is no non-zero transforms coefficients. LUT operation implementation is used to reduce the complexity of the described operations. No additional parameters are determined during encoding and no new syntax elements are proposed. LUT operation makes complexity manageable.

Questions recommended to be answered during EE tests.

[Q]: What the size of the lookup table would be?

[A]: Test 1 examines performance original proposal (LUT size 56148 bytes). Test 2 examines performance of the algorithm for reduced size of LUT 2202 bytes (5 bits precision per coefficient).

|  | LUT size = 56148 bytes |  |  |  |  | LUT size = 2202 bytes |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| AI | -0.4 | 0.1 | 0.1 | 106% | 104% | -0.4 | 0.2 | 0.1 | 106% | 103% |
| RA | -0.5 | 0.1 | 0.1 | 103% | 98% | -0.5 | -0.2 | -0.2 | 102% | 99% |
| LD | -0.5 | 0.4 | 0.2 | 103% | 100% | -0.5 | 0.3 | 0.2 | 102% | 100% |
| LDP | -0.6 | 0.1 | 0.3 | 104% | 101% | -0.6 | 0.4 | 0.4 | 104% | 101% |

[Q]: It is recommended to further reduce the size of LUT

[A]: Test 2 was designed for those purposes.

Summary: 0.4%, 0.5%, 0.5% and 0.6% gain is observed for AI, RA, LD and LD-P respectively from bi-lateral filtering after inverse transformation with LUT size 2202 bytes. Encoder run-time increment ranges from 2% to 6% and decoder run time increment ranges from 0% to 3%.

## EE3: Decoder Side Motion Vector Derivation

JVET-F0032 EE3: Enhanced Template Matching in FRUC Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

The enhancement to FRUC template matching mode is proposed. An identified issue is that motion information derived by the existing uni-directional template matching is unreasonably used for bi-prediction in the FRUC template matching mode. This issue can be solved by the following proposed methods: 1) bi-directional template matching which jointly uses list0 and list1 reference pictures; 2) selection between uni-prediction and bi-prediction based on template matching distortion.

get template T0 matched to current template Tc from list0 reference pictures  ( motion info MV0, cost0 )

get template T1 matched to updated template T’C from list1 reference pictures ( motion info MV1, cost1 )

Bi-prediction based on MV0 and MV1

Update current template:

T’C = 2*TC – T0

cost0 < 0.5*cost1

Uni-prediction based on MV0

get template T0 matched  to current template Tc from list0 reference pictures ( motion info MV0 )

get template T1 matched to current template Tc from list1 reference pictures  ( motion info MV1 )

Bi-prediction based on MV0 and MV1

- Proposed FRUC template matching mode

- Existing FRUC template matching mode

Y

N

One difference compared to current FRUC is the fact that the template based search cannot be implemented in parallel but must be performed sequentially for the two reference pictures.

Questions recommended to be answered during EE tests.

[Q]: Check gain on top of JEM5.0 (is it additive to JVET-E meeting adoptions)?

[A]: original contribution tested vs JEM4.0 provided 0.38% (RA) / 0.12%(LDB) gain.

On top of JEM5.0 gain is 0.36% (RA) and 0.14% (LDB) (aspects 1 and  2)

On top of JEM5.0 gain is 0.32% (RA) and 0.08% (LDB) (only aspect 2)

Summary: Major gain come from selection between uni-prediction and bi-prediction based template matching: 0.3%(RA) / 0.1% (LD);  run-time increment 1-5% (encoder) and 1-2% (decoder).

JVET-E0028 EE3: bi-directional optical flow w/o block extension [A. Alshin, E. Alshina (Samsung)]

In JEM5.0 BIO performs prediction and calculates gradients for the extended block WH  (W+4)  (H+4). Block extension has been removed. After this modification the memory bandwidth of BIO is equal to regular bi-directional motion compensation. Computational complexity also has been reduced which results in both encoder and decoder run time reduction.

Questions recommended to be answered during EE tests.

[Q]: Test results with weights non-depending on PU size are requested,

[A]: Proposal was tested with all weights equal to 1. Original proposal has no performance drop in RA and 0.1% performance drop for LD. With all weight equal to 1 drop is 0.1% in both R and LD test.

Summary: With 0.1%performance drop in RA and LD encoder run time can be reduced by 5% and 2% respectively, decoder run time can be reduced by 12% (RA) and 5% (LD) if BIO doesn’t use block extension. Memory for modified BIO is equivalent to the regular bi-directional MC.

# Recommendation

- Review ALL EE tests’ contributions and cross-checks.

- Make decisions on each part on technology under consideration by reviewing EE results.

# Acknowledgement

- Thanks all proponents for providing EE s/w in time with proper announcement and explanation about modification in JVET reflector.

- Thanks all volunteers provided comprehensive cross-check.

# References

[1] E. Alshina, and L. Zhang, “Description of Exploration Experiments on Coding Tools,” JVET-E1011, January 2017.

[2] K. Panusopone, S. Hong, and L. Wang, “Unequal Weight Planar Prediction and Constrained PDPC,” JVET-E0068, January 2017.

[3] J. Ström, P. Wennersten, K. Andersson, J. Enhorn, “Bilateral filter strength based on prediction mode,” JVET-E0032, January 2017.

[4] Y. Lin, X. Chen, J. An, J. Zheng, “Enhanced Template Matching in FRUC Mode,” JVET-E0035, January 2017.

[5] A. Alshin, E. Alshina, “EE3: bi-directional optical flow w/o block extension,” JVET-E0028, January 2017.