| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 3 March – 7 April 2017 | Document: JVET-F0030 |
| --- | --- |

| Title: | Cross-check for block-based design for Bi-directional optical flow (JVET-F0022) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E.Alshina Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

Block based design for BIO is verified in this report. Original proposal includes 3 modifications: redesign of MC pipeline in case of OBMC and BIO, clustering samples for MV refinement into 44 groups, MV refinement information storage and update. Performance and complexity impact for both three modifications is evaluated in this report.

# Introduction

JEM includes 3 so-called decoder side motion derivation techniques: FRUC, BIO and DMVR Both three provide great performance but introduce not negligible complexity. Proposal verified in this report suggests several simplifications for BIO in JEM5.0. Among them:

- Redesign of the CU processing pipeline for the case of OBMC (in order BIO will be applied only once, not up to five times as in JEM);

  - This is reasonable JEM design change, pure simplification, it introduces 0.2% performance degradation while saves 10% encoder and 19% decoder run-time,

- Clustering samples to 44 groups for MV BIO refinement

  - This is reasonable modification of BIO for high resolution videos, since block’s size goes high sample-wise MV refinement is overkill. While for the small blocks and small resolution videos sample-wise MV refinement seams useful. This change leads to 0.1~0.2% performance degradation for small resolution classes D and C, but provides 0.1% gain in class A1 Clustering samples in BIO to 44 groups leads to reduces the number of divisions by factor 16. So run time reduction is 3% at encoder and 6% at decoder side.

- MV refinement information storage for 44 cluster of samples and up-date

  - This change requires storage [(MaxBlockSize>>2) (MaxBlockSize>>2)] but helps performance improvement by 0.2% in average.

In average three changes together provide 0.0% performance impact (with 0.1 and 0.3% drop in class C and D but 0.1% gain for higher resolution classes. Decoder runtime reduction is 10%, encoder speed up is 8%.

# Test results

Code for JVET-F0022 was uploaded together with the contribution [1]. It was modified (by cross-checker) in order to test each proposed modification separately (by cross-checker). Because of the lack of time only RA testing was performed.

Test results are summarized in Table 1

Test1. Performance summary for all tests

|  | Only OBMC |  |  |  |  |  | Clustering 44 in BIO |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 | 0.1% | -0.1% | 0.0% | 93% | 84% |  | -0.1% | -0.2% | -0.1% | 98% | 96% |
| Class A2 | 0.2% | 0.0% | 0.0% | 90% | 80% |  | 0.0% | 0.0% | -0.1% | 97% | 95% |
| Class B | 0.1% | 0.1% | 0.1% | 88% | 78% |  | 0.0% | 0.0% | 0.0% | 95% | 93% |
| Class C | 0.2% | 0.0% | 0.0% | 90% | 79% |  | 0.1% | 0.0% | 0.0% | 96% | 92% |
| Class D | 0.3% | -0.2% | 0.0% | 89% | 81% |  | 0.2% | 0.0% | 0.1% | 96% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall JVET | 0.2% | 0.0% | 0.0% | 90% | 81% |  | 0.1% | 0.0% | 0.0% | 97% | 94% |
| Class F | 0.1% | 0.1% | 0.0% | 90% | 86% |  | 0.0% | 0.0% | 0.1% | 93% | 93% |

|  | F0022 as it is |  |  |  |  | F0022 w/o MV refinement up-date |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.1% | -0.4% | -0.3% | 95% | 96% | 0.0% | -0.3% | -0.1% | 92% | 82% |
| Class A2 | -0.1% | -0.2% | -0.3% | 93% | 92% | 0.2% | 0.0% | -0.1% | 89% | 78% |
| Class B | -0.1% | -0.2% | -0.1% | 89% | 88% | 0.1% | 0.0% | 0.0% | 86% | 75% |
| Class C | 0.1% | -0.2% | -0.2% | 91% | 87% | 0.3% | 0.1% | 0.1% | 90% | 77% |
| Class D | 0.3% | -0.2% | -0.2% | 90% | 86% | 0.6% | 0.1% | 0.0% | 87% | 78% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall JVET | 0.0% | -0.2% | -0.2% | 92% | 90% | 0.2% | 0.0% | 0.0% | 89% | 78% |
| Class F | 0.0% | 0.0% | 0.0% | 92% | 91% | 0.1% | 0.1% | 0.1% | 90% | 85% |

There was discussion about potential the combination of JVET-F0022 [1] and JVET-F0028 [2] among EE subscriber. Some aspects on [1] and [2] were already combined in published PCS2016 paper [3].

# Conclusions

The Major complexity reduction (10% encoder and 19% decoder run time saving) comes due to simplification of OBMC and BIO integration in JEM5.0. This change leads to the performance drop of 0.2%. The clustering of samples in BIO into 44 groups leads to the performance degradation 0.1%. Major benefit of the clustering is reduction of division operations by factor 16. MV refinement storage and refinement helps resolving performance drop while requires additional memory.

# References

[1] H.-C. Chuang, J. Chen, X. Li, Y.-W. Chen, M. Karczewicz, W.-J. Chien (Qualcomm) “EE3-related: A block-based design for Bi-directional optical flow (BIO),” JVET-F0022, Jan. 2017, Geneva

[2] A.Alshin, E.Alshina, BIO w/o block extension, JVET-F0028, Jan. 2017, Geneva

[2] A.Alshin, E.Alshina, BIO performance complexity trade-off, PCS 2016.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).