| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0072 |
| --- | --- |

| Title: | AHG8: Stretching ratio based adaptive quantization for 360 video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun, Lu Yu Zhejiang University Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

To make the coding distortion in spherical space uniform, stretching ratio based adaptive quantization for 360 video is proposed. Different CTUs in images are assigned with different quantization according to the stretching ratio. The experimental results show that this proposed method achieves average gain is -2.15%, -2.22% for E2E WS-PSNR Y and WS-PSNR (encoder) Y respectively.

Stretching ratio based adaptive quantization

In common test condition, QP for each coded frame is fixed. However coding errors (MSE) in projection formats in different locations are different by using fixed QP in the whole frame. The reason is that stretching ratio (sample interval) is different on projection formats, which results in that (1) is not correct for 360 video. As an example, for equirectangular projection (ERP), coding distortion of CTU near the pole is much smaller than that near the equator.

Assuming residual distribution is uniform, the relationship between Qstep and distortion D (MSE) is:
D=Qstep212                (1)

For 360 video, (1) should be revised and the only influence factor is stretching ratio, which is equal to weights derived in WS-PSNR [1]. Hence the (1) could be revised as:

D'=w*Qstep212   (2)

where w is the stretching ratio (sampling interval), which is the weight derived by WS-PSNR.

To make the coding distortion in spherical space uniform, the Qstepshould be revised:

Qstep(i)'=wi*Qstep(i)  (3)

Stretching ratio based adaptive quantization is implemented in CTU-level in this proposal, so wi is the average stretching level for i-th CTU. Detailed implementation is shown below.

The relationship between Qstep and QP in HEVC is:

Qstep=2QP-46        (4).

To make distortion of 360 video in sphere is uniform, each CTU needs to adjust the baseQP according (3) and (4):

1Wi=2(deltaQP)/3       (5)

And QP is integer, deltaQP=int（3log21w+0.5).

To normalize the weights which change the quality in different CTUs, deltaQP_ave is calculated by average weights of all CTUs: deltaQP_ave=int（3log21w_ave+0.5). Finaly, QP_offset for each CTU can be obtained:

QP_offset=deltaQP-deltaQPave     (5).

For ERP, Weights of CTU can be approximately calculated by:

Wj=cos32+j*64-a2-12*πa              (6)

where W(j) is the weight of CTU in j-th row. (The resolution of equirectangular format is defined as 2a*a).

For an image of 3328*1664 in ERP, QP_Offset and ideal ratio of D(MSE) in different CTU row are show below.

Figure.1 QP_offset in each CTU row

Figure.2 ratio of MSE in each CTU row

Experimental results and analysis

The tests are conducted according to the JVET common test conditions and on HM 16.15-360Lib with RA.

| Proposed vs. ERP | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 0.6% | 2.2% | 1.3% | 0.6% | 2.2% | 1.4% | 0.6% | 2.2% | 1.4% | 0.6% | 2.2% | 1.4% |
| GasLamp | 0.7% | 2.0% | 3.8% | 0.7% | 2.0% | 3.8% | 0.7% | 2.0% | 3.8% | 0.7% | 2.0% | 3.8% |
| Skateboarding_in_lot | -4.9% | -7.0% | -4.7% | -4.9% | -7.0% | -4.7% | -4.9% | -7.0% | -4.8% | -4.9% | -6.9% | -4.8% |
| Chairlift | -4.9% | -1.5% | -0.1% | -4.9% | -1.5% | -0.1% | -4.8% | -1.4% | -0.1% | -4.8% | -1.4% | -0.1% |
| KiteFlite | -1.2% | 1.1% | 0.5% | -1.2% | 1.1% | 0.5% | -1.3% | 1.0% | 0.4% | -1.3% | 1.0% | 0.5% |
| Harbor | 0.7% | 3.1% | 3.1% | 0.8% | 3.1% | 3.1% | 0.7% | 3.1% | 3.2% | 0.7% | 3.1% | 3.2% |
| PoleVault | -5.4% | -2.1% | -1.8% | -5.4% | -2.0% | -1.7% | -5.4% | -2.0% | -1.7% | -5.3% | -2.0% | -1.8% |
| AerialCity | -2.3% | 0.8% | 0.4% | -2.3% | 0.8% | 0.4% | -2.2% | 0.9% | 0.4% | -2.2% | 0.8% | 0.4% |
| DrivingInCity | 0.6% | 6.8% | 6.0% | 0.6% | 6.8% | 6.0% | 0.6% | 6.8% | 6.0% | 0.6% | 6.8% | 6.0% |
| DrivingInCountry | -5.5% | -4.4% | -3.9% | -5.4% | -4.5% | -4.0% | -5.5% | -4.6% | -4.0% | -5.5% | -4.4% | -3.9% |
| Overall | -2.2% | 0.1% | 0.5% | -2.2% | 0.1% | 0.5% | -2.2% | 0.1% | 0.5% | -2.1% | 0.1% | 0.5% |

| WS-PSNR (codec) |  |  | PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V |
| 0.6% | 2.3% | 1.5% | 3.2% | 6.1% | 3.3% |
| 0.6% | 2.0% | 3.8% | 4.1% | 5.8% | 9.2% |
| -5.2% | -6.8% | -4.7% | 9.0% | -1.9% | -3.8% |
| -5.0% | -1.3% | 0.0% | 13.9% | 17.3% | 9.9% |
| -1.3% | 1.1% | 0.5% | 6.7% | 7.4% | 6.3% |
| 0.8% | 3.1% | 3.2% | 5.5% | 7.1% | 7.2% |
| -5.5% | -2.2% | -1.9% | 5.8% | 11.7% | 9.9% |
| -2.3% | 1.0% | 0.6% | 5.8% | 7.4% | 7.8% |

Figure.3 is an ERP image which is divided into 3 kinds of color block. Blue area means normalized weights are less than 1, where QPoffset is positive. The normalized weights in Yellow area equals to 1 and QPoffset is 0. And normalized weights in white are larger than 1 and QPoffset is negative.

Figure.3 Normalized weight of ERP

The change of RD-performance for ERP is shown in below table. For blue area, as QPoffset is positive, which results in D2>D1and R2<R1. If we don’t consider the weight, RD performance can be considered unchanged as only QP is changed. But actually, RD-perfomance is related to weight. The reduction of RDcost is: (1-weight)*D1 < (1-weight)*D2. So RD-Performance is better with weight. White area becomes better too as QPoffset is negative.

|  | Fixed QP | Adaptive QP | Relationship | RD-Performance without weight | RD-Performance with weight |
| --- | --- | --- | --- | --- | --- |
| Blue area | D1, R1 | D2,R2 | D2>D1,R2<R1 | UNCHANGED | BETTER |
| Yellow area | D3,R3 | D4,R4 | D3=D4,R3=R4 | UNCHANGED | UNCHANGED |
| White area | D5,R5 | D6,R6 | D6<D5,R6>R5 | UNCHANGED | BETTER |

Conclusion

Stretching ratio based adaptive quantization for 360 video is proposed to make the coding distortion in spherical space uniform. As an example, ERP is tested in this proposal. The experimental results report that the proposed method achieves -2.15%, -2.22% for E2E WS-PSNR Y and WS-PSNR (encoder) Y respectively. And other formats can also be applied to adjust quantization according to weight map derived in WS-PSNR. What is more, the influence of stretching ratio on quality is suggested to be considered in viewport-based subjective test.

References

[1] Y. Sun, A. Lu, L. Yu, “AHG8: WS-PSNR for 360 video objective quality evaluation,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0040, Oct. 2016, Chengdu, China.

Patent rights declaration(s)

Zhejiang university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).