| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, July 13-21, 2017 | Document: JVET-G0051 |
| --- | --- |

| Title: | AHG8: A study on quality impact of line re-sampling rate in EAP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA 92127, USA | Tel: Email: | [+1 858 521 5838] minhua.zhou@broadcom.com |
| Source: | Broadcom Limited |  |  |

_____________________________

# Abstract

This contribution studied the quality impact of line re-sampling resolution in the Equal Area projection (EAP), and introduced a line re-sampling resolution control parameter to improve the EAP performance. Experimental results per the 360 video common test conditions revealed that the modified EAP outperforms the ERP on average by 4.6%, 3.5% and 3.6% (E2E SPSNR-NN, RA-configuration) in Y, U, V, respectively.

# Introduction

Among all the alternative 360 video projection formats being investigated in the JVET, the EAP (Equal Area Projection) is the only format that exhibits overall loss against ERP according to the 360 video quality metrics defined in the JVET 360 common test conditions [3].  This contribution introduces a modification to improve the performance of the EAP.

Referring to definitions described in [1], for 3D-to-2D coordinate conversion the ERP is defined as

u= ∅ 2π+0.5v=-θπ+0.5             Equation 1

While the EAP is defined as

u= ∅ 2π+0.5                 v=-0.5*sinθ+0.5        Equation 2

Where ∅ is longitude in the range of [-π: π], and θ is latitude in the range of [-π2  :π2  ], respectively.

Figure1. Example of a 360 video picture: (a) ERP and (b) EAP

An example of a 360 video picture in ERP and EAP is depicted in Figure 1. Compared to the ERP, in EAP the sample lines are squeezed around the North and South Pole (i.e. the top and bottom parts of a 360 video picture), and are stretched along the equator (i.e. the middle part of a 360 video picture).  This can be explained by deriving the line resampling rate (i.e. ∂v∂θ) from Equation 1 and 2.  For ERP, the number of sample lines per latitude degree is a constant over the entire latitude range -π2  :π2  ,  which can be derived from Equation 1:

∂v∂θ=1π   ,     θϵ[-π2  :π2  ]    Equation 3

While for EAP it is a cosine function of latitude as it can be derived from Equation 2:

∂v∂θ=0.5*cosθ  ,     θϵ[-π2  :π2  ]   Equation 4

As shown in Equation 4, in EAP the number of sample lines per latitude degree (i.e. line resampling resolution) around the North Pole (i.e. θ=π2) and the South Pole (i.e. θ=-π2) is approaching to zero, which may cause viewport quality issues in viewing directions near the North and South Pole, and may also explain the overall loss of EAP against ERP due to ultra-low line resampling resolution along the North and South Pole in the projection.

# Adjusted Equal Area Projection (AEP)

Referring to [1], for 2D-to-3D coordinate conversion, we first start from a given sampling position (m, n), and calculate (u, v) using (6) and (7) of [1], that is

| u = (m + 0.5) / W, 0≤ m <W |  |
| --- | --- |
| v = (n + 0.5) / H, 0 ≤ n < H |  |

Then, the longitude and latitude (ϕ, θ) on the sphere can be calculated from (u, v) as:

∅=u-0.5*2*π                                              θ=1β*sin-1((1.0 - 2*v)*sin(0.5* π*β))                         Equation 5

Finally, (X, Y, Z) can be calculated using Equations (1) (2) (3) of [1].

For 3D-to-2D coordinate conversion, given (X, Y, Z), first (ϕ, θ) is calculated using Equations (4) (5) of [1], then calculate (u, v) using

u=∅2*π+0.5             v=-0.5*sinβ*θsin0.5*β*π+0.5                                                                    Equation 6

Finally, (m, n) is calculated by solving Equations (6) and (7) of [1].

Here β ∈(0:1] is a parameter which controls line resampling rate in the projection. As it can be seen from the projection format definition of Equation 6, the equal area projection (EAP) is just a special case of AEP in which β=1.

Figure 2. Example of a 360 video picture: (a) EAP and (b) AEP (β=11.4)

Figure 2 provides an example of comparison between the EAP and AEP of  β=11.4.  As shown in Figure 2, in AEP the line resampling resolution along the North and South Pole is better preserved.

# Experimental results

The adjusted equal area projection format described in section 2 was implemented into the HM16.15 + JVET 360Lib-3.0 library [2] for investigation. Per common test conditions described in [3], the random access (RA) results for β=11.4 are summarized in Table 1.

Table 1. RA BD-rates per common test conditions for β=11.4

For studying the coding efficiency impact of line resampling rate in projection, different β values were tried out. The following table summarizes the coding efficiency of AEP w.r.t. β.

Table 2. RA BD-rates (E2E PSNR-NN) per common test conditions for different β values

The HM “lossless” coding mode was used to measure the conversion only quality, for β=11.4 the results are shown in Table 3. For E2E WSPSNR the AEP (with β=11.4) the conversion only quality is around 46.50 dB for Luma, which is about 0.87 dB and 2.46 dB higher than that of ERP (45.63 dB) and of EAP (44.14 dB), respectively.

Table 3. Conversion only quality of the AEP (β=11.4)

The detailed results are provided in the attached spreadsheets.

# Conclusions

The experimental results revealed that compression efficiency can be improved by properly adjusting the line resampling rate in single face projection formats such as ERP and EAP.

The EAP appears to have too low line re-sampling resolution along the North and South Pole, which leads to overall loss when compared to ERP. It is recommended to extend the EAP in the 360 Lib with the AEP (β=11.4) described in this contribution and use it as a new reference data point for EAP during the further exploration of 360 video projection formats.

# References

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET-F1003, April 2017.

- 360Lib-3.0: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-3.0/

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, JVET-F1030, April 2017.

# Patent rights declaration(s)

Broadcom Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).