| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0055 |
| --- | --- |

| Title: | Test Sequences for Virtual Reality Video Coding from InterDigital |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Eduardo Asbun, Yong He, Philippe Hanhart, Yuwen He, Yan Ye 9710 Scranton Rd, Suite 250 San Diego, CA 92121, USA | Email: | Ed.Asbun@InterDigital.com |
| Source: |  | InterDigital Communications, Inc. |  |  |

____________________________

# Abstract

This contribution makes three Virtual Reality (VR) video sequences available to the JVET.  The video sequences are provided in 6K (6144x3072) and 8K (8192x4096) resolutions at 60 fps.

# Introduction

360° video is a rapidly growing format emerging in the media industry. It is enabled by the rising availability of VR devices and able to provide the viewer with an immersive experience.

# Sequences

The contributed sequences were captured by InterDigital using GoPro Omni rig, a synchronized camera array with 6 GoPro Hero4 Black cameras. Kolor Autopano Video Pro is used to stitch the video. No encoding is done after stitching; therefore, the stitched content is provided in uncompressed raw YUV format.

For recording these sequences, each camera was set at a resolution of 1920x1440, the maximum resolution able to record at 60 fps. Therefore, an output resolution of 6K (6144x3072) was selected to minimize interpolation during rendering.

The following 6K sequences are provided in equirectangular projection (ERP):

| Sequence | Resolution@FPS | Bit Depth | Frames |
| --- | --- | --- | --- |
| balboa360_6144x3072_60fps_600frames_8bits.yuv | 6144x3072@60 | 8 | 600 |
| broadway360_6144x3072_60fps_600frames_8bits.yuv | 6144x3072@60 | 8 | 600 |
| horton360_6144x3072_60fps_600frames_8bits.yuv | 6144x3072@60 | 8 | 600 |

Sequences were also re-rendered at 8K (8192x4096) resolution. The following 8K sequences are provided in equirectangular projection (ERP):

| Sequence | Resolution@FPS | Bit Depth | Frames |
| --- | --- | --- | --- |
| balboa360_8192x4096_60fps_600frames_8bits.yuv | 8192x4096@60 | 8 | 600 |
| broadway360_8192x4096_60fps_600frames_8bits.yuv | 8192x4096@60 | 8 | 600 |
| horton360_8192x4096_60fps_600frames_8bits.yuv | 8192x4096@60 | 8 | 600 |

Due to the resolution of input video (1920x1440) from each camera, the stitching software relies on interpolation to render at 8K. Therefore, the 8K versions of the contributed sequences are not “true 8K”.

# Sequence Thumbnails

- balboa360_6144x3072_60fps_600frames_8bits.yuv

- broadway360_6144x3072_60fps_600frames_8bits.yuv

- horton360_6144x3072_60fps_600frames_8bits.yuv

# Results

The three  contributed sequences were encoded in 4K ERP format at four different QP values using HM-16.15 and JM-6.0 following the CTC [1][2]. The table below summarizes the bit rate and end-to-end WS-PSNR (E2E WS-PSNR) values of HM and JM for each QP value, as well as the BD-rate for each sequence.  The Excel file that accompanies this document contains the full set of results.

|  |  | HM-16.15 |  |  |  | JEM-6.0 |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | QP | kbps | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V | kbps | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V | E2EWS-PSNR Y | E2EWS-PSNR U | E2EWS-PSNR V |
| Balboa | 22 | 35402 | 42.76 | 51.36 | 51.48 | 30017 | 43.16 | 51.26 | 51.42 | -33.6% | -37.1% | -41.5% |
|  | 27 | 13901 | 39.93 | 49.62 | 49.85 | 11307 | 40.49 | 49.92 | 50.24 |  |  |  |
|  | 32 | 6252 | 37.32 | 47.80 | 48.13 | 4958 | 37.99 | 48.69 | 49.16 |  |  |  |
|  | 37 | 2881 | 34.77 | 46.19 | 46.60 | 2248 | 35.53 | 47.77 | 48.30 |  |  |  |
| Broadway | 22 | 43919 | 41.91 | 50.74 | 50.87 | 38077 | 42.29 | 50.67 | 50.85 | -33.6% | -39.4% | -42.5% |
|  | 27 | 16270 | 39.24 | 48.89 | 49.18 | 13434 | 39.77 | 49.30 | 49.63 |  |  |  |
|  | 32 | 7378 | 36.76 | 47.04 | 47.46 | 5841 | 37.44 | 48.08 | 48.57 |  |  |  |
|  | 37 | 3506 | 34.17 | 45.44 | 45.99 | 2709 | 34.99 | 47.13 | 47.69 |  |  |  |
| Horton | 22 | 36140 | 42.34 | 50.76 | 50.39 | 31748 | 42.68 | 50.80 | 50.49 | -32.5% | -43.8% | -45.0% |
|  | 27 | 12615 | 39.78 | 49.00 | 48.71 | 10677 | 40.27 | 49.51 | 49.22 |  |  |  |
|  | 32 | 5446 | 37.47 | 47.11 | 46.88 | 4383 | 38.08 | 48.32 | 48.07 |  |  |  |
|  | 37 | 2592 | 35.02 | 45.43 | 45.33 | 2003 | 35.80 | 47.23 | 47.01 |  |  |  |
| All |  |  |  |  |  |  |  |  |  | -33.20% | -40.11% | -42.98% |

# Sequence MD5 Sums

Uncompressed 8-bit YUV420 sequences in 6K and 8K resolution in equirectangular projection will be uploaded to MPEG content repository. The MD5 of each sequence is listed below:

| Sequence | MD5 |
| --- | --- |
| balboa360_6144x3072_60fps_600frames_8bits.yuv | 1457bb109ae0d47265f5a589cb3464d7 |
| balboa360_8192x4096_60fps_600frames_8bits.yuv | 32fa87fe48e0926fad1019e912bdc8f2 |
| broadway360_6144x3072_60fps_600frames_8bits.yuv | 6b01584be2c806db5c06656ec10217b9 |
| broadway360_8192x4096_60fps_600frames_8bits.yuv | 99a21fb108a88341e2d50f83be89db94 |
| horton360_6144x3072_60fps_600frames_8bits.yuv | 1baf6667ef84bc3534809463faee8e82 |
| horton360_8192x4096_60fps_600frames_8bits.yuv | 42573904d6662a0e61536e50d40c236d |

# Copyright Information

The sequences provided in this contribution are owned by InterDigital Communications, Inc.

The following uses are allowed for the contributed sequences:

- Sequences may be published in technical papers, played at technology research and development events.

- Sequences may be used by Standards committees. (e.g., ITU, MPEG, VQEG).

The following uses are NOT allowed for the contributed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

# References

- K. Suehring, and X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, Feb. 2016, San Diego, USA.

- E. Alshina, J. Boyce, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video,” JVET-F1030, Mar. – Apr. 2017, Hobart, AU