| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, July 13-21, 2017 | Document: JVET-G0056 |
| --- | --- |

| Title: | AHG8: A study on Equi-Angular Cubemap projection (EAC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA 92127, USA | Tel: Email: | [+1 858 521 5838] minhua.zhou@broadcom.com |
| Source: | Broadcom Limited |  |  |

_____________________________

# Abstract

This contribution studied the compression efficiency of the Equi-Angular Cubemap (EAC) projection, a modified cube map projection format similar to the ACP (Adjusted Cube Map Projection) being studied the JVET. Experimental results HM16.15 + JVET 360Lib-3.0 library per the 360 video common test conditions revealed that the EAC outperforms the ERP on average by 11.1%, 6.0% and 6.1% (E2E SPSNR-NN, RA-configuration) in Y, U, V, respectively. For E2E WSPSNR measure, the conversion only quality of EAC is about 0.6 dB higher than that of ACP (i.e. 49.12 dB vs. 48.55 dB).

# Introduction

Like ACP (Adjusted Cube Map Projection) [5], the Equi-Angular Cubemap (EAC) projection is aimed for having a uniform sampling rate regardless of sampling location on a cube face [1]. This contribution reports the performance of the EAC by integrating it into the HM16.15 + JVET 360Lib-3.0 library [2].

Figure 1. Comparison between CMP and EAC

# Equi-Angular Cubemap (EAC) Projection

The difference between ACP and EAC is highlighted in the yellow parts of description, the rest parts of description directly inherited from [5] for completeness.

The Equal Angular Cube map projection has 6 square faces in total, labelled as PX, PY, PZ, NX, NY, NZ (with “P” standing for “positive” and “N” standing for “negative”), in Figure 3. Table 1 specifies the face index values corresponding to each of the six cubemap faces [1].

The uv plane definition for each cubemap face is shown in Figure 2. Each face in the uv plane is a 2x2 square, with u and v being defined in the range of [−1, 1].

Figure 2. Coordinates definition for cubemap

Table 1. Face index of EAC

| Face index | Face label | Notes |
| --- | --- | --- |
| 0 | PX | Front face with positive X axis value |
| 1 | NX | Back face with negative X axis value |
| 2 | PY | Top face with positive Y axis value |
| 3 | NY | Bottom face with negative Y axis value |
| 4 | PZ | Right face with positive Z axis value |
| 5 | NZ | Left face with negative Z axis value |

Denote the dimension of any square face as A×A. For 2D-to-3D coordinate conversion, given the position (m, n) on a given face f, (u’, v’) is first calculated as:

| u’ = (m + 0.5) * 2 / A − 1, 0≤ m <A | (3) |
| --- | --- |
| v’ = (n + 0.5) * 2 / A − 1, 0 ≤ n < A | (4) |

The (u,v) values are adjusted to perturb the lookup vector by using Equation (1):

| u=tanu'π4.0 | (5) |
| --- | --- |
| v=tanv'π4.0 | (6) |

Then, the 3D coordinates (X, Y, Z) as defined in [1] are derived using Table 2 given the position (u’, v’) and the face index f.

Table 2. (X, Y, Z) derivation given (u, v) and the face index f

| f | X | Y | Z |
| --- | --- | --- | --- |
| 0 | 1.0 | −v | −u |
| 1 | −1.0 | −v | u |
| 2 | u | 1.0 | v |
| 3 | u | −1.0 | −v |
| 4 | u | −v | 1.0 |
| 5 | −u | −v | −1.0 |

For 3D-to-2D coordinate conversion, given (X, Y, Z), the (u’, v’) and face index f is calculated per Table 3.

Table 3. Derivation of (u’, v’) and the face index f given (X, Y, Z)

| Condition |  | f | u’ | v’ |
| --- | --- | --- | --- | --- |
| ¦X¦ ≥ ¦Y¦ and ¦X¦ ≥ ¦Z¦ and X >0 |  | 0 | −Z/¦X¦ | −Y/¦X¦ |
| ¦X¦ ≥ ¦Y¦ and ¦X¦ ≥ ¦Z¦ and X <0 |  | 1 | Z/¦X¦ | −Y/¦X¦ |
| ¦Y¦ ≥ ¦X¦ and ¦Y¦ ≥ ¦Z¦ and Y >0 |  | 2 | X/¦Y¦ | Z/¦Y¦ |
| ¦Y¦ ≥ ¦X¦ and ¦Y¦ ≥ ¦Z¦ and Y <0 |  | 3 | X/¦Y¦ | −Z/¦Y¦ |
| ¦Z¦ ≥ ¦X¦ and ¦Z¦ ≥ ¦X¦ and Z >0 |  | 4 | X/¦Z¦ | −Y/¦Z¦ |
| ¦Z¦ ≥ ¦X¦ and ¦Z¦ ≥ ¦Y¦ and Z <0 |  | 5 | −X/¦Z¦ | −Y/¦Z¦ |

The final (u,v) values obtained by adjusting (u’,v’) by

| u=4πtan-1u' | (7) |
| --- | --- |
| v=4πtan-1v' | (8) |

Then, (m, n) on the face f is calculated by solving the following equations,

| u = (m + 0.5) * 2 / A − 1, 0≤ m <A | (9) |
| --- | --- |
| v = (n + 0.5) * 2 / A − 1, 0 ≤ n < A. | (10) |

# Experimental results

The EAC was integrated into the HM16.15 + JVET 360Lib-3.0 library [2] for investigation. Per common test conditions described in [3], the random access (RA) are summarized in Table 4.

Table 4. RA results of EAC

The conversion only results are provided in Table 5. For E2E WSPSNR measure for example, the conversion only quality of EAC is about 0.6dB higher than that of ACP (49.12 dB vs. 48.55 dB).

Table 5. Conversion only results of EAC

The detailed results are provided in the attached spreadsheets.

# Summary

The experimental results revealed that the EAC has almost the same performance of ACP in 360Lib. Given the fact that it is used in YouTube 360 content, EAC could be important for 360 video applications in the future. It is therefore recommended to include the EAC into the JVET future investigation of alternative 360 video projection formats.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET-F1003, April 2017.

- 360Lib-3.0: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-3.0/

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, JVET-F1030, April 2017.

- Equi-Angular Cubemap Projection (EAC): https://blog.google/products/google-vr/bringing-pixels-front-and-center-vr-video/

- Muhammed Coban, Geert Van der Auwera, Marta Karczewicz, “AHG8: Adjusted cubemap projection for 360-degree video”, JVET-F0025, April 2017.

# Patent rights declaration(s)

Broadcom Limited does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity.