| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0073-v2 |
| --- | --- |

| Title: | Adaptive quantization and denoising for future video coding CfP and CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Kenneth Andersson and Per Wennersten Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46761153563 rickard.sjoberg (at) ericsson.com, kenneth.r.andersson (at) ericsson.com, per.wennersten (at) ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution states that video codec developments within ITU-T and MPEG have mainly been done using sequences containing unfiltered noise and test models using static QP values within pictures. The contribution points out that it is common in the industry to apply denoising filters and use adaptive quantization to optimize the subjective quality.

The contribution proposes that inclusion of these two tools should be considered for the test model and common test conditions for the future video coding project. It is also proposed that a test case for SDR video where denoising and adaptive quantization is allowed should be considered for the CfP for future video coding.

The contribution claims that if this is not addressed, there is a risk that the final video coding standard becomes less efficient than it could be for practical encoders that optimize for subjective quality.

It is reported in the contribution that compared to JEM-6.0, MS-SSIM luma BD-rates of −2.8%, −11.8%, −12.2%, −13.0% for AI, RA, LDB, LDP are achieved when applying denoising before encoding and using adaptive quantization. The corresponding PSNR luma BD-rate numbers are reported to be 15.4%, 7.6%, 8.2%, 7.4%.

Class B (HD) bitstreams were prepared and the proponents propose that an informal subjective viewing is held during the Torino meeting and that the contribution is revisited afterwards.

# Introduction

Currently video codec development within ITU-T and MPEG has mainly been done using test models operating without denoising input sequences and by using static QP values within each picture. This is not what typically is used in the industry where instead both adaptive QP and denoising is used by encoders to optimize the subjective quality.

Assuming that adaptive QP and denoising substantially improves the subjective quality and that those tools are used in the industry, it makes sense to us to consider including those tools in the test model and common test conditions for the future video coding project. A risk we see otherwise is that the final video coding standard becomes less efficient than it could have been for encoders that optimize for subjective quality.

# Denoising

Camera noise is typically uncorrelated both spatially and temporally. This means that compression of noisy input sequences is difficult since there is no substantial correlation to exploit. In our experience, compression at high quantization parameter (QP) values have less noise issues than compression at low QP values since the noise amplitude is too small to be coded at high QP. The noise at high QP values is removed by the quantizer and the codec acts as a denoiser. At low QP values, the noise generates high frequency coefficients. Those can be costly to encode and the bits spent on those are to a large degree useful for the current picture only.

The noise level is generally larger for larger spatial picture resolutions (such as 2160p) and darker scenes. Figure 1 shows the noise difference between the dark 3840x2160 CampfireParty and the 832x480 brighter PartyScene test sequences. The difference in noise levels is obvious to us.

Figure 2 shows the noise in the JVET CTC Class A1 and Class A2 test sequences.

Figure 1 – Luma component of cropped parts of CampfireParty (left) and PartyScene (right)

| Tango | Drums100 |
| --- | --- |
| CampfireParty | ToddlerFountain |
| CatRobot | TrafficFlow |
| DaylightRoad | Rollercoaster |

Figure 2 - Luma component of cropped parts of JVET CTC class A1 and A2 sequences

We have studied the effect of applying a denoising filter on the common test condition test sequences before encoding. The Avisynth tool “MC_Spudsmod” [1] was used, a tool uses motion compensated filtering.

The settings used were MC_Spudsmod(frames=4,strength=1,sharpp=0,chroma=true).

MC_Spudsmod is only capable of denoising 8 bit video so the 10 bit sequences were first downscaled to 8 bits and then the luma and chroma components were filtered.

To evaluate the performance, a test configuration consisting of the following steps was used:

- 10 bit source sequences were downscaled to 8 bits

- The denoising filter was applied

- Sequences were encoded and decoded using JEM 6.0.

- PSNR and MS-SSIM values were calculated against the CTC source sequences before denoising

This test configuration was then compared against the JEM 6.0 anchor.

# Variance based adaptive QP

Using a fixed quantizer within a picture is an adequate method for reducing the absolute error and to optimize for PSNR. The human eye, however, is more sensitive to relative errors. Because of this, many commercial encoders use adaptive QP algorithms in order to spend more bits where the eye is more sensitive to errors. Changing the QP values within a picture usually impacts PSNR negatively, but used with care it often gives subjective gains. The gains may be measured more accurately by error metrics such as MS-SSIM rather than PSNR.

An example of an adaptive QP algorithm is the implementation in the x264 AVC encoder. It sets macroblock QP values based on the variances of the macroblocks. Larger QP values are set for macroblocks with higher variance and lower QP values are set for macroblocks with lower variance.

We implemented an adaptive QP algorithm, that is very similar to the x264 algorithm, on top of JEM 6.0.

We used a max depth of 3 for signalling delta QP (MaxCuDQPDepth = 3, -dqd 3) which corresponds to 16x16 blocks, same as the macroblock size in AVC.

# Re-noising

A video encoder should preserve the source video signal as closely as possible. Applying denoising obviously removes noise from the source signal. This makes the signal easier to compress but the decoded signal may contain much less noise than the source. Therefore, applying generated noise to the decoded video may result in video that is subjectively closer to the source.

The Film grain characteristics SEI message, which is supported in both AVC and HEVC, seems fully capable of conveying noise characteristics from the source video that can be used by a decoder to generate the appropriate noise.

# Subjective results

We believe that denoising and adaptive QP in many cases improve the subjective quality. Some improvements can be seen in the screen shots below. Adaptive quantization uses lower QP values in areas with lower variance, this can improve the quality of textures such as water and streets as illustrated below.

Since noise is not easy to compress, applying denoising before encoding can save bits. For instance, the Intra picture pulsing effect can be perceived as lower after denoising. Then adding noise after decoding can mask temporal coding effects and other coding errors, and also make the video look more similar to the source.

The effect of denoising/re-noising does not show up as clearly in still pictures but below some examples of improvements from denoising and adaptive QP can be seen.

BQSquare QP 37 – Anchor 113kbps

BQSquare QP 37 – Denoising and adaptive QP 110kbps (-2.5%) BD-PSNR: +7% BD-MSSSIM: -32%

| TrafficFlow – QP 37 Anchor 724kbps | TrafficFlow QP 37 – Denoising and adaptive QP 734kbps (+1.4%) BD-PSNR: +7% BD-MSSSIM: -16% |
| --- | --- |

# Subjective viewing at the Torino meeting

A set of bitstreams were prepared for viewing at the Torino meeting. The following bitstreams were prepared for class B sequences from the JEM common test conditions:

- Uncompressed source sequences

- Anchor generated bitstreams using JEM 6.0

- Bitstreams generated by measuring the noise in the source sequence, applying a denoiser to the source, encoding the denoised sequences using a modified JEM encoder with adaptive quantization, decoding the bitstream with unmodified JEM decoder, and applying re-noising to the decoded sequences.

We propose that a subjecting viewing of the bitstreams are conducted during the Torino meeting.

# Results

Results are presented using the CTC and JEM-6.0 for AI, RA, LD and LDP. All results are given compared to the original sources before denoising. We provide BD-rate measurements both using PSNR as well as using MS-SSIM. As you can see, the BD-PSNR is often worse compared to the anchor while the BD-MS-SSIM is often better than the anchor.

## Adaptive quantization and denoising

The tested configuration here is using both adaptive quantization and denoising before encoding as explained in section 2.

AI

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | 20.26% | 43.02% | 39.20% | 4.65% | 13.49% | 12.86% | 86% | 101% |
| Class A2 | 22.07% | 44.30% | 44.18% | 3.44% | 16.10% | 19.99% | 88% | 102% |
| Class B | 14.22% | 16.29% | 14.12% | -4.57% | 12.32% | 9.84% | 94% | 101% |
| Class C | 10.28% | 11.33% | 10.27% | -6.63% | 6.28% | 6.09% | 99% | 100% |
| Class D | 9.29% | 10.12% | 10.38% | -8.88% | 7.31% | 8.85% | 103% | 102% |
| Class E | 16.92% | 11.64% | 9.71% | -4.59% | 13.64% | 9.41% | 101% | 101% |
| Overall | 15.40% | 22.98% | 21.49% | -2.76% | 11.47% | 11.19% | 95% | 101% |
| Class F (optional) | 11.24% | 7.94% | 7.52% | -13.01% | 0.17% | -0.37% | 104% | 103% |

RA

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | 9.85% | 31.65% | 28.77% | -8.31% | -5.20% | -2.76% | 91% | 91% |
| Class A2 | 13.05% | 44.36% | 43.85% | -7.90% | 3.05% | 6.53% | 94% | 89% |
| Class B | 4.31% | 2.19% | 5.57% | -15.05% | -2.23% | 0.83% | 99% | 100% |
| Class C | 5.96% | 2.98% | 2.31% | -11.66% | -3.00% | -2.84% | 99% | 102% |
| Class D | 5.42% | 4.91% | 4.39% | -15.26% | 1.26% | 1.90% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall (Ref) | 7.56% | 16.50% | 16.44% | -11.80% | -1.27% | 0.74% | 97% | 97% |
| Class F (optional) | 8.88% | 5.79% | 5.88% | -17.72% | -5.84% | -4.76% | 102% | 100% |

LD-B

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 6.69% | 0.23% | -0.96% | -13.18% | -5.28% | -7.34% | 101% | 101% |
| Class C | 6.64% | 1.34% | -1.06% | -11.24% | -4.79% | -6.75% | 102% | 102% |
| Class D | 5.34% | -0.42% | -2.43% | -16.44% | -5.45% | -5.94% | 102% | 101% |
| Class E | 16.91% | 10.51% | 4.86% | -6.34% | 14.53% | 6.64% | 100% | 104% |
| Overall (Ref) | 8.25% | 2.27% | -0.26% | -12.23% | -1.48% | -4.22% | 101% | 102% |
| Class F (optional) | 13.77% | 6.18% | 6.45% | -13.32% | -5.68% | -4.87% | 107% | 102% |

LD-P

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 5.45% | -1.19% | -2.15% | -14.32% | -6.95% | -8.77% | 96% | 96% |
| Class C | 6.26% | 0.79% | -1.68% | -11.65% | -5.42% | -7.56% | 100% | 98% |
| Class D | 4.95% | -1.76% | -3.12% | -16.96% | -6.17% | -6.95% | 103% | 102% |
| Class E | 15.48% | 7.04% | 3.16% | -7.44% | 10.83% | 4.45% | 96% | 103% |
| Overall (Ref) | 7.41% | 0.71% | -1.28% | -13.02% | -3.04% | -5.53% | 99% | 99% |
| Class F (optional) | 13.46% | 4.98% | 5.55% | -14.20% | -7.04% | -5.65% | 103% | 101% |

## Adaptive quantization only

The tested configuration here is using adaptive quantization but not denoising.

AI

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | 6.98% | 14.93% | 18.56% | -5.65% | 10.49% | 12.25% | 103% | 101% |
| Class A2 | 9.15% | 16.34% | 21.19% | -3.41% | 13.09% | 17.46% | 104% | 104% |
| Class B | 7.92% | 22.26% | 27.20% | -9.58% | 15.68% | 19.17% | 106% | 104% |
| Class C | 6.15% | 12.04% | 11.62% | -9.78% | 6.15% | 6.57% | 105% | 103% |
| Class D | 5.02% | 11.29% | 11.19% | -11.30% | 26.42% | 28.26% | 107% | 103% |
| Class E | 12.45% | 17.53% | 15.72% | -7.88% | 17.27% | 13.24% | 115% | 106% |
| Overall | 7.76% | 15.93% | 18.06% | -8.00% | 14.78% | 16.41% | 106% | 103% |
| Class F (optional) | 7.46% | 8.39% | 8.15% | -17.15% | -0.66% | -0.68% | 108% | 105% |

RA

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | 13.57% | 12.15% | 19.60% | -3.76% | 6.63% | 11.16% | 100% | 91% |
| Class A2 | 20.04% | 26.53% | 30.61% | 0.65% | 21.78% | 25.78% | 103% | 89% |
| Class B | 15.36% | 25.16% | 30.60% | -8.99% | 16.40% | 20.07% | 107% | 101% |
| Class C | 6.93% | 7.24% | 6.31% | -11.01% | 0.86% | 0.55% | 103% | 101% |
| Class D | 5.42% | 8.82% | 6.86% | -14.49% | 30.43% | 29.08% | 102% | 102% |
| Class E |  |  |  |  |  |  |  |  |
| Overall (Ref) | 12.41% | 16.42% | 19.36% | -7.59% | 15.28% | 17.46% | 103% | 97% |
| Class F (optional) | 8.55% | 9.63% | 10.03% | -18.84% | -2.91% | -1.69% | 105% | 102% |

LD-B

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 14.97% | 20.06% | 24.49% | -10.10% | 8.97% | 11.73% | 111% | 106% |
| Class C | 7.14% | 5.41% | 4.01% | -10.93% | -2.00% | -2.42% | 106% | 101% |
| Class D | 5.20% | 5.13% | 2.80% | -16.19% | 26.63% | 26.85% | 106% | 102% |
| Class E | 34.04% | 38.05% | 31.63% | -0.20% | 34.50% | 26.81% | 118% | 103% |
| Overall (Ref) | 14.15% | 16.04% | 15.29% | -9.97% | 15.43% | 14.80% | 110% | 103% |
| Class F (optional) | 13.72% | 11.53% | 12.75% | -14.44% | -1.57% | 0.18% | 108% | 101% |

LD-P

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 15.04% | 19.40% | 22.80% | -10.18% | 7.86% | 9.65% | 109% | 100% |
| Class C | 7.08% | 5.14% | 3.59% | -11.25% | -2.25% | -3.24% | 104% | 100% |
| Class D | 5.19% | 4.72% | 2.00% | -16.59% | 26.67% | 24.74% | 106% | 102% |
| Class E | 34.38% | 36.98% | 30.70% | -0.75% | 33.88% | 25.16% | 126% | 107% |
| Overall (Ref) | 14.21% | 15.46% | 14.28% | -10.28% | 14.91% | 13.11% | 110% | 102% |
| Class F (optional) | 13.50% | 10.52% | 11.46% | -15.02% | -3.09% | -0.61% | 109% | 100% |

## Denoising only

Test configuration here is using denoising before encoding as explained in section 2.

AI

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | 10.51% | 29.84% | 20.09% | 10.14% | 7.47% | 2.06% | 83% | 98% |
| Class A2 | 8.69% | 27.15% | 23.06% | 5.34% | 4.05% | 3.72% | 81% | 96% |
| Class B | 5.45% | -0.53% | -4.58% | 4.55% | 0.46% | -3.27% | 88% | 100% |
| Class C | 4.00% | 0.50% | -0.04% | 3.16% | 0.63% | 0.27% | 96% | 100% |
| Class D | 4.04% | -0.59% | -0.24% | 2.21% | -0.27% | 0.05% | 97% | 99% |
| Class E | 4.32% | -0.65% | -0.91% | 2.51% | -0.18% | -0.62% | 95% | 101% |
| Overall | 6.22% | 9.29% | 6.07% | 4.73% | 2.05% | 0.26% | 90% | 99% |
| Class F (optional) | 3.94% | 0.05% | -0.12% | 4.56% | 1.01% | 0.36% | 100% | 103% |

RA

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -0.93% | 27.24% | 13.40% | -2.20% | -4.57% | -7.90% | 87% | 92% |
| Class A2 | -1.85% | 21.72% | 18.18% | -5.50% | -10.76% | -10.22% | 87% | 86% |
| Class B | -4.29% | -10.45% | -10.06% | -2.95% | -9.08% | -8.32% | 93% | 97% |
| Class C | 0.47% | -2.36% | -2.40% | 0.02% | -2.58% | -2.52% | 97% | 102% |
| Class D | 1.31% | -1.44% | -1.40% | -0.01% | -1.79% | -1.99% | 99% | 102% |
| Class E |  |  |  |  |  |  |  |  |
| Overall (Ref) | -1.21% | 6.12% | 2.89% | -2.17% | -5.91% | -6.29% | 93% | 96% |
| Class F (optional) | 1.21% | -2.41% | -2.35% | 1.43% | -1.81% | -2.49% | 96% | 101% |

LD-B

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -1.78% | -7.92% | -10.94% | -0.74% | -6.08% | -8.98% | 97% | 100% |
| Class C | 0.75% | -2.38% | -3.31% | 0.54% | -1.82% | -2.93% | 98% | 100% |
| Class D | 1.66% | -3.18% | -3.28% | 0.95% | -2.86% | -2.92% | 102% | 105% |
| Class E | -6.89% | -14.84% | -14.07% | -5.18% | -11.59% | -11.31% | 92% | 98% |
| Overall (Ref) | -1.24% | -6.65% | -7.70% | -0.83% | -5.24% | -6.39% | 97% | 101% |
| Class F (optional) | 0.79% | -3.31% | -3.64% | 1.42% | -3.00% | -3.35% | 94% | 99% |

LD-P

|  | Y psnr | U psnr | V psnr | Y ms-ssim | U ms-ssim | V ms-ssim | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -2.81% | -8.91% | -11.82% | -1.60% | -7.23% | -10.00% | 88% | 92% |
| Class C | 0.43% | -2.84% | -3.66% | 0.25% | -2.39% | -2.95% | 95% | 99% |
| Class D | 1.46% | -2.94% | -2.99% | 0.68% | -2.35% | -2.02% | 99% | 104% |
| Class E | -7.81% | -15.74% | -14.03% | -5.67% | -12.34% | -11.40% | 90% | 97% |
| Overall (Ref) | -1.87% | -7.18% | -7.99% | -1.33% | -5.76% | -6.51% | 93% | 98% |
| Class F (optional) | 0.82% | -3.40% | -3.57% | 1.18% | -3.50% | -3.19% | 96% | 99% |

# Conclusion

We propose that inclusion of adaptive QP and denoising tools is considered for the test model and common test conditions for the future video coding project.

We also propose that a test case for SDR video where denoising and adaptive quantization is allowed is included in the CfP for future video coding.

# References

[1] http://avisynth.nl/index.php/MC_Spuds

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).