| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13-21 July 2017 | Document: JVET-G0074 |
| --- | --- |

| Title: | AHG8: ECP with padding for 360-degree video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Muhammed Coban, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | geertv@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The equatorial cylindrical projection (ECP), first introduced in JVET-F0026, is proposed with padding for efficient compression of 360-degree video and reduced seam artifacts in the rendered viewports. ECP projects the equatorial region of the sphere using the Lambert cylindrical equal-area projection, and projects the poles of the sphere onto square faces in the video picture to avoid inactive pixels. Without padding, the RA coding gain is 11.3% (e2e WS-PSNR) compared with the equirectangular projection in accordance with the JVET common test conditions for 360-degree video, while with the proposed padding the gain is only slightly reduced to 11.2%.

# Proposal

## ECP

It is proposed to apply the Lambert cylindrical equal-area projection to the equatorial sphere region as illustrated in Figure 1(a). The latitude interval of the equatorial region is ±sin-12/3≈±41.81°±sin-123≈±41.81°±sin-123≈±41.81°±sin-123≈±41.81°±sin-123≈±41.81°, which is chosen such that the equatorial region is 2/3 of the total sphere area, hence, one pole area is 1/6 of the sphere area. Similarly, the number of samples utilized by the equatorial region when mapped to square faces is equal to 2/3 of the video frame packed as in Figure 2(b). The number of samples of the video frame that are utilized by both pole segments is equal to 1/3 if the pole discs are mapped to square faces as illustrated in Figure 1(b). This way, all available pixels are utilized in the video frame, hence, there are no inactive samples in the packed video frame that would require additional decoder complexity. Section 3 formulates the conversion equations involved in the proposed equatorial cylindrical projection (ECP).

The equatorial and pole faces are arranged 3×2 into a video frame as illustrated in Figure 2(b). The pole faces are numbered 0 and 1, while the equatorial faces are numbered 2 to 5. Figure 2(a) is a frame in the equirectangular projection (ERP) format with corresponding faces indicated. In Figure 2(b), the top row consists of the first three equatorial faces (2-4) and the bottom row contains the rotated 5th equatorial face in between the rotated square pole faces. Figure 3 illustrates the packing with a frame from the ‘PoleVault’ sequence. Compared with CMP and ACP, for example, lines are not broken at any boundary within the cylindrical region, however, lines are curved due to the cylindrical projection.

| (a) |  |  | (b) |
| --- | --- | --- | --- |

Figure 1. (a) Mapping of equatorial sphere region onto cylinder geometry. (b) Disc ↔ square mapping.

| (a) | (b) |
| --- | --- |

Figure 2. (a) ERP partitioning into equatorial and pole regions. (b) 3x2 packing after ECP projection.

Figure 3. ECP 3×2 packed frame from ‘PoleVault’ sequence.

## Padded ECP

After the projection process, the video pictures are compressed, stored or transmitted, and decoded at the receiver side where the renderer generates the viewport that matches the viewer’s head orientation. Depending on this orientation and the field of view, the viewport may cross the picture boundaries or boundaries within the picture between adjacent or non-adjacent regions.

In case of 3×2 frame packing of the ECP regions, the viewport may cross various boundaries as depicted in Figure 4. The top picture half is a continuous region corresponding with three quarters of the unwrapped cylinder. In this packing, one quarter of the unwrapped cylinder is placed in the back position. Other boundaries are equivalent with CMP and ACP boundaries, with some additional discontinuity between the top and back regions, and the bottom and back regions due to minor misalignment in the ECP scheme. Alternative packing schemes such as 6×1 or 1×6 keep the unwrapped cylinder region connected, however, more boundaries are present between the cylindrical region and the pole regions.

| - ECP viewport1 (-180°, 35°) | - ECP viewport2 (0°, -35°) |
| --- | --- |

Figure 4. Viewport regions (shaded) crossing projected picture boundaries (bold) and boundaries between adjacent and non-adjacent picture regions (shaded regions are approximate).

The picture boundaries and boundaries between adjacent and non-adjacent regions within the picture may result in visible seam artifacts within the rendered viewport. The following is a list of causes:

- Motion: In case of motion across the picture boundary directed into the region, the motion estimation process may select a block that partially includes the padded samples that the encoder adds to extend the reference pictures. At higher QP values, those padded samples appear on one side of the boundary in the viewport, while the other side of the boundary is different as illustrated in Figure 3(a). This type of seam artifact is visible at higher QP values. A similar problem occurs inside the ECP picture (3×2 packing), when motion estimation selects a block that is located partially across the middle picture boundary.

- In-loop filtering: Since filtering is disabled on the picture boundaries, a sharp unfiltered seam may be visible in the rendered viewport especially at higher QP values. In the ECP picture (3×2 packing), the deblocking filtering occurs across the middle picture boundary resulting in visible seam artifacts as illustrated in Figure 3(b). Disabling in-loop filtering across tile boundaries (top and bottom picture halves) is not feasible if the middle picture boundary falls within the CTUs.

- …

Guard bands or padded regions are proposed for the ECP projection. Guard bands are regions added into the projected picture consisting of additional samples that are not essential for projecting the picture back onto the sphere, but they are helpful in reducing artifacts such as seams on the sphere or in the rendered viewport. The proposed ECP guard bands consist of additional projected samples, i.e., samples obtained around the projected regions with the ECP projection processes. The guard band regions are added to the projected picture before encoding, hence, the guard band regions are available to the video encoding processes for prediction, such as motion compensation, which may improve coding efficiency.

Figure 5 illustrates the proposed guard bands for 3×2 packed ECP pictures. Guard bands are added along the top and bottom halves of the projected picture. The main difference with CMP or ACP 3×2 packed pictures is that the ECP front region is continuous.

Figure 5. Proposed guard bands for 3×2 packed ECP. Guard bands are added around the top and bottom picture halves.

The padding width to completely remove the seams is dependent on the motion intensity in the video. However, for most of the test sequences it was experimentally determined that padding width equal to 4 samples is a reasonable trade-off between quality and coding efficiency (see Section 2). Figure 6 and Figure 7 depict ECP viewport2 (Figure 4b) to compare the seam regions with and without padding for the ‘Polevault’ and ‘DrivingInCountry’ sequences. Figure 8 illustrates the picture corner seams (viewport1) with the ‘DrivingInCountry’ sequence. The test conditions are RA and QP = 32. In these cases, the viewport seams are adequately removed by applying the 4-sample padding. In case of high motion intensity some seams may be slightly visible, but they are not objectionable given the overall low quality of the content at QP 32, for example.

(a)

(b)

Figure 6. (a) ECP viewport2 (0°, -35°) without padding. (b) ECP with padding. (‘Polevault’ sequence, QP=32, frame #0)

(a)

(b)

Figure 7. (a) ECP viewport2 (0°, -35°) without padding. (b) ECP with padding. (‘DrivingInCountry’ sequence, QP=32, frame #0)

(a)

(b)

Figure 8. (a) ECP viewport1 (-180°, 35°) without padding. (b) ECP viewport1 with padding. (‘DrivingInCountry’ sequence, QP=32, frame #28)

Lastly, the minor discontinuity between the bottom, top and back regions is remedied by gradually and continuously adjusting the sampling from the back region (rotated) to the bottom and top regions as illustrated in Figure 9.

Section 3 formulates the conversion equations including the proposed padding and gradual sampling adjustment.

| (a) |  |
| --- | --- |
| (b) |  |

Figure 9. (a) Minor discontinuities between back region, top and bottom regions (arrows). (b) Continuous transitioning between back, top and bottom regions. (‘Polevault’ sequence)

# Coding Results

The proposed ECP is implemented into the JVET 360Lib-3.0 [3] library for evaluation. The padding width is made configurable. Per common test conditions described in [2], the random access (RA) coding gain of 3×2 packed ECP without padding (incl. gradual sampling adjustment) is enumerated per sequence in Table 1. The ECP face sizes are set equal to cube map face sizes in [2]. Frame sizes are 3552×2368 for 8K test sequences and 2880×1920 for 4K.

Table 1. BD-rates for RA test conditions. ECP without padding (with gradual sampling adjustment).

| Proposed vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Face W | Face H | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -7.5% | 2.6% | -2.2% | -7.6% | 2.4% | -2.3% | -7.6% | 2.4% | -2.4% | -7.4% | 2.5% | -2.3% |
| GasLamp | 1184 | 1184 | -7.9% | -1.1% | 2.8% | -8.0% | -1.4% | 2.6% | -7.9% | -1.4% | 2.5% | -7.8% | -1.3% | 2.7% |
| Skateboarding_in_lot | 1184 | 1184 | -17.1% | -19.1% | -18.3% | -17.3% | -19.3% | -18.5% | -17.1% | -19.2% | -18.6% | -17.0% | -19.1% | -18.5% |
| Chairlift | 1184 | 1184 | -22.1% | -14.3% | -14.6% | -22.2% | -14.4% | -14.7% | -22.2% | -14.4% | -14.8% | -22.1% | -14.3% | -14.7% |
| KiteFlite | 1184 | 1184 | -8.1% | -2.3% | -3.7% | -8.3% | -2.5% | -3.9% | -8.1% | -2.5% | -3.9% | -8.0% | -2.3% | -3.7% |
| Harbor | 1184 | 1184 | -11.4% | -3.9% | -4.2% | -11.7% | -4.1% | -4.4% | -11.8% | -4.3% | -4.5% | -11.6% | -4.1% | -4.3% |
| PoleVault | 960 | 960 | -9.1% | -10.7% | -10.8% | -8.4% | -10.7% | -11.0% | -8.4% | -10.7% | -11.1% | -9.1% | -10.9% | -11.0% |
| AerialCity | 960 | 960 | -4.7% | -4.0% | -3.1% | -5.0% | -4.4% | -3.5% | -4.9% | -4.4% | -3.6% | -4.7% | -4.1% | -3.2% |
| DrivingInCity | 960 | 960 | -4.8% | 6.4% | 5.3% | -4.1% | 5.6% | 4.6% | -3.9% | 5.6% | 4.6% | -4.7% | 6.4% | 5.3% |
| DrivingInCountry | 960 | 960 | -20.9% | -15.6% | -18.6% | -19.8% | -16.1% | -18.9% | -19.8% | -16.0% | -18.8% | -20.9% | -15.6% | -18.5% |
| Overall |  |  | -11.3% | -6.2% | -6.7% | -11.2% | -6.5% | -7.0% | -11.2% | -6.5% | -7.0% | -11.3% | -6.3% | -6.8% |

Additional results in Table 2 are obtained with padding (width = 4 samples).

Table 2. BD-rates for padded ECP (padding width = 4 samples).

| Proposed with padding vs. ERP |  |  | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Face W | Face H | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 1184 | 1184 | -6.6% | 3.0% | -2.1% | -6.8% | 2.8% | -2.3% | -6.8% | 2.9% | -2.3% | -6.7% | 3.0% | -2.2% |
| GasLamp | 1184 | 1184 | -7.4% | -1.3% | 2.8% | -7.4% | -1.4% | 2.6% | -7.4% | -1.3% | 2.6% | -7.3% | -1.2% | 2.8% |
| Skateboarding_in_lot | 1184 | 1184 | -17.0% | -19.5% | -18.8% | -17.1% | -19.7% | -19.0% | -17.0% | -19.6% | -19.1% | -16.9% | -19.5% | -19.0% |
| Chairlift | 1184 | 1184 | -22.0% | -13.9% | -14.3% | -22.1% | -13.9% | -14.3% | -22.1% | -13.9% | -14.4% | -22.0% | -13.9% | -14.4% |
| KiteFlite | 1184 | 1184 | -7.5% | -1.7% | -3.6% | -7.7% | -1.9% | -3.8% | -7.6% | -1.9% | -3.7% | -7.4% | -1.8% | -3.6% |
| Harbor | 1184 | 1184 | -11.2% | -5.4% | -5.2% | -11.4% | -5.4% | -5.2% | -11.7% | -5.5% | -5.3% | -11.4% | -5.4% | -5.1% |
| PoleVault | 960 | 960 | -8.7% | -9.3% | -9.4% | -8.1% | -9.2% | -9.5% | -8.2% | -9.2% | -9.6% | -8.8% | -9.4% | -9.6% |
| AerialCity | 960 | 960 | -6.1% | -5.2% | -4.2% | -6.3% | -5.3% | -4.5% | -6.2% | -5.3% | -4.5% | -6.1% | -5.2% | -4.3% |
| DrivingInCity | 960 | 960 | -4.6% | 5.2% | 3.6% | -3.9% | 4.8% | 3.2% | -3.8% | 4.8% | 3.2% | -4.5% | 5.3% | 3.7% |
| DrivingInCountry | 960 | 960 | -20.6% | -17.0% | -20.2% | -19.7% | -17.2% | -20.3% | -19.7% | -17.2% | -20.2% | -20.6% | -17.0% | -20.1% |
| Overall |  |  | -11.2% | -6.5% | -7.1% | -11.0% | -6.6% | -7.3% | -11.0% | -6.6% | -7.3% | -11.2% | -6.5% | -7.2% |

# Conversion Equations

## 2D-to-3D Conversion

Following the descriptions in [1], denote the dimension of one face as A×A. In 2D-to-3D conversion, a point (m, n) on the face is mapped to a point on the sphere (ϕ, θ) per equations below, depending on the face it belongs to in Figure 2b.

First, the point (x, y) is determined as follows:

x=2Am+12-1

y=2An+12-1

The following scaling factors are first defined:

padfactor = 1+2  padmarginA-2  padmargin

apadfactor = AA-padmargin

with padmargin specifying the padding width in sample units (must be smaller than A/2). Next, the following scaling is applied to (x, y) depending on face index f:

| Index f | 2D-to-3D Conversion |
| --- | --- |
| 0 | x'=x × padfactor y'=y+1 × apadfactor-1 |
| 1 | x'=x × padfactor y'=y-1 × apadfactor+1 |
| 2 | x'=x-1 × apadfactor+1 y'=y × padfactor |
| 3 | x'=x y'=y × padfactor |
| 4 | x'=x+1 × apadfactor-1 y'=y × padfactor |
| 5 | x'=x × padfactor y'=y |

If face index f = 0, the gradual sampling transitioning is given by:

x''=1+tanh-y'-1b

x=tan(tan-1x'' x')/x''

with b=0.2 and y=y'.

If face index f = 1, the gradual sampling transitioning is given by:

x''=1+tanhy'-1b

x=tan(tan-1x'' x')/x''

with b=0.2 and y=y'.

If face index f = 0 or 1, then square-to-disc mapping [4] is applied:

s=x2+y2-x2y2x2+y2

u=A2sx

v=A2sy

For the face f = 0, the following applies:

ϕ=tan-1uv

θ=π2-2rAπ2-sin-123

with r=u2+v2, θ∈sin-123,π2, ϕ∈-π,π.

For the face f = 1, the following applies:

ϕ=tan-1uv+π2

θ=2rAπ2-sin-123-π2

where r=u2+v2 and θ∈-π2,-sin-123, ϕ∈-π,π.

For the faces with face index f = 2…5, the following applies:

ϕ=π4u+2f-7

θ=sin-1-2v/3

with yaw in the range ϕ∈-π+f-2π2,-π2+f-2π2 depending on f, and pitch in the range θ∈-sin-123,sin-123.

## 3D-to-2D Conversion

Following descriptions in [1], a point on the sphere (ϕ, θ) is mapped to a point (m, n) on the face with dimension A×A depending on the face index f per equations below.

For the face f = 0, θ∈sin-123,π2, ϕ∈-π,π, the following applies:

u=π2-θsinϕπ2-sin-123

v=π2-θcosϕπ2-sin-123

Followed by disc-to-square mapping [4]:

w=sgnuv2u2+v2-u2+v2u2+v2-4u2v2

x',y'=wv,wu  if w>0u,v,   otherwise

with sgn the sign function. Next, the gradual sampling transitioning is applied:

x''=1+tanh-y'-1b

x=tan-1x''x'/tan-1x''

with b=0.2 and y=y'.

For the face f = 1, θ∈-π2,-sin-123, ϕ∈-π,π, the following applies:

u=π2+θsinϕπ2-sin-123

v=-π2+θcosϕπ2-sin-123

Followed by disc-to-square mapping:

w=sgnuv2u2+v2-u2+v2u2+v2-4u2v2

x',y'=wv,wu  if w>0u,v,   otherwise

Next, the gradual sampling transitioning is applied:

x''=1+tanh-y'-1b

x=tan-1x''x'/tan-1x''

with b=0.2 and y=y'.

For the square faces with face index f = 2…5 yaw in the range ϕ∈-π+f-2π2,-π2+f-2π2 depending on f, and pitch in the range θ∈-sin-123,sin-123, the following applies:

x=4π ϕ+7-2f

y=-32sinθ

For all index f, the following scaling factors are first defined:

padfactor = 1+2  padmarginA-2  padmargin

apadfactor = AA-padmargin

with padmargin specifying the padding width in sample units (must be smaller than A/2). Next, the following scaling is applied to (x, y) depending on face index f:

| Index f | 3D-to-2D Conversion |
| --- | --- |
| 0 | x'=x / padfactor y'=y+1 / apadfactor-1 |
| 1 | x'=x / padfactor y'=y-1 / apadfactor+1 |
| 2 | x'=x-1 / apadfactor+1 y'=y / padfactor |
| 3 | x'=x y'=y / padfactor |
| 4 | x'=x+1 / apadfactor-1 y'=y / padfactor |
| 5 | x'=x / padfactor y'=y |

Finally, (m, n) are given by:

m=A2x'+1-12

n=A2y'+1-12

# Conclusion

ECP with 3×2 frame packing and padding was proposed, including gradual sampling adjustment. The equatorial region of the sphere is projected using the Lambert cylindrical equal-area projection and the pole regions are projected onto squares. The reported coding gain without padding is 11.3% (e2e WS-PSNR) for RA conditions, while the gain is 11.2% with the proposed padding.

# References

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET-F1003, Apr. 2017.

- E. Alshina, J. Boyce, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360-degree video”, JVET-F1030, Apr. 2017.

- 360Lib-3.0: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-3.0/

- M. Fernandez-Gausti, “Classroom notes: Analytic geometry of some rectilinear figures”, Int. J. Mathematical Education in Science and Technology, Vol. 23, No. 6, 1992, pp. 895–913.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).