| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0145 |
| --- | --- |

| Title: | AHG4: Evaluation report of drone test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | You-Huei Ju, Ching-Chieh Lin1, Po-Han Lin and Chun-Lung Lin2 2870 Zanker Road, Suite 140, San Jose, CA 95134, USA | Tel: Email: | +886-3-591-5758 1JackLin@itri.com 2Chunlung@itri.com |
| Source: | ITRI international |  |  |

_____________________________

- Abstract

This contribution crosschecked the encoding results from JVET-G0096. The two drone test sequences, BeachMountain2 and MountainBay2, were encoded by HM16.15 and JEM6.0. In addition, subjective evaluations are also provided in this document.

Introduction

In this report, the two new 4K drone test sequences proposed in JVET-F0062 [1] were evaluated. They were encoded with HM16.15 and JEM6.0 software. The test conditions and the target bitrates are settled according to JVET-G0096 [2].

Encoding conditions

The encoding conditions:

- Encoder: JEM6.0, HM16.15

- Encode only 4:2:0 yuv 10 bit

- I period: 32 (30fps)

- Test Condition: Random Access(RA)

Test sequences

The details of the drone test sequences are as follows:

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| BeachMountain2 | 1500 | 2400 | 4000 | 7000 |
| MountainBay2 | 1000 | 1500 | 2400 | 4000 |

Table 1: Target bitrates of the aerial photography test sequences

Table 2: Descriptions of aerial photography test sequences

| Scenario | Description | Thumbnail |
| --- | --- | --- |
| BeachMountain2_ 3840x2160_30fps_ 420_10bit | The scenario contains the left-down to right-up pull up motion. The beach, mountain, and sky are shown in the scenario which has plentiful details. |  |
| MountainBay2_ 3840x2160_30fps_ 420_10bit | The scenario contains slow clockwise rotation. The sea, mountain, sky, and cloud are shown in the scenario which has plentiful details. |  |

Test sequences evaluation

Bitrate Matching

In this report, the values of base QP, RAS number and qpif were chosen according to [3]. Followings are our test results. The PSNR values are identical with what provided by proponents. More details can be found in the accompanying excel file.

Table 3: Encoding results of the test sequences by using HM16.15 and JEM6.0

| BeachMountain2 (30p) |  | HM-16.15 (sequential enc) |  |  |  | HM-16.6-JEM-6.0 (parallel enc) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | target rate [kbps] | QP | qpif | bitrate [kbps] | PSNR Y [dB] | QP | RAS# | qpif | bitrate [kbps] | PSNR Y [dB] |
| R4 | 7000 | 32 | 8 | 7090.2112 | 34.2917 | 32 | 1 | 48 | 6981.4632 | 34.61978867 |
| R3 | 4000 | 35 | 16 | 4003.8864 | 32.9295 | 35 | 1 | 48 | 3947.8256 | 33.300559 |
| R2 | 2400 | 38 | 49 | 2386.488 | 31.8394 | 38 | 3 | 112 | 2383.9896 | 32.250672 |
| R1 | 1500 | 40 | 17 | 1510.8736 | 31.0838 | 40 | 1 | 48 | 1502.1432 | 31.45967533 |
| MountainBay2 (30p) |  | HM-16.15 (sequential enc) |  |  |  | HM-16.6-JEM-6.0 (parallel enc) |  |  |  |  |
|  | target rate [kbps] | QP | qpif | bitrate [kbps] | PSNR Y [dB] | QP | RAS# | qpif | bitrate [kbps] | PSNR Y [dB] |
| R4 | 4000 | 29 | 8 | 4072.572 | 38.5683 | 29 | 4 | 144 | 4018.2408 | 39.085981 |
| R3 | 2400 | 33 | 288 | 2386.964 | 37.1512 | 32 | 3 | 112 | 2386.0032 | 37.788555 |
| R2 | 1500 | 35 | 40 | 1496.5112 | 35.9284 | 35 | 6 | 208 | 1498.5856 | 36.638013 |
| R1 | 1000 | 38 | 184 | 993.6736 | 34.8962 | 37 | 1 | 48 | 992.9048 | 35.642857 |

Subjective Evaluation

For the two drone test sequences, it is observed that the differences between the uncompressed and compressed video are not obvious when the QP value is small (i.e. the rate point is high). Therefore, we focused more on evaluating the encoding performances of the rate point 2 and rate point 3.

Figure 1: Rate point 3 of BeachMountain2 sequence when the frame number is 299 
The left one is encded by HM16.15 and the right one is encoded by JEM6.0.

More details of the mtountain vegetation and the boat are missing in HM. In addition, the edge of the mountain is blured in HM.

Figure 2: Rate point 3 of BeachMountain2 sequence when the frame number is 284. 
The top one is encded by HM16.15 and the bottom one is encoded by JEM6.0.

The details of the submontane community, vegitation of the hill and the waves of the sea blur seriouslier in HM. The flickering effetcs are observed on the water surface.

Figure 3: Rate point 2 of MountainBay sequence when the frame number is 21. 
The top one is encded by HM16.15 and the bottom one is encoded by JEM6.0.

The pattern of the roof and the details of the trees in HM are missing in JEM.

Figure 4: Rate point 2 of MountainBay sequence when the frame number is 291. 
The top one is encded by HM16.15 and the bottom one is encoded by JEM6.0.

In HM, part of depth information of the cloud is missing. It makes the cloud looks flatter. Moreover, flickers are also observerd on the mountain.

Conclusion

The contribution reports the encoding results with specific rates and subjective evaluation results of the two 4K drone test sequences. Bitstreams were generated by HM16.15 and JEM6.0 according to the test conditions specified in JVET-G0096. Since aerial photography is widely used nowadays, it is suggested to consider including drone test sequences as test material in the upcoming call for proposal of future video coding standard.

Reference

- “New aerial photography sequences for video coding standard development,” JVET-F0062, April 2017

- “AhG4: Evaluation on drone test sequences,” JVET-G0096, July 2017

- ftp://ftp.ient.rwth-aachen.de/

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.15/

- https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-5.0.1/

Patent rights declaration(s)

ITRI international does not have any current or pending patent rights relating to the technology described .in this contribution.